TSG-RAN Working Group 4 meeting #88bis	  R4-1813063

Chengdu, China, 8 – 12 October 2018

Source:	Sony Mobile

Title:	MU improvement proposal by promoting the white or similar box approach.

Agenda item:	10.1.1.

Document for:	Discussion

# Background

A significant contributor to the UE FR2 overall Measurement Uncertainty budget, given in section B.1.1.3. of [1], is related to unprecise positioning of the Device Under Test in the Quiet Zone of the anechoic chamber. At R4 #84 it was decided to allow for a test procedure where the D.U.T. can be randomly oriented in the QZ, the s.c. “black box approach” further described in [2]. It is however believed most 3GPP parties share the opinion the desire for reduced MU.

# Proposal

For rel. 16 it is proposed UE manufacturers mandatory to declare in one way or another (FFS) the location of their antennas in their D.U.T.´s such that a more precise positioning vs. the QZ can take place. Thus MU related to misalignments will be reduced.

# References

[1]	TR 38.810 v.2.4.0.

[2]	R4-1708200 Rohde & Schwarz, Black Box vs White Box Testing for NR