3GPP TSG-RAN WG4 Meeting #88bis                 	R4-1813202

Chengdu, China, 8 – 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for SPUCCH format 4

Agenda item:	6.7.6

Document for:	Discussion

Introduction

Demodulation requirements for SPUCCH format 1a were agreed in [1] in Montreal AH. In this contribution we provide simulation results for SPUCCH format 4 demodulation based on the agreed simulation assumptions in [2].

Simulation assumptions

Simulation assumptions as agreed for SPUCCH in [2] are shown below.

Table 1: Simulation assumption for sPUCCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| SPUCCH formats | Format 1a, format 4 |
| Number of bits per PRB for format 4 | 3 bits per 2 PRB |

Simulation results

Simulation results for format 4 using the parameters from Table 1 are shown below in Table 2 for 2 OS TTI, 3OS TTI, and as an average of these two.

Table 2: Simulation results for SPUCCH format 4.

|  | Number of Tx Antennas | Number of Rx Antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [dB] with DTXtoACK probability 1 % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 Mhz | 10 MHz | 15 MHz | 20 MHz |
| 2 OS | 1 | 2 | Normal | EPA5 Low | 5.40 | 5.61 | 5.53 | 5.52 |
|  |  | 4 | Normal | EPA5 Low | -0.42 | -0.45 | -0.41 | -0.30 |
| 3 OS | 1 | 2 | Normal | EPA5 Low | 3.26 | 3.53 | 3.40 | 3.56 |
|  |  | 4 | Normal | EPA5 Low | -2.39 | -2.44 | -2.47 | -2.39 |
| Average | 1 | 2 | Normal | EPA5 Low | 4.7 | 4.9 | 4.8 | 4.9 |
|  |  | 4 | Normal | EPA5 Low | -1.1 | -1.1 | -1.1 | -1.0 |

Conclusion

In this contribution we have presented simulation results for SPUCCH with format 4.

- References

- R4-1809373, Performance requirements for SPUCCH, Ericsson, 3GPP TSG-RAN WG4 AH1807, Montreal, Canada, July 2018.

- R4-1805495, Simulation assumptions for sPUCCH and sPUSCH, Ericsson, 3GPP TSG-RAN WG4 #86bis, Melbourne, Australia, April 2018.