3GPP TSG-RAN WG4 Meeting #88bis                 	R4-1813204

Chengdu, China, 8 – 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on how to count carriers in synchronous inter-band EN-DC (38.133) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is still not agreed how the carriers are counted when MN and SN configure MO for same carrier frequency layer in inter-band synchronous EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to count the two MOs as one carrier frequency layer in inter-band synchronous EN-DC under the same conditions as in synchronous intra-band EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.3.2	NSA: Maximum allowed layers for multiple monitoring

If a UE is configured with E-UTRA-NR dual connectivity operation, the UE shall be capable of monitoring at least:

-	Depending on UE capability, 7 NR inter-frequency carriers configured by PScell, and

-	Depending on UE capability, 7 NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell [15], and

-	Depending on UE capability, 6 E-UTRA TDD inter-frequency carriers configured by E-UTRA PCell [15], and

-	Depending on UE capability, 6 E-UTRA FDD inter-frequency carriers configured by E-UTRA PCell [15], and

-	Depending on UE capability, 3 FDD UTRA carriers, and

-	Depending on UE capability, 3 TDD UTRA carriers, and

-	Depending on UE capability, 32 GSM carriers (one GSM layer corresponds to 32 carriers), and

-	Depending on UE capability, 1 E-UTRA FDD inter-frequency carrier for RSTD measurements configured via LPP [22], and

-	Depending on UE capability, 1 E-UTRA TDD inter-frequency carrier for RSTD measurements configured via LPP [22].

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM (one GSM layer corresponds to 32 carriers) layers. The UE shall be capable of monitoring a total of at least 7 effective NR carrier frequency layers excluding NR serving carrier(s), comprising of any above defined combination of NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell and NR inter-frequency carriers configured by PSCell.

When the E-UTRA PCell and PSCell configure the same NR carrier frequency layer to be monitored by the UE in synchronous intra-band or inter-band EN-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFNs and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have

- different subcarrier spacing or

- different RSSI measurement resources or

- different deriveSSB-IndexFromCell indications or

- different SMTC configurations.

Note 1:	The E-UTRA-NR dual connectivity capable UE configured with PSCell shall fulfil the requirements defined in only one of Section 9.1.3.2 and Section 8.1.2.1.1b.1 of [15].