3GPP TSG RAN WG4 Meeting #88Bis	R4-1813278

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.9.2.3

Source:	Huawei, HiSilicon

Title:	TP for FR2 base conformation test models

Document for:	Approval

# Introduction

In RAN#88, text proposals were agreed for the test model in [4]. This document provides revised text proposals for additional details in the test model.

# Discussion

There are several remaining details for the test models including

- PDSCH DMRS

- Power boosting for the PDSCH

- SS block

- TDD

The TP captures the details from [1] [2] [3] [5].

# Conclusion

The following text proposal should be used for the test model description in 38.141-2 [4].

# References

R4-1813280, “SS block in test models”, Huawei, HiSilicon, RAN4#88Bis, Oct. 8-12, 2018.

R4-1813281, “Boosting patterns in test models”, Huawei, HiSilicon, RAN4#88Bis, Oct. 8-12, 2018.

R4-1813283, “Reference Symbol for Test Model”, Huawei, HiSilicon, RAN4#88Bis, Oct. 8-12, 2018.

38.141-2

R4-1813279, “Reference Symbol for Test Model”, Huawei, HiSilicon, RAN4#88Bis, Oct. 8-12, 2018.

# Appendix

## Text Proposal

****** BEGIN *****

### 4.9.2	Test models

#### 4.9.2.1	General

The NR test models described in TS 38.141-1 is also applicable for 1-O. The following sections will describe the NR test models needed for 2-O.

#### 4.9.2.2	NR test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	The test models are defined for a single antenna port; 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise]

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	Normal CP

-	Virtual resource blocks of localized type

-	Cell ID NIDcell=[1]

For NR TDD, test models are derived based on the uplink/downlink configuration as showing in the table 4.9.2-1.

Table 4.9.2-1: Configurations of TDD gNB test models

| SCS [kHz] | Number of DL slots | Number of DL symbols in S slot | Number of UL symbols in S slot | Number of UL slots |
| --- | --- | --- | --- | --- |
| 60 (Note 1) | 14 | 12 | 8 | 4 |
| 120 (Note 2) | 29 | 10 | 2 | 9 |
| 240 (Note 3) | 59 | 6 | 4 | 18 |
| Note 1: There are two S slots. First S slot has 12 DL symbols followed by 2 flexible symbols; second S slot has 6 flexible symbols followed by 8 UL symbols. Note 2: There are two S slots. First S slot has 10 DL symbols followed by 4 flexible symbols; second S slot has 12 flexible symbols followed by 2 UL symbols. Note 3: There are three S slots. First S slot has 6 DL symbols followed by 8 flexible symbols; third S slot has 10 flexible symbols followed by 4 UL symbols. |  |  |  |  |

Table 4.9.2-1: Configurations of TDD gNB test models

| SCS [kHz] | Number of DL slots | Number of DL symbols in S slot | Number of UL symbols in S slot | Number of UL slots |
| --- | --- | --- | --- | --- |
| 60 | 3 | 4 | 4 | 1 |
| 120 (Note 1) | 6 | 8 | 8 | 2 |
| 240 (Note 2) | 13 | 2 | 2 | 5 |
| Note 2: There are two S slots. First S slot has 8 DL symbols followed by 6 flexible symbols; second S slot has 6 flexible symbols followed by 8 UL symbols. Note 3: There are two S slots. First S slot has 2 DL symbols followed by 12 flexible symbols; second S slot has 12 flexible symbols followed by 2 UL symbols. |  |  |  |  |

Measurements are not performed when the number of DL symbols in an S slot is 6.

For the PDSCH, the l-th symbol of a slot for l=0,…,13 carries DMRS if l{2, 5, 8, 11} and symbol l is configured as a DL symbol.

NRB is considering the maximum transmission bandwidth configuration seen in table 5.3.2-2.

##### 4.9.2.2.1	NR test model 1.1 (NR-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

Table 4.9.2.2.1-1: Physical channel parameters of NR-TM1.1 for x kHz SCS with y MHz bandwidth

| Parameter | SCS: x kHz, BW: y MHz |
| --- | --- |
| PDCCH |  |
| # of symbols used for control channel | [1] |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the first symbol (# of RBs for PDSCH) | NRB – 6 |
| Starting RB for PDCCH | 0 |
| DM-RS configuration and density | REs 1, 5, 9 of each REG in each symbol |
| SS Block |  |
| SCS | 120 kHz |
| Number of slots | [2] |
| Starting symbol locations for SS block | [center] |
| Location of SS block | Symbols 4 and 8 of slot 0, symbols 2 and 6 of slot 1 |
| Power boosting |  |
| PDSCH |  |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 |
| PT-RS pattern and density | Distributed pattern, every symbol for every second PRB (time density 1 and frequency density ½) |
| # of QPSK PDSCH PRBs which are boosted or deboosted | 0 |

##### 4.9.2.2.2	NR test model 2 (NR-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Table 4.9.2.2.2-1: Physical channel parameters of NR-TM 2 for x kHz SCS with y MHz bandwidth

| Parameter | SCS: x kHz, BW: y MHz |
| --- | --- |
| PDCCH |  |
| # of symbols used for control channel | [1, 2] |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the first symbol | NRB – 6 |
| Starting RB for PDCCH | 0 |
| DM-RS configuration and density | REs 1, 5, 9 of each REG in each symbol |
| SS Block |  |
| SCS | 120 kHz |
| Number of slots | [2] |
| Starting symbol locations for SS block | [center] |
| Location of SS block | Symbols 4 and 8 of slot 0, symbols 2 and 6 of slot 1 |
| Power boosting | [10 log10 (20/NRB*2µ)] |
| PDSCH |  |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) |
| # of QPSK PDSCH PRBs which are boosted | 1 |
| Level of boosting [dB] | B=0 |
| Location of boosted RB | Ps=s×NRB10×2μ+fmod2mod NRB Where s=0,⋯,10×2μ-1, f=0,1,… |
| # of QPSK PDSCH PRBs which are not allocated | NRB-1 |
| Level of deboosting [dB] | -inf |

#### 4.9.2.2.3	NR test model 3.1 (NR-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation

Physical channel parameters are defined in table 4.9.2.2.1-1 with 64QAM PDSCH PRBs used.