3GPP TSG RAN WG4 Meeting #88bis	R4-1813354

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.5.2.3

Source:	Huawei, HiSilicon

Title:	RB configuration for mixed numerologies

Document for:	Approval

# Introduction

The maximum transmission bandwidth configuration for each BS channel bandwidth is given in TS38.104 by NRB in Table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2. The general issue is that for a given channel bandwidth, if more than one numerology is configured for the carrier, it may not be possible to configure the value provided in 38.104 for all configured numerologies. Further clarification of the problem description is provided.

# Discussion

In 38.104, the transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N.A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2

| SCS [kHz] | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |

In RAN1, the location of the RBs in a channel is described using a grid and point A.  In 38.331, the location of a carrier and the number of RBs of a carrier is signaled with IE SCS-SpecificCarrier as follows.

-- ASN1START

-- TAG-SCS-SPECIFIC-CARRIER-START

SCS-SpecificCarrier ::=				SEQUENCE {

offsetToCarrier						INTEGER (0..2199),

subcarrierSpacing					SubcarrierSpacing,

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks),

...

}

-- TAG-SCS-SPECIFIC-CARRIER-STOP

-- ASN1STOP

Based on the signalling, gNB can configure the location of carrier with offsetToCarrier (Offset in frequency domain between Point A) and configure the number of RBs of carrier with carrierBandwidth flexiblely. Figure 1 shows the relationship between point A and the grid.

Figure 2. Configuration on RB grid of a numerology using Point A.

If only one numerology is configured for a carrier, it is possible to configure the maximum number of RBs specified in 38.104 for the given numerology. For example, for 15MHz channel bandwidth, if only 30KHz SCS is configured, the value of carrierBandwidth=38 can be configured for 30kHz SCS. However, if more than one numerology is configured for a carrier, since there is RB alignment between different numerologies and each configured numerology must fulfil the minimum guardband requirement, it is not always possible to configure the maximum number of RBs specified in 38.104 for all configured numerologies.  As an example, for 15MHz channel bandwidth, if 15kHz and 30kHz are configured, it is not possible to configure carrierBandwidth=79 for 15kHz and carrierBandwidth=38 for 30kHz. This is illustrated in Figure 2, where all possible RB configurations for 15MHz channel with (15kHz, 30kHz) mixed numerology are shown.

Figure 1. Possible RB configurations for 15MHz channel bandwidth with (15kHz, 30kHz) mixed numerology.

It can be seen from the figure that, for mixed numerology in 15MHz channel bandwidth, one of the numerologies should be configured with one RB less than the maximum value of NRB provided in Table 5.3.2-1.

Observation 1: If more than one numerology is configured for a carrier, it is not always possible to configure carrierBandwidth=NRB from table 5.3.2-1 for all configured numerologies.

To clarify the above, the following TP is proposed to 38.104, 38.101-1, and 38.101-2:

Proposal 1: Adopt the following TPs in section 5.3.4 of 38.104, 38.101-1, 38.101-2:

-------------------------- Begin of Text Proposal for TS 38.104 Subclause 5.3.4 -----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] shall not exceed the transmission bandwidth configuration specified in subclause 5.3.2 and must fulfil the minimum guardband requirement specified in subclause 5.3.3 corresponding to the minimum channel bandwidth that fully contains the said transmission bandwidth configuration.

-------------------------- End of Text Proposal for TS 38.104 Subclause 5.3.4 --------------------------

-------------------------- Begin of Text Proposal for TS 38.101-1 Subclause 5.3.4 ----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] must fulfil the minimum guardband requirement specified in subclause 5.3.3.

-------------------------- End of Text Proposal for TS 38.101-1 Subclause 5.3.4 ------------------------

-------------------------- Begin of Text Proposal for TS 38.101-2 Subclause 5.3.4 ----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] must fulfil the minimum guardband requirement specified in subclause 5.3.3.

-------------------------- End of Text Proposal for TS 38.101-2 Subclause 5.3.4 ------------------------

# Conclusion

This contribution discusses the RB signaling in mixed numerologies.

Observation 1: If more than one numerology is configured for a carrier, it is not always possible to configure carrierBandwidth=NRB from table 5.3.2-1 for all configured numerologies.

Proposal 1: Adopt the following TPs in section 5.3.4 of 38.104, 38.101-1, 38.101-2:

-------------------------- Begin of Text Proposal for TS 38.104 Subclause 5.3.4 -----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] shall not exceed the transmission bandwidth configuration specified in subclause 5.3.2 and must fulfil the minimum guardband requirement specified in subclause 5.3.3 corresponding to the minimum channel bandwidth that fully contains the said transmission bandwidth configuration.

-------------------------- End of Text Proposal for TS 38.104 Subclause 5.3.4 --------------------------

-------------------------- Begin of Text Proposal for TS 38.101-1 Subclause 5.3.4 ----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] must fulfil the minimum guardband requirement specified in subclause 5.3.3.

-------------------------- End of Text Proposal for TS 38.101-1 Subclause 5.3.4 ------------------------

-------------------------- Begin of Text Proposal for TS 38.101-2 Subclause 5.3.4 ----------------------

### 5.3.4	RB alignment with different numerologies

For each numerology, its common resource blocks are specified in subclause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The transmission bandwidth configuration indicated by higher layer parameter carrierBandwidth [11] must fulfil the minimum guardband requirement specified in subclause 5.3.3.

-------------------------- End of Text Proposal for TS 38.101-2 Subclause 5.3.4 ------------------------