3GPP TSG-RAN WG4 Meeting #88bis	R4-1813503

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	6.7.7.1.1

Source: 	Qualcomm Incorporated

Title: 	Simulation result for slot-based PDSCH demodulation in TDD

Document for:	Discussion

# Introduction

In this contribution, we present the simulation result for slot-based SPDSCH demodulation requirement in TDD.

# Simulation results

Figure 1 and 2 show the simulation results for slot-based SPDSCH throughput performance for CRS-based and DMRS-based transmission in TDD based on the FRC shown in Table 1 and 2, respectively.

Table 1: Fixed Reference Channel Slot-PDSCH (Cell-Specific Reference Symbols)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.1 TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Uplink-Downlink Configurtion |  | 1 |
| Allocated subframes per Radio Frame (D) |  | 2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 |  | 2 |
| Slot index 1 |  | 4 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 22400 |
| Slot index 1 | Bits | 28288 |
| For Sub-Frame 0,1,5,6 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 5.112 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission only happens on slot 0. For an information bit payload first transmitted at slot 1, any retransmission only happens on slot 1. |  |  |

Table 2: Fixed Reference Channel Slot-PDSCH (User-Specific Reference Symbols)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.2 TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Uplink-Downlink Configurtion |  | 1 |
| Allocated subframes per Radio Frame (D) |  | 2 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 2856 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frames 0,1,5,9 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 9600 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frame 0,1,5,6 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 1.4496 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission only happens on slot 0. For an information bit payload first transmitted at slot 1, any retransmission only happens on slot 1. |  |  |

It is observed that the 70% of the maximum throughput can be achieved at 11.5dB SNR for CRS-based SPDSCH transmission, and 5dB SNR for DMRS-based SPDSCH transmission, respectively.

Figure 1. Slot-based CRS-based SPDSCH throughput in TDD

Figure 2. Slot-based DMRS-based SPDSCH throughput in TDD

Observation 1. For CRS-based slot-based SDPSCH in TDD, 70% of the maximum configured throughput is achieved at 11.5dB without considering any impairment margin.

Observation 2. For DMRS-based slot-based SDPSCH in TDD, 70% of the maximum configured throughput is achieved at 5dB without considering any impairment margin.

# Conclusions

In this paper, we presented the simulation result for slot-based CRS/DMRS-based SPDSCH transmission in TDD. The observations made in this paper are summarized as follows:

Observation 1. For CRS-based slot-based SDPSCH in TDD, 70% of the maximum configured throughput is achieved at 11.5dB without considering any impairment margin.

Observation 2. For DMRS-based slot-based SDPSCH in TDD, 70% of the maximum configured throughput is achieved at 5dB without considering any impairment margin.