3GPP TSG-RAN WG4 Meeting #88-Bis		  												R4-1813880

Chengdu, China, 08 - 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-1: Data content for FR1

Agenda item:	7.9.2.3

Document for:	Approval

# Introduction

Current TS38.141-1 [1] doesn’t include subclause for data content of physical channels and signals for NR test models. Such subclause currently exists in E-UTRA specification and provide more information and details related to physical channels and signals for test models and provides references to TS36.211.

In this contribution, we provide TP to 38.141-1 for subclause 4.9.3 that include information and references to TS38.211 [2] on physical channels.

# References

- 3GPP TS 38.141-1 V1.0.0 (2018-09) 3rd Generation Partnership Project; Technical Specification Group RAN; NR; Base Station (BS) conformance testing; Part 1: Conducted conformance testing (Release 15)

- 3GPP TS 38.211 V15.2.0 (2018-06) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation (Release 15)

<Start of TP>

#### 4.9.2.2 	NR test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	[The test models are defined for a single antenna port, 1 layer, precoding is not used; unless specified otherwise]

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	Normal CP

-	Virtual resource blocks of localized type

<Unchanged sections>

### 4.9.2.3	Data content of Physical channels and Signals for NR-FR1-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS38.211, Clause 5.2 [xx] which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS38.211 [xx] use the following additional parameters:

-	[ = 1 for the lowest configured carrier,  = 2 for the 2nd lowest configured carrier,…,  = n for the nth configured carrier]

-  Antenna ports starting with 1000 for PDSCH

- 	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

#### 4.9.2.3.1	PDCCH

-	PDCCH modulation to be QPSK as described in TS38.211, clause 5.1.3

-	For each subframe the required amount of bits for all PDCCHs is as follows: (# of PDCCH)*(# of CCE per PDCCH)* 6(REG per CCE)*12(RE per REG)*2(bits per RE) with these parameters according to the NR -FR1-TM definitions in subclause 4.9.2.2

-	Generate this amount of bits according to ‘all 0’ data

-

-	1 CCE shall be according to TS38.211, clause 7.3.2 using non-interleaved CCE-to-REG mapping. PDCCH should occupy first symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	[Perform PDCCH multiplexing and scrambling according to TS38.211, clause 7.3.2.3]

-	Perform mapping to REs according to TS38.211, clause 7.3.2.5

-	NID = 0 in DM-RS sequence generation in TS38.211, clause 7.4.1.3

#### 4.9.2.3.2		PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data

-	NR-FR1-TMs utilize 1 user PDSCH transmissions distinguished by .

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR1-TM

| NR-FR1-TM1.1 | 0 for all PRBs |
| --- | --- |
| NR-FR1-TM1.2 | 0 for boosted PRBs 1 for de-boosted PRBs |
| NR-FR1-TM2 | 0 for all PRBs |
| NR-FR1-TM2a | 0 for all PRBs |
| NR-FR1-TM3.1 | 0 for all PRBs |
| NR-FR1-TM3.1a | 0 for all PRBs |
| NR-FR1-TM3.2 | 0 for QPSKPRBs 1 for 16QAM PRBs |
| NR-FR1-TM3.3 | 0 for 16QAM PRBs 1 for QPSK PRBs |

- 	The required amount of PDSCH ‘0’ bits within slot and allocated PRBs shall be generated

-	[Perform user specific scrambling according to TS38.211, clause 7.3.1.1.]

-	[Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS38.211, clause 6.3.2]

-	Perform mapping of the complex-valued symbols to layer according to TS38.211, clause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS38.211

-	DM-RS sequence generation according to TS38.211, clause 7.4.1.1.1 where l is the OFDM symbol number within the slot, using symbol 3 and symbol 11 for every other subcarrier

-	DM-RS mapping according to TS38.211, clause 7.4.1.1.2 where type 1 configuration is considered

<End of TP>