3GPP TSG-RAN WG4 Meeting RAN4#88bis                                               R4-1814236

Chengdu ,China, 8 -12 Oct, 2018

Agenda Item:   7.13.1.1

Source:        Samsung

Title:          Template for CSI and PDSCH test parameters in TS38.101-4

Document for:  Discussion

# Introduction

In order to better align specification drafting for introducing performance requirements in TS38.101-4, template of PDSCH and CSI requirements test parameters were discussed among interested companies through email. This contribution provided template for test parameters based on offline input from Volunteers.

# Discussion

## Template for PDSCH test parameters (provided by Intel)

Table x. Common test parameters

| Parameter |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 | So far, only ” Transmission scheme 1” is supported |
| EPRE ratio of PTRS to PDSCH |  | dB |  | In FR1, NA |
| DL BWP configuration #1 | Cyclic prefix |  |  | Values: Normal/Extended Note: Extended is only supported for 60 kHz SCS |
| Common serving cell parameters | Physical Cell ID |  |  | Default value 0 |
|  | SSB position in burst |  |  | Time domain positions of the transmitted SSB in an burst (corresponds to RRC parameter ssb-PositionsInBurst) |
|  | SSB periodicity | ms |  | SSB periodicity (corresponds to RRC parameter ssb-periodicityServingCell) |
|  | First DMRS position for Type A PDSCH mapping |  |  |  |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | PDCCH TX parameters for PDSCH tests |
|  | Symbols with PDCCH |  |  |  |
|  | Number of PDCCH candidates and aggregation levels |  |  |  |
|  | DCI format |  |  |  |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | k0 parameter in TS 38.211 Clause 7.4.1.5.3 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | CSI-RS resource 1: TBD CSI-RS resource 2: TBD CSI-RS resource 3: TBD CSI-RS resource 4: TBD | l0 parameter in TS 38.211 Clause 7.4.1.5.3 for each CSI-RS resource l0 = {4,5,6} for CSI-RS resource 1 and 3 l0 = {8,9,10} for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 | Always 1 for CSI-RS for tracking |
|  | CDM Type |  | No CDM | Always “No CDM” for CSI-RS for tracking |
|  | Density (ρ) |  | 3 | Always “3” for CSI-RS for tracking |
|  | CSI-RS periodicity | Slots |  | Current agreement corresponds to: 20 for 15 kHz SCS 40 for 30 kHz SCS |
|  | CSI-RS offset | Slots |  | CSI-RS offset for each CSI-RS resource Current agreement corresponds to: 10 for 15 kHz SCS for CSI-RS resource 1 and 2 11 for 15 kHz SCS for CSI-RS resource 3 and 4 20 for 30 kHz SCS for CSI-RS resource 1 and 2 21 for 30 kHz SCS for CSI-RS resource 3 and 4 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | k0 parameter in TS 38.211 Clause 7.4.1.5.3 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | l0 parameter in TS 38.211 Clause 7.4.1.5.3 |
|  | Number of CSI-RS ports (X) |  |  | Values: 2 or 4 |
|  | CDM Type |  |  | Only FD-CDM2 for 2 and 4 CSI-RS ports |
|  | Density (ρ) |  |  | Values: 1 or 0.5 for 2 ports, 1 for 4 ports |
|  | CSI-RS periodicity |  |  |  |
|  | CSI-RS offset |  |  |  |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | ZP CSI-RS configuration should be aligned with one of the possible CSI-IM configurations |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  |
|  | CDM Type |  |  |  |
|  | Density (ρ) |  |  |  |
|  | CSI-RS periodicity |  |  |  |
|  | CSI-RS offset |  |  |  |
| PTRS configuration | Frequency density (KPT-RS) |  |  | KPT-RS parameter in TS 38.211. Values: 2 or 4 |
|  | Time density (LPT-RS) |  |  | LPT-RS parameter in TS 38.211. Values: 1, 2 or 4 |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 | At current stage we consider only TB-based HARQ processing |
| Maximum number of HARQ transmission |  |  | 4 |  |
| HARQ ACK/NACK bundling |  |  |  |  |
| Redundancy version coding sequence |  |  | {0,2,3,1} |  |
| Precoding configuration |  |  | SP Type I, Random per slot with PRB bundling granularity |  |
| Symbols for all unused REs |  |  |  | OCNG type |
| Note 1: at antenna port and SSS transmit power are specified in Clause TBD. |  |  |  |  |

Table Y. Test parameters

| Parameter |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz |  | Further discussion is needed whether this is UE CBW or BW of the active BWP |
| Duplex mode |  |  | FDD or TDD | For TDD detailed configurations are captured Annex A |
| Active DL BWP index |  |  | 1 | Active BWP index |
| DL BWP configuration #1 | First PRB |  |  | As defined in TS 36.213 Section 12 |
|  | Number of contiguous PRB |  |  | As defined in TS 36.213 Section 12 |
|  | Subcarrier spacing | kHz |  | Values: 15/30/60/120 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs |  |  |
| PDSCH configuration | Mapping type |  |  | As defined in TS 38.214 Section 5.1.2.1 Type A or Type B |
|  | k0 |  |  | As defined in TS 38.214 Section 5.1.2.1 |
|  | Starting symbol (S) |  |  | As defined in TS 38.214 Section 5.1.2.1 |
|  | Length (L) |  |  | As defined in TS 38.214 Section 5.1.2.1 |
|  | PDSCH aggregation factor |  |  | As defined in TS 38.214 Section 5.1.2.1 |
|  | PRB bundling type |  |  | As defined in TS 38.214 Section 5.1.2.3. Values: “Static” or “Dynamic” |
|  | PRB bundling size |  |  | As defined in TS 38.214 Section 5.1.2.3. Values: 2, 4 or WB |
|  | Resource allocation type |  |  | Values: 0 or 1 |
|  | VRB-to-PRB mapping type |  |  | Values: Interleaved / Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | Applicable when Interleaved mapping is used Values: 2 or 4 PRBs |
| PDSCH DMRS configuration | DMRS Type |  |  | As defined in TS 38.211 Section 7.4.1.1. Values: 1 or 2 |
|  | Number of additional DMRS |  |  | As defined in TS 38.211 Section 7.4.1.1. Values: 0, 1, 2, 3 |
|  | Length |  |  | As defined in TS 38.211 Section 7.4.1.1. Values: “Single-symbol DM-RS” or “Double-symbol DM-RS” |
|  | Antenna ports indexes |  |  | Values: 1000 -1007 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | Values: 1 or 2 |
| Number of HARQ Processes |  |  |  |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  | PDSCH-to-HARQ-timing-indicator parameter in DCI. For FDD this parameter is same for all tests. For TDD this parameter depends on UL-DL patter and is captured in Annex |

Table Z1: Performance requirements (FDD)

| Test num. | Reference channel | Modulation format [and Rank] | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 |  |  |  |  |  | [TBD] |
| 2 |  |  |  |  |  | [TBD] |

Note: For FR2 test cases “SNR” should be replaced by “SNRBB”

Table Z2: Performance requirements (TDD)

| Test num. | Reference channel | Modulation format [and Rank] | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 |  |  |  |  |  |  | [TBD] |
| 2 |  |  |  |  |  |  | [TBD] |

## Template for CSI requirements (provided by Samsung)

Table X Common Test parameters

| Parameter |  |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 | So far, only ” Transmission scheme 1” is supported |
| EPRE ratio of PTRS to PDSCH |  |  | dB |  | Test specific parameter based on 38.101-1/2. For the most part of test cases it will be constant. |
| Active DL BWP index |  |  |  | 1 | Active BWP index |
| Cyclic prefix |  |  |  | Normal | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 | Default value 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 | Time domain positions of the transmitted SSB in an burst (corresponds to RRC parameter ssb-PositionsInBurst) |
|  | SSB periodicity |  | ms | 20 | SSB periodicity (corresponds to RRC parameter ssb-periodicityServingCell) |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot | PDCCH TX parameters for CSI tests |
|  | Symbols with PDCCH |  |  | 0,1 |  |
|  | Number of PDCCH candidates and aggregation levels |  |  | TBD |  |
|  | DCI format |  |  | TBD |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| PDSCH configuration | Mapping type |  |  | Type A | As defined in TS 38.214 Section 5.1.2.1 Type A or Type B |
|  | k0 |  |  | 0 | As defined in TS 38.214 Section 5.1.2.1 |
|  | Starting symbol (S) |  |  | 2 | As defined in TS 38.214 Section 5.1.2.1 |
|  | Length (L) |  |  | 12 | As defined in TS 38.214 Section 5.1.2.1 |
|  | PDSCH aggregation factor |  |  | 1 | As defined in TS 38.214 Section 5.1.2.1 |
|  | PRB bundling type |  |  | Static | As defined in TS 38.214 Section 5.1.2.3. Values: “Static” or “Dynamic” |
|  | PRB bundling size |  |  | 2 | As defined in TS 38.214 Section 5.1.2.3. Values: 2, 4 or WB |
|  | Resource allocation type |  |  | 0 | Values: 0 or 1 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved | Values: Interleaved / Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | TBD | Applicable when Interleaved mapping is used Values: 2 or 4 PRBs |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 | As defined in TS 38.211 Section 7.4.1.1. Values: 1 or 2 |
|  | Number of additional DMRS |  |  | 1 | As defined in TS 38.211 Section 7.4.1.1. Values: 0, 1, 2, 3 |
|  | Length |  |  | Single-symbol DM-RS | As defined in TS 38.211 Section 7.4.1.1. Values: “Single-symbol DM-RS” or “Double-symbol DM-RS” |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 2 | Values: 1 or 2 |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | Only for FR2 KPT-RS parameter in TS 38.211. Values: 2 or 4 |
|  | Time density (LPT-RS) |  |  |  | Only for FR2 LPT-RS parameter in TS 38.211. Values: 1, 2 or 4 |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | [0] | k0 parameter in TS 38.211 Clause 7.4.1.5.3 |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | [4] | l0 parameter in TS 38.211 Clause 7.4.1.5.3 for each CSI-RS resource l0 = {4,5,6} for CSI-RS resource 1 and 3 l0 = {8,9,10} for CSI-RS resource 2 and 4 |
|  |  | Number of CSI-RS ports (X) |  | 1 | Always 1 for CSI-RS for tracking |
|  |  | CDM Type |  | No CDM | Always “No CDM” for CSI-RS for tracking |
|  |  | Density (ρ) |  | 3 | Always “3” for CSI-RS for tracking |
|  |  | CSI-RS periodicity | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 | Current agreement corresponds to: 20 for 15 kHz SCS 40 for 30 kHz SCS |
|  |  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 | CSI-RS offset for each CSI-RS resource Current agreement corresponds to: 10 for 15 kHz SCS for CSI-RS resource 1 and 2 11 for 15 kHz SCS for CSI-RS resource 3 and 4 20 for 30 kHz SCS for CSI-RS resource 1 and 2 21 for 30 kHz SCS for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  |  |  | 4 For FDD 8 for TDD |
| HARQ ACK/NACK bundling |  |  |  |  |  |
| Redundancy version coding sequence |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  |  |
| Symbols for unused REs |  |  |  |  | OCNG as specified in A.5 |
| Note 1: at antenna port and SSS transmit power are specified in Clause TBD. |  |  |  |  |  |

Place in general session of CSI requirements i.e. section 6.1 and section 8.1

Add note for Noc and SSS transmission power to refer to general section or annex

Table Y: test parameters

| Parameter |  |  |  | Unit | Test 1 | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz |  | Channel/BWP Bandwidth |
| Duplex Mode |  |  |  |  |  | FDD, TDD For TDD detailed configuration can be captured in Annex FRC |
| TDD Slot Configuration |  |  |  |  |  | If Annex A specified detailed pattern, only need to capture pattern number here |
| DL BWP configuration #1 |  |  | First PRB |  |  |  |
|  |  |  | Number of contiguous PRB |  |  |  |
|  |  |  | Subcarrier spacing | kHz |  |  |
| SNR |  |  |  | dB |  | Only used for CQI/RI test cases |
| Propagation channel |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  |
| Beamforming Model |  |  |  |  |  |  |
| DMRS port indexes |  |  |  |  |  | Port 10000, Port 1000,10001 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  |  | Periodic, Aperiodic |
|  | Number of CSI-RS ports (X) |  |  |  |  | 4 by default |
|  | CDM Type |  |  |  |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  |
|  | CSI-RS periodicity and offset |  |  |  |  | Applied for periodic CSI-RS resources For aperiodic CSI, change as interval and offset |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  |  | Periodic, aperiodic , semi-persistent |
|  | Number of CSI-RS ports (X) |  |  |  |  | 2, 4, 8 Port 3000, .. |
|  | CDM Type |  |  |  |  | fd-CDM2, cdm4-FD2-TD2 |
|  | Density (ρ) |  |  |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  | 5 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  |  |  | For aperiodic CSI, change as interval and offset |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  |  |  | For aperiodic CSI, change as interval and offset |
| ReportConfigType |  |  |  |  |  | Periodic , Aperiodic |
| CQI-table |  |  |  |  |  | Table 1 , Table 2 |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | configured, Not configured, Default not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | configured, Not configured, Default not configured |
| cqi-FormatIndicator |  |  |  |  |  | Wideband, Subband |
| pmi-FormatIndicator |  |  |  |  |  | Wideband, Subband |
| Sub-band Size |  |  |  | RB |  | Configured for subband CQI/PMI |
| CSI-Report periodicity and offset |  |  |  |  |  | Configured for periodic CSI report For aperiodic CSI, change as CSI reporting interval and offset |
| aperiodicTriggeringOffset |  |  |  |  |  | Configured for aperiodic CSI |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |
|  |  | Codebook Mode |  |  |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | Configured for Tx ports>=4 |
|  |  | CodebookSubsetRestriction |  |  |  | Configured for 2Tx ports |
|  |  | RI Restriction |  |  |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH/PUCCH |
| CQI/RI/PMI delay |  |  |  | ms |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | PMI 4 and 1 for other test cases |
| Measurement channel |  |  |  |  |  | Configured for PMI test cases |

Orders: SYS parameters->CSI-RS resources configurations (ZP->NZP->CSI-IM)->CSI report configuration->codebook configurations

## Naming of TDD DL-UL configurations and Channel model

Details of UL-DL configuration are provided in Annex A. In Table 1 connection between TDD UL-DL pattern and naming in Annex A is provided

Table 1. TDD UL-DL configuration naming

| FR and SCS | TDD UL-DL pattern | Naming from Annex A |
| --- | --- | --- |
| FR1, 15 kHz | DDDSU | FR1.15-1 |
| FR1, 30 kHz | 7DS2U | FR1.30-1 |
|  | DDDSU | FR1.30-2 |
|  | DDDSUDDSUU | FR1.30-3 |
|  | SU | FR1.30-4 |
|  | DDSU | FR1.30-5 |
|  | DS1S2U | FR1.30-6 |
| FR2, 60 kHz | DDSU | FR1.60-1 |
| FR2, 120 kHz | DDDSU | FR1.120-1 |
|  | DDSU | FR1.120-2 |

In Table 2 naming for previously agreed channel model is provided.

Table 2. Channel model naming

| TDL-A, 30ns, 5Hz | TDLA30-5 |
| --- | --- |
| TDL-A, 30ns, 10Hz | TDLA30-10 |
| TDL-B, 100ns, 400 Hz | TDLB100-400 |
| TDL-C, 300ns, 100 Hz | TDLC300-100 |
| TDL-A, 30ns, 75Hz | TDLA30-75 |
| TDL-A, 30ns, 300Hz | TDLA30-300 |

# Conclusion

This contribution provided template for test parameters on introducing performance requirements into TS38.101-4 based on offline input from Volunteers.

# Reference