3GPP TSG-RAN WG4 Meeting #89	R4-1814441

Spokane, US, 12 - 16 November 2018

Source:	China Telecom

Title:	TP to TS 38.141-2: Radiated test requirements for DFT-s-OFDM based PUSCH

Agenda Item:	7.13.2.2.2

Document for:	Approval

- Introduction

It has been agreed in RAN4 to test DFT-s-OFDM based PUSCH, and the radiated conformance test requirements are still absent in the specifications.

This TP introduces the radiated conformance test requirements for DFT-s-OFDM based PUSCH.

- Text Proposal

< Start of first change >

8.2	OTA performance requirements for PUSCH

8.2.2	Performance requirements for PUSCH with transmission precoding enabled

8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

A test for a specific channel bandwidth is only applicable if the BS supports it.

The applicability of tests in TS 38.104 [2] with different SCS and BW combinations is according to the following principle:

Editor’s note: Applicability rule is FFS.

8.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.2.

8.2.2.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.4	Method of test

8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.36).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.62).

8.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in Annex [E.x.x]. Each of the RX antenna signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in Annex A, and according to additional test parameters listed in Table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DMRS | DMRS configuration type | 1 |  |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |  |
|  | Number of additional DMRS symbols | 0, 1 | 0 |
|  | Number of DMRS CDM group(s) without data | 2 |  |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |  |
|  | DMRS port | 0 |  |
|  | DMRS sequence generation | NID=0, group hopping and sequence hopping are disabled |  |
| Time domain resource | PUSCH mapping type | A | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Thresholds for frequency density | N.A. | NRB0=0, NRB1=8, NRB2=NRB3=32, NRB4=108 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex TBD.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.2.5.1 and 8.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 kHz | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  | 120 kHz | 100 | TBD |

8)	For reference channels applicable to the BS, measure the throughput, according to [TBD].

8.2.2.5	Test Requirement

8.2.2.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.2.5.1-1 to Table 8.2.2.5.1-2.

Table 8.2.2.5.1-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 | 70 % | G-FR1-A3-29 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |

Table 8.2.2.5.1-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 | 70 % | G-FR1-A3-30 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [TBD] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex C.

8.2.2.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.2.5.2-1 to Table 8.2.2.5.2-2.

Table 8.2.2.5.2-1 Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | G-FR2-A3-11 | 1+0 | [TBD] |

Table 8.2.2.5.2-2 Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 70 % | G-FR2-A3-12 | 1+0 | [TBD] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex C.

< End of first change >