3GPP TSG-RAN WG4 Meeting #89               	R4-1814476

Spokane, US, 12th – 16th November 2018

Agenda item:	7.13.2.4.1

Source:	Samsung

Title:	Discussion and simulation results for NR PRACH

Document for:	Discussion

# 1	Introduction

During RAN4 #88bis meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PRACH performance requirements are captured and agreed in the WF [1].

In this contribution, the initial ideal and impairment are provided for alignment purpose.

# 2	Simulation Results

In this section, the initial simulation results for NR PRACH are provided as shown in Table 1 and Table 2 for FR1 and FR2.

As for timing offset setting, format A1 with 30, 60, 120KHz SCS, which has smallest CP length compared with other format, obviously, timing offset based on Ncs/2 is large then the CP length of format A1. Considering timing offset, as well as channel delay spread within the CP length of Format A1, we prefer time offset for Format A1 is 0. For other format, Ncs/2 is applied for the initial timing offset.

Table 1: Simulation assumption for NR PRACH in FR1

| Parameters | Format0 | Format A1 | Format A2 | Format A3 | Format B4 | Format C0 | FormatC2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz |
| SCS | 1.25KHz | 15KHz, 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz |
| Rx | 2.,4,8 | 2.,4,8 | 2.,4,8 | 2.,4,8 | 2.,4,8 | 2.,4,8 | 2.,4,8 |
| Propagation channel | AWGN /TDLC300-100 | AWGN /TDLC300-100 | AWGN /TDLC300-100 | AWGN /TDLC300-100 | AWGN/ TDLC300-100 | AWGN /TDLC300-100 | AWGN /TDLC300-100 |
| Logical sequence index | 22 | 0 | 0 | 0 | 0 | 0 | 0 |
| Preamble index | 32 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ncs (15kHz) | 13 | 23 | 23 | 23 | 23 | 23 | 23 |
| Ncs(30kHz) | 13 | 46 | 46 | 46 | 46 | 46 | 46 |
| Set type | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted |

Table 2: Simulation assumption for NR PRACH in FR2

| Parameters | Format A1 | Format A2 | Format A3 | Format B4 | Format C0 | FormatC2 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| SCS | 60KHz, 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz |
| Rx | 2 | 2 | 2 | 2 | 2 | 2 |
| Propagation channel | AWGN /TDL30-300 | AWGN/ TDL30-300 | AWGN/ TDL30-300 | AWGN/ TDL30-300 | AWGN/ TDL30-300 | AWGN/ TDL30-300 |
| Logical sequence index | 0 | 0 | 0 | 0 | 0 | 0 |
| Preamble index | 0 | 0 | 0 | 0 | 0 | 0 |
| Ncs | 69 | 69 | 69 | 69 | 69 | 69 |
| Set type | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted |

The ideal and impairment simulation results can be referred as Table 3 and Table 4 in FR1 and FR2, respectively

Table 3: ideal and impairment simulation results for NR PRACH in FR1 with 2Rx

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 | -16.38 | -14.38 | -8.48 | -6.48 |
| Format A1 | 15 | -10.86 | -8.86 | -3.68 | -1.68 |
| Format A1 | 30 | -10.75 | -8.75 | -4.70 | -2.70 |
| Format A2 | 15 | -13.29 | -11.29 | -6.10 | -4.10 |
| Format A2 | 30 | -13.16 | -11.16 | -6.99 | -4.99 |
| Format A3 | 15 | -14.54 | -12.54 | -7.46 | -5.46 |
| Format A3 | 30 | -14.45 | -12.45 | -8.22 | -6.22 |
| Format B4 | 15 | -16.80 | -14.80 | -9.58 | -7.58 |
| Format B4 | 30 | -16.67 | -14.67 | -10.27 | -8.27 |
| Format C0 | 15 | -8.29 | -6.29 | -1.29 | 0.71 |
| Format C0 | 30 | -8.13 | -6.13 | -2.22 | -0.22 |
| Format C2 | 15 | -13.27 | -11.27 | -6.06 | -4.06 |
| Format C2 | 30 | -13.16 | -11.16 | -7.06 | -5.06 |

Table 4: ideal and impairment simulation results for NR PRACH in FR1 with 4Rx (to be updated)

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 |  |  |  |  |
| Format A1 | 15 |  |  |  |  |
| Format A1 | 30 |  |  |  |  |
| Format A2 | 15 |  |  |  |  |
| Format A2 | 30 |  |  |  |  |
| Format A3 | 15 |  |  |  |  |
| Format A3 | 30 |  |  |  |  |
| Format B4 | 15 |  |  |  |  |
| Format B4 | 30 |  |  |  |  |
| Format C0 | 15 |  |  |  |  |
| Format C0 | 30 |  |  |  |  |
| Format C2 | 15 |  |  |  |  |
| Format C2 | 30 |  |  |  |  |

Table 5: ideal and impairment simulation results for NR PRACH in FR1 with 8Rx (to be updated)

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 |  |  |  |  |
| Format A1 | 15 |  |  |  |  |
| Format A1 | 30 |  |  |  |  |
| Format A2 | 15 |  |  |  |  |
| Format A2 | 30 |  |  |  |  |
| Format A3 | 15 |  |  |  |  |
| Format A3 | 30 |  |  |  |  |
| Format B4 | 15 |  |  |  |  |
| Format B4 | 30 |  |  |  |  |
| Format C0 | 15 |  |  |  |  |
| Format C0 | 30 |  |  |  |  |
| Format C2 | 15 |  |  |  |  |
| Format C2 | 30 |  |  |  |  |

Table6: ideal and impairment simulation results for NR PRACH in FR2  (to be updated)

| Format | SCS(KHz) | AWGN |  | TDLA30-300 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format A1 | 60 | -10.6 |  |  |  |
| Format A1 | 120 | -13.2 |  |  |  |
| Format A2 | 60 | -14.5 |  |  |  |
| Format A2 | 120 | -16.5 |  |  |  |
| Format A3 | 60 | -8.1 |  |  |  |
| Format A3 | 120 | -13.0 |  |  |  |
| Format B4 | 60 | -10.6 |  |  |  |
| Format B4 | 120 | -13.2 |  |  |  |
| Format C0 | 60 | -14.5 |  |  |  |
| Format C0 | 120 | -16.5 |  |  |  |
| Format C2 | 60 | -8.1 |  |  |  |
| Format C2 | 120 | -13.0 |  |  |  |

# 3	Conclusion

In this contrition, both ideal and impairment simulation results are provided.

# References

R4-1813757, WF on NR PRACH performance requirement. Ericsson