3GPP TSG-RAN WG4 Meeting RAN4#89                                              R4-1814491

Spokane ,US, 12 -16 Nov, 2018

Agenda Item:   7.13.1.4.2

Source:        Samsung

Title:          Simulation results for CQI test cases

Document for:  Discussion

# Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (Wideband) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2 - Sub-band CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4

In this contribution, simulation results for CQI test cases were provided.

# Discussion

## Simulation results for static CQI test cases

The static CQI test cases were summarized in below tables.

Table 1: Static CQI results for FR1 FDD

| SNR [dB] | FR1 FDD 2*2 |  |  | FR1 FDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI Index in {median CQI1 -1, median CQI1 +1} (%) | BLER with following median CQI-1 | BLER with following median CQI+1 | Percentage of Reported CQI Index in {median CQI1 -1, median CQI1 +1} (%) | BLER with following median CQI-1 | BLER with following median CQI+1 |
| 0 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 2 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 4 | 100.00 | 0.00 | 0.95 | 100.00 | 0.00 | 1.00 |
| 6 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 8 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 10 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 12 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 14 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 16 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 18 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |
| 20 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 1.00 |

Table 2: Static CQI results for FR1 TDD

| SNR [dB] | FR1 TDD 2*2 |  |  | FR1 TDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI Index in {median CQI1 -1, median CQI1 +1} (%) | BLER with following median CQI-1 | BLER with following median CQI+1 | Percentage of Reported CQI Index in {median CQI1 -1, median CQI1 +1} (%) | BLER with following median CQI-1 | BLER with following median CQI+1 |
| 0 | 100.00 | 0.00 | 0.88 | 100.00 | 0.00 | 1.00 |
| 2 | 100.00 | 0.00 | 1.00 | 100.00 | 0.00 | 0.89 |
| 4 | 100.00 | 0.00 | 0.59 | 100.00 | 0.00 | 0.88 |
| 6 | 100.00 | 0.00 | 0.29 | 100.00 | 0.00 | 0.88 |
| 8 | 100.00 | 0.00 | 0.75 | 100.00 | 0.00 | 0.88 |
| 10 | 100.00 | 0.00 | 0.02 | 100.00 | 0.00 | 0.88 |
| 12 | 100.00 | 0.00 | 0.01 | 100.00 | 0.00 | 0.88 |
| 14 | 100.00 | 0.00 | 0.01 | 100.00 | 0.00 | 0.88 |
| 16 | 100.00 | 0.00 | 0.10 | 100.00 | 0.00 | 0.88 |
| 18 | 100.00 | 0.00 | 0.17 | 100.00 | 0.00 | 0.88 |
| 20 | 100.00 | 0.00 | 0.88 | 100.00 | 0.00 | 0.88 |

Table 3: Static CQI results for FR2 TDD

| SNR [dB] | FR2 TDD 2*2 |  |  |
| --- | --- | --- | --- |
|  | Percentage of Reported CQI Index in {median CQI1 -1, median CQI1 +1} (%) | BLER with following median CQI-1 | BLER with following median CQI+1 |
| 0 | 100.00 | 0.00 | 1.00 |
| 2 | 100.00 | 0.00 | 0.96 |
| 4 | 100.00 | 0.00 | 0.96 |
| 6 | 100.00 | 0.00 | 0.76 |
| 8 | 100.00 | 0.00 | 0.76 |
| 10 | 100.00 | 0.00 | 1.00 |
| 12 | 100.00 | 0.00 | 0.75 |
| 14 | 100.00 | 0.00 | 0.67 |
| 16 | 100.00 | 0.00 | 1.00 |
| 18 | 100.00 | 0.00 | 0.64 |
| 20 | 100.00 | 0.00 | 0.58 |

Such observations and proposals were given for static CQI test cases based on evaluation results:

Observation1: It’s feasible to reuse LTE test metric for static CQI test cases of both FR1 and FR2

Proposal 1: Introduce below test metric for static CQI test cases

- The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Proposal 2: introduce below test SNR points

- FR1 (FDD and TDD) 2Rx:[8/9]dB, [14/15]dB

- FR1 (FDD and TDD) 4Rx:[5/6]dB, [11/12]dB

- FR2 2Rx:[8/9]dB, [14/15]dB

## Simulation results for wideband fading CQI test cases

In below tables, simulation results were provided for fading CQI test cases.

Table 4: Wideband fading CQI test case for FR1 FDD

| SNR [dB] | FR1 FDD 2*2 |  |  | FR1 FDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 20.00 | 0.08 | 1.46 | 1.65 | 0.14 | 1.24 |
| 2 | 22.20 | 0.04 | 1.41 | 3.35 | 0.19 | 1.37 |
| 4 | 30.45 | 0.04 | 1.37 | 8.30 | 0.11 | 1.31 |
| 6 | 34.75 | 0.06 | 1.38 | 11.10 | 0.08 | 1.35 |
| 8 | 38.35 | 0.07 | 1.60 | 10.30 | 0.07 | 1.21 |
| 10 | 36.50 | 0.09 | 1.68 | 11.10 | 0.07 | 1.30 |
| 12 | 30.85 | 0.09 | 1.27 | 18.40 | 0.07 | 1.18 |
| 14 | 39.40 | 0.11 | 1.31 | 19.40 | 0.09 | 1.42 |
| 16 | 52.75 | 0.11 | 1.65 | 22.20 | 0.11 | 1.22 |
| 18 | 38.15 | 0.14 | 1.16 | 15.85 | 0.13 | 1.34 |
| 20 | 31.55 | 0.16 | 1.09 | 5.80 | 0.18 | 1.18 |

Table 5: Wideband fading CQI test case for FR1 TDD

| SNR [dB] | FR1 TDD 2*2 |  |  | FR1 TDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 7.86 | 0.24 | 1.07 | 3.94 | 0.21 | 1.27 |
| 2 | 10.60 | 0.20 | 1.42 | 6.72 | 0.17 | 1.21 |
| 4 | 23.84 | 0.12 | 1.18 | 9.24 | 0.18 | 1.38 |
| 6 | 19.00 | 0.19 | 1.16 | 12.12 | 0.16 | 1.26 |
| 8 | 7.72 | 0.22 | 1.50 | 10.00 | 0.11 | 1.22 |
| 10 | 14.00 | 0.16 | 1.75 | 9.88 | 0.10 | 1.34 |
| 12 | 14.64 | 0.13 | 1.66 | 14.00 | 0.09 | 1.25 |
| 14 | 8.40 | 0.10 | 1.70 | 16.16 | 0.10 | 1.33 |
| 16 | 2.04 | 0.08 | 1.11 | 21.04 | 0.10 | 1.33 |
| 18 | 3.28 | 0.08 | 1.18 | 14.24 | 0.09 | 1.29 |
| 20 | 7.86 | 0.04 | 1.36 | 4.20 | 0.12 | 1.21 |

Table 6: Wideband fading CQI test case for FR2 TDD

| SNR [dB] | FR2 TDD 2*2 (Periodic with 64 slots delay) |  |  | FR2 TDD 2*2 (aperiodic with 27 slots delay) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 7.36 | 0.3714 | 0.7981 | 7.4985 | 0.3042 | 0.9410 |
| 2 | 6.48 | 0.3676 | 0.9582 | 6.3587 | 0.3026 | 1.1083 |
| 4 | 7.04 | 0.3102 | 1.0084 | 7.3785 | 0.235 | 1.1487 |
| 6 | 29.92 | 0.316 | 0.7576 | 30.174 | 0.2544 | 0.8585 |
| 8 | 19.52 | 0.4022 | 0.9517 | 19.796 | 0.332 | 1.1044 |
| 10 | 3.28 | 0.3362 | 0.9995 | 3.1794 | 0.272 | 1.1368 |
| 12 | 6.64 | 0.3286 | 0.7692 | 6.4187 | 0.2524 | 0.8851 |
| 14 | 8.4 | 0.3606 | 0.7514 | 8.5783 | 0.2686 | 0.8845 |
| 16 | 16.96 | 0.3632 | 0.7353 | 16.9766 | 0.282 | 0.8456 |
| 18 | 20.08 | 0.3232 | 1.1590 | 20.6959 | 0.2414 | 1.3160 |
| 20 | 3.12 | 0.129 | 1.0019 | 3.0594 | 0.1028 | 1.0386 |

Based on above simulation results, we have below proposals:

Observation 2: For FR2 wideband CQI test cases, current test set-up not feasible to introduce test cases:

- Under the assumption: periodic CSI with minimum 64 slots (8ms) CSI delay, no performance gain with following CQI.

- Under the assumption: aperiodic CSI with minimum 27 slots CSI delay, no performance gain or even worse than with median CQI transmission

Proposal3: For FR1 2Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>20%

  - BLER>2%

  - Gamma>1.05

Proposal4: For FR1 4Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 3/4dB  +9/10 dB

- Test requirements:

  - Aifa>2%

  - BLER>2%

  - Gamma>1.05

## Simulation results for frequency-selective CQI test cases

In below tables, simulation results were provided for sub-band CQI test cases.

Table 6: Sub-band CQI test case for FR1 FDD

| SNR [dB] | FR1 FDD 2*2 |  |  | FR1 FDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

Table 7: Sub-band CQI test case for FR1 TDD

| SNR [dB] | FR1 TDD 2*2 |  |  | FR1 TDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

# Conclusion

In this contribution, we provided simulation results for CQI test with below observations and proposals.

Static CQI test cases

Observation1: It’s feasible to reuse LTE test metric for static CQI test cases of both FR1 and FR2

Proposal 1: Introduce below test metric for static CQI test cases

- The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Proposal 2: introduce below test SNR points

- FR1 (FDD and TDD) 2Rx:[8/9]dB, [14/15]dB

- FR1 (FDD and TDD) 4Rx:[5/6]dB, [11/12]dB

- FR2 2Rx:[8/9]dB, [14/15]dB

Wideband Fading CQI test cases

Observation 2: For FR2 wideband CQI test cases, current test set-up not feasible to introduce test cases:

- Under the assumption: periodic CSI with minimum 64 slots (8ms) CSI delay, no performance gain with following CQI.

- Under the assumption: aperiodic CSI with minimum 27 slots CSI delay, no performance gain or even worse than with median CQI transmission

Proposal3: For FR1 2Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>20%

  - BLER>2%

  - Gamma>1.05

Proposal4: For FR1 4Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 3/4dB  +9/10 dB

- Test requirements:

  - Aifa>2%

  - BLER>2%

  - Gamma>1.05

Sub-band CQI test cases

Need to be updated…

# Reference

[1] R4-1814235, “Way forward on NR UE CSI requirements”, Samsung

[2] RAN4 email reflector, “Simulation assumption of CSI test cases”, Samsung