3GPP TSG-RAN WG4 Meeting #89	R4-1814496

# Spokane, WA, USA, 12th – 16th Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.102 | CR | 0381 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR protection in TS25.102 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of requirements for the NR FR1 new band of n77, n78 and n79 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following subslause is updated: 6.6.3.1.2 Spurious emissions requirements for1.28 Mcps TDD Option |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements of UTRA TDD UE protection to NR new band n77, n78 and n79 are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS34.122 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

##### 6.6.3.1.2	1.28 Mcps TDD Option

These requirements are only applicable for frequencies which are greater than 4 MHz away from the UE center carrier frequency.

# Table 6.7C: General Spurious emissions requirements (1.28 Mcps TDD Option)

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |

# Table 6.7D: Additional Spurious emissions requirements (1.28 Mcps TDD Option)

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| a | 703 MHz f < 803 MHz | 1 MHz | -50 dBm (note 3) |
|  | 921 MHz f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz f 935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f 960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz f 1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 2010 MHz f 2025 MHz | 1MHz | -65 dBm (Note2) |
|  | 1880 MHz f 1920 MHz | 1MHz | -65 dBm (Note 3) |
|  | 2300 MHz f 2400 MHz | 1MHz | -65 dBm (note 3) |
|  | 2496 MHz f 2690 MHz | 1MHz | -50dBm (note 3) |
|  | 3300 MHz f < 4200 MHz | 1 MHz | -50 dBm (note 3) |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm (note 3) |
| b | 1850 MHz f 1910 MHz | 1 MHz | -65 dBm (Note 4) |
|  | 1930 MHz f 1990 MHz | 1 MHz | -65 dBm (Note 5) |
|  | 2010 MHz f 2025 MHz | 1MHz | -65 dBm |
| c | 2010 MHz f 2025 MHz | 1 MHz | -65 dBm |
| d | 1900 MHz f 1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz f 2025 MHz | 1 MHz | -65 dBm |
|  | 2620 MHz f 2690 MHz | 3.84 MHz | -37 dBm |
| e | 703 MHz f < 803 MHz | 1 MHz | -50 dBm (note1) |
|  | 921 MHz f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz f 935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f 960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz f 1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 1880 MHz f 1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz f 2025 MHz | 1 MHz | -65 dBm |
|  | 2496 MHz f 2690 MHz | 1MHz | -50dBm |
|  | 3400 MHz  f < 3600 MHz | 1 MHz | -50 dBm |
| f | 703 MHz f < 803 MHz | 1 MHz | -50 dBm (note1) |
|  | 921 MHz f<925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz< f <935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz< f <960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz f 1850 MHz | 100 kHz | -71 dBm (note1) |
|  | 2010 MHz f 2025 MHz | 1MHz | -65 dBm |
|  | 2300 MHz f 2400 MHz | 1MHz | -65 dBm |
|  | 2496 MHz f 2690 MHz | 1MHz | -50dBm |
|  | 3300 MHz f < 4200 MHz | 1 MHz | -50 dBm |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm |
| Note 1 The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7c are permitted for each UARFCN used in the measurement. Note 2: This requirement is only applicable when UE operating in 1900-1920MHz of band a. Note 3: This requirement is only applicable when UE operating in 2010-2025MHz of band a. Note 4: This requirement is only applicable when UE operating in 1930-1990MHz of band b. Note 5: This requirement is only applicable when UE operating in 1850-1910MHz of band b. |  |  |  |

<End of changes>