# 3GPP TSG-RAN WG4 Meeting #89                                                            R4-1814504

# Spokane, US, 12th - 16th Nov, 2018

Source: 	ZTE Corporation

Title: 	TP to TS38.141-2: Removal of the multi-band test for BS type 2-O

Agenda Item:	7.9.4.2

Document for:	Approval

1	Introduction

- In this contribution, we provide a TP to TS 38.141-2 to delete the descriptions related to multi-band RIB test for BS type 2-O, including the following requirements:

- OTA transmitter OFF power

- OTA transmitter transient period

- OTA transmitter spurious emissions

- OTA receiver spurious emissions

2	Reference

[1] TS38.141-2, v1.1.0

--------------Start of TP  -------------

6.5	OTA transmit ON/OFF power

6.5.1	OTA transmitter OFF power

6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/Nµs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band co-location test antenna conducted output(s), the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band co-location test antenna conducted output(s) supporting multiple operating bands, the requirement is applicable per supported operating band.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band RIBs supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

6.5.1.4	Method of test

6.5.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for mult-carrier and/or CA:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested:

- The requirement for FR1 is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

- The requirement for FR2 is specified as TRP requirement. The beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP output power level is achieved.

6.5.1.4.2	Procedure

6.5.1.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.1.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.2.

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an NR BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.1.4.2.3	BS type 2-O

[3) Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

4)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.3.

5)	Set the NR BS in the reference direction of the appropriated TRP measurement grid (see annex I).

6)	Measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For an NR BS supporting contiguous CA, measure EIRP by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Repeat step 6-8 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

9)	Calculate TRP using the EIRP measurements.

]

6.5.1.5	Test requirements

6.5.1.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.1.5.2	BS type 2-O

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

6.5.2	OTA transmitter transient period

6.5.2.1	Definition and applicability

The OTA transmitter transient period requirements apply only to TDD operation of BS.

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.5.2.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each co-location test antenna conducted output supporting reception in the operating band for BS type 1-O, and at each RIB supporting transmission in the operating band for BS type 2-O.

6.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.3.3.

6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

6.5.2.4	Method of test

6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested: The requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

6.5.2.4.2	Procedure

6.5.2.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.2.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.2.4.2.3	BS type 2-O

[3)	Place the measurement antenna at the transmitter reference direction

4)	Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

5)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.3.

6)	Measure EIRP sample when transmitter is ON at maximum rated power, call this P_EIRP_ON by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Measure EIRP sample when transmitter is OFF, call this P_EIRP_OFF by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

8)	For an NR BS supporting contiguous CA, measure EIRP by either a) or be below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz, call this P_EIRP_OFF.

9)	 Calculate the difference between EIRP when transmitter is on and off, call this Delta_EIPR. Delta_EIRP = P_EIRP_ON – P_EIRP_OFF

10)	 Estimate the TRP off power as P_TRP_ON – Delta_EIRP.

]

6.5.2.5	Test requirements

6.5.2.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.2.5.2	BS type 2-O

[The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

]

6.7.5	OTA transmitter spurious emissions

6.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA transmitter spurious emissions limits are specified as TRP per RIB, unless otherwise stated.

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1. For some operating bands, the upper limit of the spurious range might be higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [5].

For multi-band RIB the above exclusion applies for each supported operating band.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification.

BS type 1-O requirements consists of OTA transmitter spurious emission requirements based on TRP and co-location requirements not based on TRP.

The OTA transmitter spurious emission limits for FR2 shall apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1.

6.7.5.2	General OTA transmitter spurious emissions requirements

6.7.5.2.1	Definition and applicability

The general OTA transmitter spurious emissions requirements are specified as TRP per RIB, per cell, unless otherwise specified.

6.7.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], subclause 9.7.5.2.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], subclause 9.7.5.3.2.

6.7.5.2.3	Test purpose

The test purpose is to verify if the radiated spurious emissions from the BS at the RIB are within the specified minimum requirements.

6.7.5.2.4	Method of test

6.7.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see subclause 4.9.1:

- For FR1:

  - B when testing from 30 MHz to FDL_low - ΔfOBUE

  - T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

- For FR2:

  - B when testing from 30 MHz to FDL_low - ΔfOBUE

  - T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band multi-carrier operation, see subclause 4.9.1:

- For FR1:

  - BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

  - TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or 5th harmonic)

- For FR2:

  - BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

  - TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in multi-band multi-carrier operation, see subclause 4.9.1:

- For FR1:

  - BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

  - B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

  - BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP output power level is achieved.

<Unchanged are omitted>

7.7	OTA receiver spurious emissions

7.7.1	Definition and applicability

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

Unless otherwise stated, all requirements are measured as mean power.

The OTA receiver spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1. For some operating bands, the upper limit of the spurious range might be higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [5].

For multi-band RIB the above exclusion applies for each supported operating band.

The OTA transmitter spurious emission limits for FR2 shall apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in subclause 6.7.1.

For a BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

For a BS operating in TDD, the OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

The metric used to capture OTA receiver spurious emissions for BS type 1-O and BS type 2-O is total radiated power (TRP), with the requirement defined at the RIB.

7.7.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], subclause 10.7.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], subclause 10.7.3.

7.7.3	Test purpose

The test purpose is to verify if the receiver radiated spurious emissions from the BS at the RIB are within the specified minimum requirements.

7.7.4	Method of test

7.7.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see subclause 4.9.1:

- For FR1:

  - B when testing from 30 MHz to FDL_low - ΔfOBUE

  - T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

- For FR2:

  - B when testing from 30 MHz to FDL_low - ΔfOBUE

  - T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band operation, see subclause 4.9.1:

- For FR1:

  - BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

  - TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

- For FR2:

  - BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

  - TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in multi-band operation, see subclause 4.9.1:

- For FR1:

  - BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

  - B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

  - BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex xx) as long as the required TRP output power level is achieved.

<Unchanged are omitted>

--------------End of TP ------------