3GPP TSG-RAN WG4 Meeting #89                                                                          R4-1814532

# Spokane, US, 12th – 16th November, 2018

Agenda Item:	7.12.4.15

Source:	Intel Corporation

Title:	Discussion about SNR levels for CSI-RS RLM test

Document for:	Discussion

# Introduction

In this contribution, we discuss the SNR levels used for CSI-RS RLM OOS and INS test.

# Method for deriving signal levels

For legacy LTE, the method for deriving signal level is according to[1]:

For out-of-sync and in-sync tests, the signal level is changed at different time instants as per the required target SNR as shown in Fig. 1 and Fig. 2.

Figure 1 (Figure 7.3.1 from TS 36.133). SNR variation for out-of-sync testing

Figure 2 (Figure 7.3.2 from TS 36.133). SNR variation for in-sync testing

The following methodology is proposed for deriving the test SNR levels.

- SNR2 = Qout + margin1 dB

- SNR3 = Qout – margin1 dB

- SNR4 = Qin – margin2  dB

- SNR5 = Qin + margin2  dB

- And finally, SNR1 = SNR5.

- Qout and Qin correspond to the average of SNR points from simulation results of different companies for out-of-sync and in-sync PDCCH formats respectively.

For ETU 70 Hz, margin1 = 3 dB and margin2 = 2.5 dB.

# SNR level design for CSI-RS RLM with 2Rx

There is an averaged PDCCH simulation results in last meeting.

| Test configuration | 2RX, Required SNR (dB) Qout | 2RX, Required SNR (dB) Qin 2Rx |
| --- | --- | --- |
| SSB SCS15KHz, data SCS15KHz | [-10.1] | [-1.8] |
| SSB SCS30KHz, data SCS30KHz | [-9.6] | [-1.7] |

The configuration for CSI-RS test case FR1 is:

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

For CSI-RS RLM test case, for SCS=15K with 10M bandwidth and SCS=30K with 40M bandwidth, the RB number is 48 and 96 respectively. Density =3 is suggested since it can provide better SNR estimation accuracy.

Proposal 1: CSI-RS with Density = 3 is configured for RLM test case.

For the test case of 10M bandwidth with SCS=15K, our simulation results is similar with that of SSB. For SNR2, 3dB margin can be used. However, for SNR3, 3 dB margin is not enough. . Our simulation results for SINR estimation for SNR=-13dB is provided in Fig.3 where 1 dB margin is considered. It shows that in 75% cases, the estimated SNR is lower than SNRout= -10dB. It means that UE can declare OOS in only 75% cases.

Fig.3 estimated SNR at SNR=-13dB.

We still simulate the CDF of estimated SNR at SNR = -15dB in Fig4 and it shows in 90% cases, estimated SNR is below -10dB and OOS can be declared. Therefore, 5dB margin to derive SNR3 is suggested.

Fig.4 SNR estimation at -15dB

The margin to derive SNR4/SNR5 are 2.5dB.

For the test case of 40M bandwidth with SCS=30K, 0.5dB~1dB accuracy improvement can be observed as the RB number increases to 96. Margin to derive to SNR3 can be reduced to 4dB.

Observation 1: for CSI-RS of 10M bandwidth with SCS=15K, 5dB margin to derive SNR3 is needed. The margin to derive SNR2 is 3dB. The margin to derive SNR4/SNR5 are 2.5dB.

Observation 2: for CSI-RS of 40M bandwidth with SCS=30K, 4dB margin to derive SNR3 is needed. The margin to derive SNR2 is 3dB. The margin to derive SNR4/SNR5 are 2.5dB.

Proposal 2: for CSI-RS RLM FR1, different margin can be applied to different SCS.

Proposal 3: we provide the SNR margins to derive 5 SNR levels in the following table for FR1 with 2Rx:

Table 2 SNR margins to derive 5 SNR levels for FR1 with 2Rx

| Configuration index | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
| 1,2,4,5(10M bandwidth with SCS=15K) | 2.5dB | 3dB | 5dB | 2.5dB | 2.5dB |
| 3,6(40M bandwidth with SCS=30K) | 2.5dB | 3dB | 4dB | 2.5dB | 2.5dB |

# Conclusion

In this contribution, SNR level for SSB RLM test are discussed. The following conclusion can be drawn:

Proposal 1: CSI-RS with Density = 3 is configured for RLM test case.

Observation 1: for CSI-RS of 10M bandwidth with SCS=15K, 5dB margin to derive SNR3 is needed. The margin to derive SNR2 is 3dB. The margin to derive SNR4/SNR5 are 2.5dB.

Observation 2: for CSI-RS of 40M bandwidth with SCS=30K, 4dB margin to derive SNR3 is needed. The margin to derive SNR2 is 3dB. The margin to derive SNR4/SNR5 are 2.5dB.

Proposal 2: for CSI-RS RLM FR1, different margin can be applied to different SCS.

Proposal 3: we provide the SNR margins to derive 5 SNR levels in the following table for FR1 with 2Rx:

Table 2 SNR margins to derive 5 SNR levels for FR1 with 2Rx

| Configuration index | SNR1 | SNR2 | SNR3 | SNR4 | SNR5 |
| --- | --- | --- | --- | --- | --- |
| 1,2,4,5(10M bandwidth with SCS=15K) | 2.5dB | 3dB | 5dB | 2.5dB | 2.5dB |
| 3,6(40M bandwidth with SCS=30K) | 2.5dB | 3dB | 4dB | 2.5dB | 2.5dB |

# References

[1] R4-092110,” Summary of results and a proposal for signal levels for radio link monitoring tests”, Motorola.