3GPP TSG-RAN WG4 Meeting #89	R4-1814536

# Spokane, USA, November 12 – 16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on BWP switching requirement in TS38.133 (Section 8.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description on BWP switching delay requirement for UL does not including TA, which is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification is made in section 8.6 to clarify the requirement for UL BWP switching delay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BWP switching delay requirement for UL will be shortened due to the exisiting of TA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change >>

## 8.6	Active BWP switch delay

### 8.6.1	Introduction

The requirements in this section apply for a UE configured with more than one BWP on PCell or any activated SCell in standalone NR, or PSCell or any activated SCell in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this section.

### 8.6.2	UE active BWP switch delay

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at DL slot n+ TBWPswitchDelay.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where n is the beginning of a subframe (FR1) or half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at DL slot n+ TBWPswitchDelay.

When BWP switch occurs in UL, after UE receives BWP switching request at DL slot n (for DCI-based BWP switch), or the BWP-inactivity timer expires at DL slot n (for timer-based BWP switch), UE shall be able to transmit PUSCH for UL active BWP switch on the new BWP on the serving cell on which BWP switch occurs no later than at UL slot n+ TBWPswitchDelay+Y, where

- Y equals to zero if  (TA + TA offset) < X;

- Y equals to the smallest number of slots (depending on SCS) that greater than (TA + TA offset –X) if (TA + TA offset) ≥ X.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the cell where DCI-based BWP switch or timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1. X is defined in Table 8.6.2-2.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | [1] | [3] |
| 1 | 0.5 | [2] | [5] |
| 2 | 0.25 | [3] | [9] |
| 3 | 0.125 | [6] | [17] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Table 8.6.2-2: X values for UL BWP switching delay

|  | NR Slot length (ms) | For Type 1 Delay UE (µs) | For Type 2 Delay UE (µs) |
| --- | --- | --- | --- |
| 0 | 1 | 185.82 | 784.72 |
| 1 | 0.5 | 292.83 | 392.35 |
| 2 | 0.25 | 96.41 | 196.17 |
| 3 | 0.125 | 123.22 | 98.10 |

## << End of Change >>