3GPP TSG-RAN WG4 Meeting #89		         R4-1814559

Spokane, US, 12th – 16th Nov, 2018

Agenda item:      6.6.4.1

Source:                Intel Corporation

Title:	                     Requirements in fading channel conditions with 1024QAM

Document for:     Discussion

# Introduction

In RAN4#88bis the PDSCH demodulation requirements in fading channel were discussed and Simulation assumption for 1024QAM demodulation requirement was approved [1].

In this contribution we provide alignment and impairment results for 1024QAM test cases based on the agreed simulation assumptions to finalize requirements with 1024QAM.

# Simulation Results

## FDD

For the simulation assumptions and FRC tables agreed in [1], we provide impairment results for FDD test cases.

Table 1: Impairment results for FDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 24.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 27.1 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 30.3 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 28.5 |

## TDD

For the simulation assumptions and FRC tables agreed in [1], we provide impairment results for TDD test cases.

Table 2: Impairment results for TDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 26.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 29.2 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 28.9 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 27.2 |

# Conclusion

In this paper we provide impairment results for 1024QAM test cases in fading channel conditions for FDD and TDD test cases. Our results for FDD tests are summarized below:

Table 1: Impairment results for FDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 24.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 27.1 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 30.3 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 28.5 |

Table 2: Impairment results for TDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 26.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 29.2 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 28.9 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 27.2 |

Proposal#1: We recommend that RAN4 considers our results in defining UE demodulation requirements for PDSCH with 1024QAM in fading channel conditons

# Reference

- R4-1813705, “Simulation assumption for 1024QAM demodulation requirement”, Qualcomm