3GPP TSG-RAN WG4 Meeting #89		         R4-1814561

Spokane, US, 12th – 16th Nov, 2018

Agenda item:       6.6.4.4

Source:                Intel Corporation

Title:	         CQI Reporting Requirements with 1024QAM

Document for:     Discussion

# Introduction

In RAN4#88bis UE demodulation and CSI requirements for 1024QAM were discussed and way forward [1] was approved. For CQI reporting tests the following agreements were made.

- Agreed test: - Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1 - Bandwidth: - FDD: 10MHz - TDD: 20MHz - SNR point: Choose [28dB] and [29dB] for FDD and TDD respectively

In this contribution we present our views on CQI reporting tests with 1024QAM.

# Discussion

With the introduction of 1024QAM a new CQI table is introduced in TS 36.213. In order to verify CQI reporting with 1024QAM it was agreed to introduce 1 test with TM1 under static channel condition. The following have been agreed for CQI reporting test:

- Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1

  - Bandwidth:

    - FDD: 10MHz

    - TDD: 20MHz

- The CQI to MCS table for 1024QAM CQI reporting tests was defined:

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 2.4063 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 | 8.3321 | 9.2578 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.x1A | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| MCS.x2A | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- SNR point: Choose [28dB] and [29dB] for FDD and TDD respectively

The high SNR verifies CQI reporting for 1024QAM, but with the introduction of new CQI table the entries for other modulation have been modified. Hence we recommend having tests at a lower SNR targeting 64QAM.

Table 1: CQI Table for 1024QAM

| CQI index | modulation | code rate x 1024 x | Efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

CQI values 6-9 correspond to 64QAM. Based on the CQI reporting results shown in figure below, CQI corresponding to 64QAM are reported for SNR 10-16dB.

Figure 1: CQI Reporting with 1024QAM

Observation #1: With the new CQI table CQI corresponding to 64QAM is reported in SNR range of 10dB – 16dB.

We recommend choosing SNR 13,14dB as the lower SNR range for 1024QAM CQI reporting test.

Proposal #1: Choose SNR of 13dB, 14dB as lower SNR point for CQI reporting test with 1024QAM

Based on results in Figure 1, CQI values corresponding to 1024QAM are reported >= 27dB SNR. Very high SNR might be susceptible to TxEVM and hence not suitable for test case. Whereas with a low SNR we might have the risk of not reporting 1024QAM CQI. Based on these observations, we recommend choosing SNR of 29dB and 30dB as the higher SNR test points for 1024QAM CQI reporting test.

Proposal #2: Choose SNR of 29dB, 30dB as higher SNR point for CQI reporting test with 1024QAM

For the proposed SNR levels, we observe the following results:

Table 2: CQI Reporting Test results with 1024QAM

|  | SNR | Med CQI | MCS for Med CQI | BLER @ Med | BLER @ Med-1 | BLER @ Med+1 |
| --- | --- | --- | --- | --- | --- | --- |
| 64QAM | 13 | 8 | 13 | 0.00125 | 0 | 1 |
|  | 14 | 8 | 13 | 0 | 0 | 1 |
| 1024QAM | 29 | 14 | 23 | 0 | 0 | 1 |
|  | 30 | 14 | 23 | 0 | 0 | 0.83 |

# Conclusion

In this paper we present our views on CQI reporting tests with 1024QAM and have the following observations and proposals:

Observation #1: With the new CQI table CQI corresponding to 64QAM is reported in SNR range of 10dB – 16dB.

Proposal #1: Choose SNR of 13dB, 14dB as lower SNR point for CQI reporting test with 1024QAM

Proposal #2: Choose SNR of 29dB, 30dB as higher SNR point for CQI reporting test with 1024QAM

# Reference

- R4-1814231, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon