3GPP TSG-RAN WG4 Meeting #89		         R4-1814563

Spokane, US, 12th – 16th Nov, 2018

Agenda item:      7.11.8.1.2

Source:                Intel Corporation

Title:	                     On Remaining Issues for Candidate Beam Detection

Document for:     Discussion

# Introduction

In RAN4#88bis candidate beam detection was discussed and way forward on beam management was agreed [1]. With regards to candidate beam detection or link recovery the following were agreed:

- -3dB as side condition for CBD measurement. - For CSI-RS based CBD, CSI-RS is assumed to be transmitted in >=24 PRBs with D=3 - [3] samples for SSB and [5] samples for CSI-RS with D=3 with FFS accuracy, FFS how to capture the accuracy requirements in the spec - CBD measurement may cause scheduling restriction when performed. When CBD measurement is performed is FFS. - Option 1: CBD measurement is performed only when beam failure is detected - Option 2: Up to UE implementation - No other is no condition for N=1 for SSB based CBD measurement - N is fixed to 8 when condition for N=1 does not apply. - Same scheduling restriction due to CBD measurement are defined as for L1-RSRP for beam reporting - FFS prioritization between L3 measurement in SMTC and CBD measurement. - For SSB based CBD, FFS if N=1 is feasible under the condition that SSB is included in SMTC - For CSI-RS based CBD, FFS if N=1 is feasible and the condition - FFS if requirements for D=1 with larger BW are defined for CSI-RS based CBD - FFS when UE scheduling restriction are allowed due to CBD measurement being performed. - Option 1: when beam failure is detected - Option 2: Up to UE implementation

In this contribution we address the open issues related to candidate beam detection (CBD).

# Discussion

The open issues related to CBD are:

- Accuracy requirements for CBD

- Conditions for N=1 for CBD

- Resource sharing factor ‘P’ for CBD

- Scheduling restriction

- CSI-RS based CBD with D=1

## Accuracy requirements for Candidate Beam Detection

In RAN4#88bis the following were agreed for accuracy requirements for CBD:

- -3dB as side condition for CBD measurement.

- For CSI-RS based CBD, CSI-RS is assumed to be transmitted in >=24 PRBs with D=3

- [3] samples for SSB and [5] samples for CSI-RS with D=3 with FFS accuracy, FFS how to capture the accuracy requirements in the spec

To define the accuracy requirements for CBD, we measured the estimation error for CSI-RS based L1-RSRP measurement for FR1 and FR2

Table 1: L1-RSRP Measurement accuracy in FR1

|  | CSI-RS BW | 3 | 5 | 7 | 10 |
| --- | --- | --- | --- | --- | --- |
| TDLA30; 10Hz 15KHz SCS | 24 PRB | 1.6 | 1.45 | 1.37 | 1.3 |
| TDLC300; 100Hz 15KHz SCS |  | 1.78 | 1.65 | 1.57 | 1.5 |
| TDLC300; 100Hz 30KHz SCS |  | 1.8 | 1.68 | 1.63 | 1.53 |

Table 2: L1-RSRP Measurement accuracy in FR2

|  | CSI-RS BW | 3 | 5 | 7 | 10 |
| --- | --- | --- | --- | --- | --- |
| TDLA30; 75Hz 120KHz SCS | 24 PRB | 1.68 | 1.5 | 1.4 | 1.35 |
|  | 48 PRB | 1.3 | 1.24 | 1.1 | 1.03 |
|  | 96 PRB | 1.15 | 1 | 0.9 | 0.8 |

Based on the results in table above, we observe that there is not significant performance improvement with 5 samples vs 3 samples. In order to not introduce additional latency due to measurement, we recommend defining requirements for CSI-RS based CBD with 3 samples.

Proposal #1: Define requirements for CSI-RS based Candidate Beam Detection with 3 samples

With CSI-RS config with D=3 and 24PRB, we observe that the estimation error is ~2dB with 3 samples without considering implementation and RF margin. Taking into account implementation and RF margin, we recommend defining the accuracy requirement as 5 dB.

Proposal #2: Define accuracy requirement for CSI-RS based Candidate Beam Detection as 5dB

During the link recovery procedure the UE doesn’t report L1-RSRP of the candidate beams to the network, hence it is not straightforward to define accuracy requirements for CBD. But the accuracy of L1-RSRP estimation for CBD could be verified implicitly in the test for BFD and CBD.

Proposal #3: Define test case for Beam Failure Detection and Candidate Beam Detection so as to verify L1-RSRP estimation accuracy for Candidate Beam Detection

## Conditions for N=1

In RAN4#88bis, it was agreed that for SSB based CBD in FR2 the evaluation period (TEvaluate_CBD_out in Table 8.5.5.2-1) shall always have scale factor of N=8.

The conditions for N=1 for CSI-RS based CBD are FFS. For CSI-RS resources configured for CBD, suitable Rx beam would need to be determined, except for the cases when the Rx beam is known. The suitable Rx beam for CSI-RS resource for CBD would be known if it is configured with repetition parameter ‘ON’ (for beam refinement) or is configured with parameter trs-Info (for tracking). Another case would be if the CSI-RS configured for CBD is QCL-TypeD with CSI-RS configured with repetition parameter ‘ON’ or with trs-info.

The conditions for CSI-RS based CBD when N=1 is applicable are:

N=1,

if CSI-RS configured for CBD is configured with repetition parameter and repetition parameter is ‘ON’, or,

if CSI-RS configured for CBD is configured with parameter trs-Info, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with repetition parameter and repetition parameter is ‘ON’ and CBD resources are mutually TDMed, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with parameter trs-Info and CBD resources are mutually TDMed

Proposal #4: The N= conditions for CSI-RS based Candidate Beam Detection shall be defined as:

N=1,

if CSI-RS configured for CBD is configured with repetition parameter and repetition parameter is ‘ON’, or,

if CSI-RS configured for CBD is configured with parameter trs-Info, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with repetition parameter and repetition parameter is ‘ON’ and CBD resources are mutually TDMed, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with parameter trs-Info and CBD resources are mutually TDMed

## Resource sharing factor ‘P’ for CBD

The resource sharing for CBD-RS shall be the same as BFD and should be updated as follows for SSB based CBD and CSI-RS based CBD.

For SSB based CBD:

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when CBD-RS is not overlapped with measurement gap and CBD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when CBD-RS is not overlapped with measurement gap and CBD-RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when CBD-RS is partially overlapped with measurement gap and CBD -RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when CBD-RS is partially overlapped with measurement gap and CBD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when CBD-RS is partially overlapped with measurement gap (TSSB <MGRP) and CBD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when CBD-RS is partially overlapped with measurement gap and CBD-RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 3.

For CSI-RS based CBD:

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when CBD-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when CBD-RS is partially overlapped with measurement gap and CBD-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when CBD-RS is not overlapped with measurement gap and CBD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when CBD-RS is not overlapped with measurement gap and CBD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when CBD-RS is partially overlapped with measurement gap and CBD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when CBD-RS is partially overlapped with measurement gap and CBD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when CBD-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and CBD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when CBD-RS is partially overlapped with measurement gap and CBD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

Proposal # 5: Define resource sharing factor ‘P’ for Candidate Beam Detection similar to Beam Failure detection

## Scheduling Restriction

In RAN4#88bis it was agreed that the scheduling restriction for CBD shall be the same as beam reporting. Also there was a discussion on when UE performs CBD. Options were:

  - Option 1: CBD measurement is performed only when beam failure is detected

  - Option 2: Up to UE implementation

When the UE performs CBD is not testable, we propose that it be left to UE implementation.

Proposal # 6: When UE performance Candidate Beam Detection is up to UE implementation

## CSI-RS based CBD with D=1

In RAN4#88bis the requirements for CSI-RS based CBD with D=1 were FFS. Based on RAN-1 UE feature list, for SSB/CSI-RS for beam measurement feature, the supported values for CSI-RS density are:

On FR2, UE is mandated to signal either “3 only” or “both 1 and 3”

On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signaling.

Based on the above, we recommend that requirements with CSI-RS density D=1 are not introduced for CSI-RS based CBD as it is not a mandatory feature for Rel-15.

Proposal #7: For CSI-RS based Candidate Beam Detection only define requirements with D=3

# Conclusion

In this paper we present our views on open issues related to Candidate beam detection requirements and have the following proposals:

Proposal #1: Define requirements for CSI-RS based CBD with 3 samples

Proposal #2: Define accuracy requirement for CSI-RS based CBD as 5dB

Proposal #3: Define test case for Bean failure Detection and Candidate Beam Detection so as to verify L1-RSRP estimation accuracy for Candidate Beam Detection

Proposal #4: The N= conditions for CSI-RS based Candidate Beam Detection shall be defined as:

N=1,

if CSI-RS configured for CBD is configured with repetition parameter and repetition parameter is ‘ON’, or,

if CSI-RS configured for CBD is configured with parameter trs-Info, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with repetition parameter and repetition parameter is ‘ON’ and CBD resources are mutually TDMed, or,

if CSI-RS configured for CBD is QCL-Type D with a CSI-RS resource with parameter trs-Info and CBD resources are mutually TDMed

Proposal # 5: Define resource sharing factor ‘P’ for Candidate Beam Detection similar to Beam Failure detection

Proposal # 6: When UE performance Candidate Beam Detection is up to UE implementation

Proposal #7: For CSI-RS based Candidate Beam Detection only define requirements with D=3

# Reference

- R4-1813736, “Way forward on NR beam management”, Huawei, HiSilicon, Nokia