3GPP TSG-RAN WG4 Meeting #89		         R4-1814567

Spokane, US, 12th – 16th Nov, 2018

Agenda item:       7.13.1.3.2

Source:                Intel Corporation

Title:	     NR PBCH UE Performance Requirements

Document for:     Discussion

# Introduction

In RAN4#88bis PBCH demodulation requirements were discussed and the way forward was agreed [1]. The agreements for PBCH demodulation requirements are as follows:

- RAN4 introduces 2Rx PBCH demodulation requirements in Rel-15. - FFS whether to introduce 4Rx PBCH requirements in Rel-15 - If 4Rx PBCH requirements is introduced, - whether to apply 4Rx PBCH requirements is up to UE declaration - Note the PBCH demodulation requirements do not need to be tested in RAN5 as same as LTE PBCH demodulation requirements. (i.e. no conformance requirements) - Decision should be made in November meeting - Test metric: - SNR_PBCH@1% Pm-bch. Pm-bch is defined as 1-A/B, where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs. - UE combines the PBCH symbols of the same SSB index within the MIB TTI (80ms). - RAN4 further study in RAN4#89 : - the PBCH demodulation performance with/without the knowledge of SSB index (or PBCH DMRS sequence). - PBCH demodulation requirements consider SSB index acquisition performance or not.

In this contribution we present our views on PBCH demodulation requirements and simulation results to define performance requirements.

# Simulation Results

## Simulation Assumptions

The simulation assumptions for PBCH demodulation requirements are summarized in tables below.

Table 1: Common Parameters for PBCH Demodulation test cases

| Parameters | Unit | Value |
| --- | --- | --- |
| Cell ID |  | 0 |
| CP length |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index |  | 0 |
| SS burst periodicity | ms | 20 |
| MIB size | bits | 24 |
| PBCH payload size (including 24 bits CRC and 8 bits of SS/PBCH location information) | bits | 56 |
| Receiver assumption |  | LMMSE |

Table 2: Simulation Parameters for PBCH Demodulation test cases

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency Range | FR1 | FR1 | FR2 | FR2 |
| Channel Bandwidth | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern (Note 1) | Case A | Case B Case C | Case D | Case E |
| Antenna Configuration (Note 2) | 1x2 [1x4] | 1x2 [1x4] | 1x2 | 1x2 |
| Propagation channel (Note 2) | TDL-C RMS DS=300ns Doppler=100Hz | TDL-A RMS DS=30ns, Doppler=10Hz | TDL-A RMS DS=30ns, Doppler=300Hz | TDL-A RMS DS=30ns, Doppler=75Hz |
| Note 1: Depending on the operating band. Refers to TS38.101-1 Table 5.4.3.3-1 for which band uses which pattern Note 2: Use PDP specified in TS38.901 |  |  |  |  |

## FR1 Results

In this section we present alignment results for FR1 test cases with 2Rx and 4Rx. The tables capture the SNR (dB) @ 1% Pm-bch. The simulations are run with simplified TDL channel models agreed in [2] and with PBCH-DMRS sequence detection.

Table 3: Alignment results for PBCH in FR1 with 2Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -8.9 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -7.2 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -7.1 |

Table 4: Alignment results for PBCH in FR1 with 4Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -11.7 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -10.4 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -10.3 |

## FR2 Results

In this section we present alignment results for FR2 test cases. The table captures the SNR (dB) @ 1% Pm-bch. The simulations are run with simplified TDL channel models agreed in [2] and with PBCH-DMRS sequence detection.

Table 5: Alignment results for PBCH in FR2 with 2Rx

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -8.6 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -8.9 |

## Impairment Results

Taking into account implementation margin, we present the SNR (dB) @ 1% Pm-bch test cases in table below.

Table 6: Impairment results for NR PBCH tests

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -7.4 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -5.7 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -5.6 |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -10.2 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -8.9 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -8.8 |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -7.1 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -7.4 |

# PBCH Demodulation Requirements

In the Section 2 we present PBCH results with TI or PBCH-DMRS sequence detection. DMRS sequence detection is part of the PBCH decoding chain especially for initial access and cell identification scenarios. Assuming perfect TI knowledge in PBCH decoding doesn’t present a realistic scenario for testing. Rather than introducing separate tests for TI detection, it would be beneficial to include it in PBCH decoding tests.

Proposal #1: Include PBCH-DMRS sequence detection in PBCH demodulation requirements

# Conclusion

In this paper we present simulation results to define performance requirements for NR PBCH. Our simulation results are summarized below:

Table 7: Alignment results for NR PBCH tests

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -8.9 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -7.2 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -7.1 |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -11.7 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -10.4 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -10.3 |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -8.6 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -8.9 |

Table 6: Impairment results for NR PBCH tests

|  | Channel BW | SS/PBCH SCS | SS Block pattern | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10MHz | 15kHz | Case A | 1x2 | TDL-C-300; 100Hz | -7.4 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x2 | TDL-A-30; 10Hz | -5.7 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x2 | TDL-A-30; 10Hz | -5.6 |
| Test 1 | 10MHz | 15kHz | Case A | 1x4 | TDL-C-300; 100Hz | -10.2 |
| Test 2-1 | 40MHz | 30kHz | Case B | 1x4 | TDL-A-30; 10Hz | -8.9 |
| Test 2-2 | 40MHz | 30kHz | Case C | 1x4 | TDL-A-30; 10Hz | -8.8 |
| Test 3 | 100MHz | 120kHz | Case D | 1x2 | TDL-A-30; 300Hz | -7.1 |
| Test 4 | 100MHz | 240kHz | Case E | 1x2 | TDL-A-30; 75Hz | -7.4 |

Proposal #1: Include PBCH-DMRS sequence detection in PBCH demodulation requirements

# Reference

- R4-1814240, “Way forward on NR PBCH demodulation requirements”, CMCC, Huawei, HiSilicon, China Telecom, ZTE, CATT, NTT DOCOMO, Vodafone, Orange, Sprint, KDDI, SoftBank

- R1-1813735, “Way Forward on simplified TDL channel model”, Huawei, HiSilicon, Spirent, Rohde & Schwarz, Keysight, Ericsson, Intel