3GPP TSG-RAN WG4 Meeting #89	R4-1814582

# Spokane WA, USA. 12th – 16th Nov. 2018

Agenda item:	7.6.13.5.2

Source: 	Intel Corporation

Title: 	UL CA MPR in FR2

Document for:	Approval

# Introduction

In this contribution, we provide our MPR evaluation results for UL contiguous CA.

# Discussion

We consider MPR for contiguous UL CA with up to 8CCs with UL CA BW up to 800MHz.

The MPR values for contiguous PRB allocations (with necessary inter-CC gaps) are determined in the following first.

For PC3, in our MPR evaluations, we took agreed PC3 single CC CP-OFDM MPR for 400MHz BW as a starting point [2], and verified for CA case with BW <400Mhz. For >=400MHz and <=800MHz, up to 8CCs were evaluated. Our proposed MPR values are listed in the following table.

Maximum power reduction (MPRC_CA) for UE Power Class 3

|  |  | Cumulative aggregated channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <=800M |
| DFT-s-OFDM | Pi/2 BPSK | 5.0 | 7.5 |
|  | QPSK | 5.0 | 7.5 |
|  | 16 QAM | 6.5 | 9.0 |
|  | 64 QAM | 9.0 | 10.5 |
| CP-OFDM | QPSK | 5.0 | 7.5 |
|  | 16 QAM | 6.5 | 9.0 |
|  | 64 QAM | 9.0 | 10.5 |

For PC1, the proposed MPR values in [1] for contiguous PRB allocations (with necessary inter-CC gaps) were defined reasonable. But Need to consider newly agreed MPRnarrow for PC1 [2]. Our proposed MPR for PC1 contiguous UL CA with contiguous PRB allocations is shown below:

For PC1, MPR for contiguous PRB allocations (MPRC_CA) is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow=14.4dB, when BWalloc,RB is less than or equal to 1.44MHz, MPRnarrow=10dB, when 1.44MHz< BWalloc,RB≤10.8MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

Maximum power reduction (MPRWT_C_CA) for UE Power Class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <=800M |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 |
|  | QPSK | ≤ 6.5 | 8.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 |
|  | 64 QAM | ≤ 6.5 | 10.7 |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 |
|  | 64 QAM | ≤ 7.5 | 10.7 |

Next, for UL contiguous CA with non-contiguous PRB allocations, we evaluated MPR values vs. A, where A = the ratio of the number of populated RBs over total RB allowance with same SCS in UL CA aggregated BW = 800MHz.

The following plots show PC1 and PC3 required power backoff vs A.

Figure 1 MPR values vs A for non-contiguous PRB allocations in contiguous CCs

Based on our evaluations, we propose the same PC3 MPR rule for non-contiguous RB allocations in [1]

MPRNCRB_CA = max(MPRC_CA, -10*A +7.0)

For PC1, we propose the following MPR rule for non-contiguous RB allocations

MPRNCRB_CA = max(MPRC_CA, -10*A +14.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated UL contiguous CA aggregated bandwidth

We use MPRNCRB_CA instead of MPRNC_CA to represent MPR for non-contiguous PRB allocations in UL CA with contiguous CCs since nomenclature of MPRNC_CA can be reserved for UL CA with Non-Contiguous CCs.

# Conclusion

In this contribution, we propose MPR for UL CA with contiguous CCs (up to 8CCs) for PC1 and PC3 with CA up to BW 800MHz.

# References

- R4-1811487, “Update FR2 CA MPR”, Qualcomm. RAN4#88, Gothenburg, Sweden. August, 2018

- Draft CR for 38.101-2-f30 based on RAN4_88Bis_v2