3GPP TSG-RAN WG4 Meeting #89                                                               R4-1814637

# Spokane, US, 12th – 16th Nov, 2018

Source: 	CATT

Title: 	TP to TS38.141-1: Correction on power boosting for FR1 NR Test model

Agenda item:	7.9.2.3

Document for:	Approval

- Introduction

In RAN4 #88bis meeting, the power boosting for NR test models for FR1 [1] were agreed. But power boosting formulas for NR-FR1-TM1.2, NR-FR1-TM3.2 and NR-FR1-TM3.3 need to be changed.

This paper will discuss this issue and give our TP on power boosting for FR1 NR test model.

- Discussion

Take the parameters for NR-FR1-TM1.2 (As defined in Table 4.9.2.2.2-1 in TS38.141-1) as an example, the calculated results for 15 kHz SCS and 20MHz BW configuration is shown in the last column in Table 2-1.

Table 2-1: Physical channel parameters of NR- FR1-TM-1.2

| Parameter | SCS: x kHz, BW: y MHz | SCS: 15 kHz, BW: 20 MHz |
| --- | --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=40% | 40% |
| # of QPSK PDSCH RBGs which are boosted | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB | xNRBP=5, NRB=106, P=8 |
| Level of boosting (dB) | 3 | 3 |
| Location of PDSCH RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 | 0, 12, 2, 4, 6 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-PxNRBP | 66 |
| Level of deboosting (dB) | 10log10NRB-PxNRBPNRB-103/10PxNRBP | 4.014 |

The 4.014dB in table 2-1 calculated by the formula for level of deboosting in table 4.9.2.2.2-1 in TS38.141-1 is not correct, which should be -4.014dB. So, the formula of level of deboosting (dB) in table 4.9.2.2.2-1 in TS38.141-1 should be changed to

10log10NRB-10310PxNRBPNRB-PxNRBP=10log10106-10310*8*0.4*1068106-8*0.4*1068=-4.014 dB

The formulas for “the level of boosting” in Table 4.9.2.2.7-1 and Table 4.9.2.2.8-1 have similar problem.

- The formula of level of boosting (dB) in Table 4.9.2.2.7-1 should be changed to  10log10NRB-10-310PxNRBPNRB-PxNRBP .

- The formula of level of boosting (dB) in Table 4.9.2.2.8-1 should be changed to  10log10NRB-10-610PxNRBPNRB-PxNRBP .

- Conclusion

It is proposed to capture the modification to the parameters for NR-FR1-TM1.2, NR-FR1-TM3.2 and NR-FR1-TM3.3. A corresponding text proposal is given in the appendix for approval.

- References

- TS 38.141-1 V1.1.0 (2018-11),Base Station (BS) conformance testing Part 1: Conducted conformance testing (Release 15)

- Appendix

<< Start of Change >>

4.9.2.2.2	NR FR1 test model 1.2 (NR- FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Table 4.9.2.2.2-1: Physical channel parameters of NR- FR1-TM-1.2

| Parameter | SCS: x kHz, BW: y MHz |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | 3 |
| Location of PDSCH RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-PxNRBP |
| Level of deboosting (dB) | 10log10NRB-PxNRBPNRB-103/10PxNRBP10log10NRB-10310PxNRBPNRB-PxNRBP |

<<Unchanged texts omitted>>

4.9.2.2.7	NR FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter |  |
| --- | --- |
| Percent of 16QAM PDSCH PRBs boosted | x=60% |
| # of 16QAM PDSCH RBGs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -3 |
| Location of 16QAM RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-3/10PxNRBP10log10NRB-10-310PxNRBPNRB-PxNRBP |

4.9.2.2.8	NR FR1 test model 3.3 (NR- FR1-TM3.3)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter |  |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=50% |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -6 |
| Location of QPSK RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of 16QAM PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-6/10PxNRBP10log10NRB-10-610PxNRBPNRB-PxNRBP |

<< End of Change >>