3GPP TSG-RAN WG4 Meeting #89	R4-1814649

Spokane, USA, 12 - 16 Nov. 2018

Title: 	NR V2X RF operating parameters for RAN1 evaluation

Source: 	CATT

Agenda item:	11

Document for:	Discussion

- Introduction

In RAN4#88bis meeting, NR V2X RF operating parameters for RAN1 evaluation are collected as in the below table and way forward [1] was approved by E-mail.

This document will discuss the TBD parameters and give our views.

| Company | AGC set time | Tx/Rx switching time | Time error | Frequency error | In-band Emission | Others |
| --- | --- | --- | --- | --- | --- | --- |
| LG | FR1: 20us (w/o PSD changed), FFS for PSD changed FR2: FFS | FR1: up to 13us, FR2: up to 7us | FR1: 0.4 us regaldress of sync. Source, FR2: FFS | FR1: 0.1 ppm regardless of sync. Sources. FR2: FFS | FR1: reuse LTE V2X IBE. FR2: FFS | Focus on FR1 operating RF parameters |
| HW | FR1&2: FFS | FR1: 13 us, FR2 : 7us | dependent on sync. Source | FR1 & FR2: 0.1 ppm regardless of sync. Sources | Whether it can be improved need FFS |  |
| Ericsson | FR1: 20 us w/o PSD change, one OFDM symbol with PSD change for 15kHz SCS, FFS for other SCS. FR2: FFS | FR1: 13 us, FR2: 7 us (Refer to 38.211, table 4.3.2-3) | Up to ±390ns all sync source except syncREF (v2x NR) UE, FFS for syncREF (v2x NR) UE | FR1&FR2: within ±0.1 ppm observed over a period of 1 msec | FR1: reuse/adapt IBE req in 6.4.2.3 in 38.101-1 FR2: reuse/adapt IBE req in 6.4.2.3 in 38.101-2 | Prioritize FR1 over FR2 |
| Intel | FFS | FR1: 13 us, FR2 : 7us | gNB synchronization reference source: Option 1: reuse TS 38.133 section 7.1.2 Option 2: 12*64*Tc for FR1 and 3.5*64*Tc for FR2. GNSS synchronization reference source : 12*64*Tc for FR1 and FFS for FR2 | FR1&FR2: ±0.1 ppm for gNB synchronization reference source FFS for GNSS at FR2 | FR1: Use tightened NR UL IBE model, FR2: FFS |  |
| CATT | FFS | FR1: up to 13us | FFS | FR1: 0.1 ppm regardless of sync. Sources. | FR1: reuse LTE V2X IBE. | Focus on FR1 operating RF parameters |
| QC | FFS |  | FFS | FFS | Whether it can be improved need FFS |  |
| RAN4 baseline assumption | TBD on next meeting | Align with RAN1 | TBD on next meeting | TBD on next meeting | TBD on next meeting | Based on progress, RAN4 send LS on the RF parameters |

- Discussion

The following discussions will take NR UE RF parameters as starting point since we think NR V2X UE should be developed based on NR UE implementation. It is reasonable to assume that NR V2X RF performance have similar performance as NR UR RF.

Tx/Rx switching time

It was well discussed in RAN4 last year. The requirements were decided and captured in TS38.133. is the Tx/Rx switching time is 13μs for FR1 and 7μs for FR2. These values can be reused for NR V2X UE.

Time error

In RAN1 coming LS [2], it is defined as the timing difference between a UE and its synchronization reference. In TS38.133, the UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.1.2-1 of TS 38.133 and reproduced as in Table 2-1 below.

Table 2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [12]*64*Tc |
|  |  | 30 | [10]*64*Tc |
|  |  | 60 | [10]*64*Tc |
|  | 30 | 15 | [8]*64*Tc |
|  |  | 30 | [8]*64*Tc |
|  |  | 60 | [7]*64*Tc |
| 2 | 120 | 60 | [3.5]*64*Tc |
|  |  | 120 | [3.5]*64*Tc |
|  | 240 | 60 | [3]*64*Tc |
|  |  | 120 | [3]*64*Tc |

The timing error limits include measurement error for DL reference signal timing, and error for setting UL signal timing. The SCS of SSB signals (i.e. received reference signal bandwidth) is also a factor for timing error. Now we can’t know what synchronization reference signal is for NR V2X, it may be similar as SSB or GNSS. Currently it is proposed to reuse the NR timing error limits for RAN1 NR V2X performance evaluation. We think key factor is not SCS of SSB but the bandwidth of SSB. The timing error should be based on reference signal bandwidth, and is proposed as in table 2-2:

Table 2-2: Timing Error Limits

| Frequency Range | Bandwidth of DL reference signal (MHz) | Timing error (μs) |
| --- | --- | --- |
| 1 | 3.6 | 0.4 |
|  | 7.2 | 0.26 |
| 2 | 28.8 | 0.11 |
|  | 57.6 | 0.1 |

Frequency error

It is proposed to reuse NR UE frequency error limit, i.e. ±0.1 PPM for both FR1 and FR2.

In-band emission

In last meeting, we proposed to reuse LTE V2X IBE. Further considering the in-band emissions requirement for NR and LTE are similar. NR in-band emissions general requirement consider different SCS. IQ Image and Carrier leakage requirements for NR (i.e -28dB) are 3dB stricter than LTE when carrier center frequency > 1GHz and output power > 10dBm for FR1. Considering NR V2X UE may also have different SCS, we think NR UE in-band emission requirement could be considered.

For in-band emissions requirement, a near-far problem was raised in [3]. We agree the near-far problem is more severe for V2X UE than normal UE. But this is not a new issue for NR and has been exist in LTE. The IBE of LTE V2X adopted the same IBE model as for normal UE. Since now it is the SI phase, we think the IBE model in NR specification can be used as a starting point for RAN1 evaluation. If RAN1 evaluations shows problem or in the future the situation is more clear in WI phase, we could consider whether to define more severe IBE requirement in RAN4 specification.

Based the above consideration, the RF parameters for RAN1 NR V2X evaluation are summarized in Table 2-3.

Table 2-3: Proposed RF parameters for RAN1 NR V2X evaluation

| Company | AGC set time | Tx/Rx switching time | Time error | Frequency error | In-band Emission | Others |
| --- | --- | --- | --- | --- | --- | --- |
| CATT | TBD | 13μs for FR1; 7μs for FR2. | As proposed in Table 2-2. | ±0.1 PPM for FR1 and FR2. | Reuse NR UE in-band emission requirement. |  |

Proposal: It is proposed to use the parameters in Table 2-3 for RAN1 NR V2X evaluation.

- Conclusion

This contribution discussed the TBD parameters for NR V2X as requested by RAN1 LS.

Proposal: It is proposed to use the parameters in Table 2-3 for RAN1 NR V2X evaluation.

| Company | AGC set time | Tx/Rx switching time | Time error | Frequency error | In-band Emission | Others |
| --- | --- | --- | --- | --- | --- | --- |
| CATT | TBD | 13μs for FR1; 7μs for FR2. | As proposed in Table 2-2. | ±0.1 PPM for FR1 and FR2. | Reuse NR UE in-band emission requirement. |  |

- Reference

[1]	R4-1814322, WF on NR V2X RF operating parameters based on RAN1 incoming LS, LG Electronics, Ericsson, Intel, Huawei, CATT, Qualcomm

[2]	R4-1812009, LS on IBE model for V2X, 3GPP RAN WG1

[3]	R4-1813452, On NR V2X RF and RRM characteristics, Intel