3GPP TSG-RAN WG4#89 Meeting	R4-1814661

Spokane, U.S.A., 12 – 16 November 2018

Agenda item:	7.13.1.4.2

Title: 	Simulation results and discussion on NR CQI reporting under AWGN conditions

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD CQI reporting tests. Also, in RAN4#88 Meeting and RAN4#88bis Meeting, WFs [6, 7] were agreed and companies were encouraged to bring static CQI simulation results with median CQI +/-1 and their BLER within [0 ~ 20]dB with 2dB step size. In this contribution, we present our static CQI results for alignment purpose.

# Initial CQI Reporting Results and Discussion

Table 1 lists the common parameters for the CQI reporting test cases specified in this contribution.

Table 1 Test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 40 | 100 |
| SCS | kHz | 15 | 30 | 120 |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| TDD DL/UL pattern |  | N/A | 7D1S2U | DDSU |
| Propagation channel |  | AWGN | AWGN | AWGN |
| Correlation and antenna configuration |  | 2×2 / 2×4 with static channel specified in [36.101] | 2×2 / 2×4 with static channel specified in [36.101] | 2×2 with static channel specified in [36.101] |
| CSI-RS resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS periodicity/slot offset |  | 5/1 | 5/1 | 5/1 |
| CSI-RS number of ports (nrofPorts) |  | 2 ports {3000,3001} | 2 ports {3000,3001} | 2 ports {3000,3001} |
| CSI-RS cdm-Type |  | fd-CDM2 | fd-CDM2 | fd-CDM2 |
| CSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [13] | [13] | [13] |
| CSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [001000] | [001000] | [001000] |
| CSI-RS density |  | 1 | 1 | 1 |
| CSI-IM periodicity/slot offset |  | 5/1 | 5/1 | 5/1 |
| CSI-IM-RE-Pattern |  | Pattern0 | Pattern0 | Pattern1 |
| ZPCSI-RS number of ports (nrofPorts) |  | 4 ports {3000, …, 3003} | 4 ports {3000, …, 3003} | 4 ports {3000, …, 3003} |
| ZPCSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [9] | [9] | [13] |
| ZPCSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000100] | [000100] | [010000] |
| ZPCSI-RS density |  | 1 | 1 | 1 |
| ZP CSI-RS reportConfigType |  | Periodic | Periodic | Periodic |
| Reporting periodicity | ms | 5 | 5 | 5 |
| Reporting offset | ms | 4 | 4 | 4 |
| CQI delay | ms | 8 | 8 | 8 |
| Reporting granularity |  | Wideband | Wideband | Wideband |
| Physical channel for reporting |  | [PUCCH] | [PUCCH] | [PUCCH] |
| reportQuantity |  | [cri-RI-CQI] | [cri-RI-CQI] | [cri-RI-CQI] |
| Codebook type |  | Type I single-panel | Type I single-panel | Type I single-panel |
| Codebook subset restriction |  | 010000 | 010000 | 010000 |
| CQI table |  | Table 5.2.2.1-3 [38.214] | Table 5.2.2.1-3 [38.214] | Table 5.2.2.1-2 [38.214] |

Table 2 to Table 6 below list our results of Median CQI reporting vs. SNR for FR1 (FDD & TDD, 2x2 & 2x4) with 256QAM CQI table and FR2 (TDD, 2x2) with 64QAM CQI table.

Table 2. Median CQI vs. SNR (256QAM CQI table for FR1 FDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 |  | 0.015 | 0.995 |
| 2 | 3 | 1 | 0 | 0.495 |  |
| 4 | 3 | 1 |  | 0.005 | 0.985 |
| 6 | 4 | 1 |  | 0.085 | 0.98 |
| 8 | 5 | 1 |  | 0.045 | 0.995 |
| 10 | 6 | 1 |  | 0.055 | 0.98 |
| 12 | 7 | 1 |  | 0.045 | 0.965 |
| 14 | 8 | 1 |  | 0.02 | 0.805 |
| 16 | 9 | 1 |  | 0.02 | 0.885 |
| 18 | 10 | 1 |  | 0.01 | 0.83 |
| 20 | 11 | 1 |  | 0.01 | 0.78 |

Table 3. Median CQI vs. SNR (256QAM CQI table for FR1 FDD 2x4)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 0.7674 |
| 4 | 4 | 1 |  | 0 | 0.2512 |
| 6 | 5 | 1 |  | 0 | 0.2 |
| 8 | 7 | 1 |  | 0.0186 | 1 |
| 10 | 8 | 1 |  | 0.0977 | 1 |
| 12 | 9 | 1 |  | 0.014 | 1 |
| 14 | 10 | 1 |  | 0.0837 | 1 |
| 16 | 11 | 1 | 0 | 0.2 |  |
| 18 | 12 | 1 |  | 0.014 | 1 |
| 20 | 13 | 1 |  | 0.0326 | 1 |

Table 4. Median CQI vs. SNR (256QAM CQI table for FR1 TDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 1 |
| 4 | 3 | 1 |  | 0 | 1 |
| 6 | 4 | 1 |  | 0 | 1 |
| 8 | 5 | 1 |  | 0.045 | 0.995 |
| 10 | 6 | 1 |  | 0 | 0.4145 |
| 12 | 7 | 1 |  | 0 | 0.3224 |
| 14 | 8 | 1 |  | 0 | 0.2895 |
| 16 | 9 | 1 |  | 0 | 0.3750 |
| 18 | 10 | 1 |  | 0 | 0.3750 |
| 20 | 11 | 1 |  | 0 | 0.2829 |

Table 5. Median CQI vs. SNR (256QAM CQI table for FR1 TDD 2x4)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 0.7632 |
| 4 | 4 | 1 |  | 0 | 0.4079 |
| 6 | 6 | 1 | 0 | 0.3487 |  |
| 8 | 7 | 1 |  | 0.0329 | 1 |
| 10 | 8 | 1 |  | 0.0724 | 1 |
| 12 | 9 | 1 |  | 0.0263 | 1 |
| 14 | 10 | 1 | 0 | 0.2829 |  |
| 16 | 11 | 1 | 0 | 0.2829 |  |
| 18 | 12 | 1 |  | 0.0066 | 1 |
| 20 | 13 | 1 |  | 0.1974 | 1 |

Table 6. Median CQI vs. SNR (64QAM CQI table for FR2 TDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0.0583 | 0.6 |
| 2 | 4 | 1 |  | 0.0333 | 0.7583 |
| 4 | 5 | 1 |  | 0.0417 | 0.4083 |
| 6 | 6 | 1 |  | 0.0083 | 0.5083 |
| 8 | 7 | 1 |  | 0.0083 | 0.425 |
| 10 | 8 | 1 |  | 0 | 0.525 |
| 12 | 9 | 1 |  | 0 | 0.5167 |
| 14 | 10 | 1 |  | 0 | 0.6583 |
| 16 | 11 | 1 |  | 0.0083 | 0.5083 |
| 18 | 12 | 1 |  | 0.025 | 0.8917 |
| 20 | 13 | 1 |  | 0.05 | 1 |

Moreover, in order to further align the simulation results, we make the following proposal on NR CQI reporting performance tests.

Proposal 1: Apply simulation assumptions in Table 1 for NR static CQI tests.

# Conclusion

In this contribution, we present our static CQI results for alignment purpose. We make the following proposal.

Proposal 1: Apply simulation assumptions in Table 1 for NR static CQI tests.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, Aug. 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[7] R4- 1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018