3GPP TSG-RAN WG4#89 Meeting	R4-1814662

Spokane, U.S.A., 12 – 16 November 2018

Agenda item:	7.13.1.4.2

Title: 	Simulation results and discussion on NR CQI reporting under fading conditions

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD CQI reporting tests. Also, in RAN4#88 Meeting and RAN4#88bis Meeting, WFs [6, 7] were agreed and companies were encouraged to bring CQI simulation results (wideband and sub-band CQI) under fading channel conditions. In this contribution, we present our initial WB and SB CQI results under fading channel conditions for alignment purpose.

# Wideband CQI reporting results in fading channels

In this section, we provide our initial simulation results for FR1 FDD wideband CQI reporting in TDL-A fading channel conditions. The simulation assumptions can be found in [7]. Moreover, it was also agreed in [6] to reuse LTE test metric for WB CQI tests under fading channel conditions:

- a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time;

- the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 2: Initial FR1 FDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 57.5% | 1.327 | 0.0849 |
| 7 | 52% | 1.432 | 0.1122 |
| 12 | 49% | 1.283 | 0.158 |
| 13 | 47.5% | 1.331 | 0.176 |

Table 3: Initial FR1 TDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 54% | 1.393 | 0.1143 |
| 12 | 50% | 1.301 | 0.1943 |

Table 4: Initial FR1 FDD 4Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 3 | 10% | 1.25 | 0.292 |
| 9 | 49.5% | 1.204 | 0.347 |

Table 5: Initial FR1 TDD 4Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 3 | 61% | 1.179 | 0.2871 |
| 9 | 57% | 1.025 | 0.2233 |

Table 6: Initial FR2 TDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 7% | 1.02 | 0.24 |
| 12 | 4% | 1.097 | 0.2233 |

# Sub-band CQI reporting results in fading channels

In this section, we provide our initial simulation results for FR1 FDD sub-band CQI reporting in fading channel conditions. The simulation assumptions can be found in [7]. Moreover, it was also agreed in [6] to reuse LTE test metric for SB CQI tests under fading channel conditions:

- sub-band differential CQI offset level of 0 shall be reported at least α % of the time but less than β % for each sub-band;

- the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ γ;

- when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 9: Initial FR1 FDD (Option 1: 2x2) SB CQI results

| SNR | Probability of 0 CQI offset | Throughput ratio (maxSB/medWB) | BLER for follow max SB CQI |
| --- | --- | --- | --- |
| 10 | 56.67% | 1.092 | 0.0728 |
| 11 | 58.75% | 1.077 | 0.089 |

Table 10: Initial FR1 FDD (Option 2: 1x2) SB CQI results

| SNR | Probability of 0 CQI offset | Throughput ratio (maxSB/medWB) | BLER for follow max SB CQI |
| --- | --- | --- | --- |
| 11 | 18.32% | 1.6367 | 0.3608 |
| 12 | 18.32% | 1.4909 | 0.3588 |

Based on simulation results in Table 9 and Table 10, it is observed that “Option 2: 1x2” antenna configuration achieves higher throughput ratio and more suitable for testing. Therefore,

Proposal 1: Apply “Option 2: 1x2” antenna configuration for SB CQI test cases.

# Conclusion

In this contribution, we present our initial WB and SB CQI results under fading channel conditions for alignment purpose.

Proposal 1: Apply “Option 2: 1x2” antenna configuration for SB CQI test cases.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, August 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[7] R4- 1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018