3GPP TSG-RAN WG4#89 Meeting	R4-1814663

Spokane, U.S.A., 12 – 16 November 2018

Agenda item:	7.13.1.4.3

Title: 	Simulation results and discussion on NR PMI reporting

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD wideband PMI reporting tests. Also, in RAN4#88 Meeting, Also, in RAN4#88 Meeting and RAN4#88bis Meeting, WFs [6, 7] were agreed and companies were encouraged to bring WB PMI simulation results for 4Tx and 8Tx cases. In this contribution, we present our PMI results for alignment purpose.

# PMI reporting results in fading channels

The PMI reporting simulation parameters can be found in the WF [7]. It was also agreed in [6] to reuse LTE test metric for PMI tests:

  - Relative Throughput ratio between following PMI and random PMI at SNR point corresponding to [70% ] TP with follow PMI.

Table 1 to 10 show our initial WB PMI results with simulation assumptions specified in [7] for alignment purpose.

- FR1, FDD, 4x2, MCS13, rank1, XP high, TDL-A

- FR1, FDD, 8x2, MCS13, rank2, XP high, TDL-A

- FR1, TDD, 4x2, MCS13, rank1, XP high, TDL-A

- FR1, TDD, 8x2, MCS13, rank2, XP high, TDL-A

- FR1, FDD, 4x4, MCS13, rank1, XP high, TDL-A

- FR1, FDD, 8x4, MCS13, rank2, XP high, TDL-A

- FR1, TDD, 4x4, MCS13, rank1, XP high, TDL-A

- FR1, TDD, 8x4, MCS13, rank2, XP high, TDL-A

- FR2, TDD, 2x2, TDL-A

- MCS13 & Low antenna correlation

- Option 1: MCS4 & Low antenna correlation

- Option 2: MCS13 & ULA Medium antenna correlation

It is observed that, for FR2 2x2 and low antenna correlation, lower MCS level (e.g. MCS4) achieves greater relative throughput ratio between following PMI and random PMI. Alternatively, FR2 2x2 and medium antenna correlation with MCS13 can also achieve greater relative throughput ratio. In addition, the latter one works at the higher SNR range, so it’s more suitable for testing. Therefore,

Proposal: Apply 2x2 and medium antenna correlation with MCS13 for FR2 PMI test.

# Conclusion

In this contribution, we present our PMI results for alignment purpose. It is observed that, for FR2 2x2 and low antenna correlation, lower MCS level (e.g. MCS4) achieves greater relative throughput ratio between following PMI and random PMI. Alternatively, FR2 2x2 and medium antenna correlation with MCS13 can also achieve greater relative throughput ratio. In addition, the latter one works at the higher SNR range, so it’s more suitable for testing. Therefore,

Proposal: Apply 2x2 and medium antenna correlation with MCS13 for FR2 PMI test.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, August 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[7] R4- 1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018