3GPP TSG-RAN WG4 RAN4-89	R4-1814683

# Spokane, USA, 12 - 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.114 | CR | 070 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.114: Exclusion Bands for Radiated Immunity Test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rx exclusion bands in TS 37.114 are not correctly defined for OTA AAS BS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Differentiate between AAS BS and OTA AAS BS when calculating the Rx Exclusion Bands, depending on the spatial exclusion consideration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the value is not defined it won’t be possible to calculate the exclusion bands for Radiated Immunity test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of Text ------------------------------

### 4.4.2	Receiver exclusion band

An exclusion band is a band of frequencies over which no tests of radiated immunity are made.

For AAS BS, the range of the exclusion band shall be:

FUL_low  – ΔfOOB < f < FUL_high + ΔfOOB

Where:

Values of FUL_low and FUL_high are defined for each operating band in TS 37.104 [33].

The values of ΔfOOB are defined in table 4.4.2-1.

For OTA AAS BS when spatial exclusion zone is considered, the range of the exclusion band shall be:

FUL_low  – ΔfOOB < f < FUL_high + ΔfOOB

For OTA AAS BS, when spatial exclusion zone is not considered, the range of the exclusion band shall be:

FUL_low  – 200 MHz  < f < FUL_high + 200 MHz

Where:

Values of FUL_low and FUL_high are defined for each operating band in TS 37.104 [33].

The values of ΔfOOB are defined in table 4.4.2-1.

Table 4.4.2-1: Maximum ΔfOOB offset outside the uplink operating band for AAS BS

| Operating band characteristics | ΔfOOB [MHz] |
| --- | --- |
| 100 MHz ≥ FUL_high – FUL_low | 20 |
| 100 MHz < FUL_high – FUL_low ≤ 900 MHz | 60 |

For BS capable of multi-band operation, the total receiver exclusion band shall be the combination of the exclusion bands for each operating band supported by AAS BS.

NOTE 1:	The receiver exclusion bands do not apply for SDL bands.

NOTE 2: 	Exclusion bands for BS type 2-O in radiated immunity test do not have to be defined considering the frequency range from 80MHz to 6GHz.

------------------------------ End of proposed text ------------------------------