3GPP TSG-RAN WG4 Meeting #89	R4-1814711

# Spokane, WA, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38-101-3 | CR | <CR#> | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configured maximum output power for inter-band EN-DC within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Pcmax for inter-band EN-DC within FR1 is not specified. The draft CR in R4-1813844 (baseline): The total transmitted power PTotalEN-DCconfigured by the UE can be confused with the measured power Pumax. Correct the specification of the requirements for the measured total transmitted power (Pumax) and the power reduction as proposed in the draft CR in R4-1813844. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the UE can also configure its the total transmitted power PTotalEN-DC. Requirements on the measured total power Pumax and NR power reduction (in accordance the Xscale indicated) introduced. The “IF-conditions” in the draft CR R4-1813844 cannot be used for an accurate specification of the Pcmax and the NR power reduction and are therefore removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of power control for inter-band EN-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The draft CR in R4-1813844 is used as baseline, additional changes proposed in this CR are indicated by a different user name for tracking. |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

< equations for Pcmax >

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its configured maximum transmission power for EN-DC operation PTotalEN-DC.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in 36.101 sub-clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, EN-DC , MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-      MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC , PLTE, PPowerClass – ΔPPowerClass}

The configured maximum output power PCMAX_ NR,c (q) in slot q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in sub-clause 6.2.4 of 38.101-1 modified by PNR as follows:

PCMAX_L,f,c,,NR = MIN { PEMAX, EN-DC , MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC , PNR , PPowerClass – ΔPPowerClass }

- PLTE and PNR are the linear values for the PLTE and PNR respectively signaled by RRC defined in [7]

- ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in 36.101 applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

- TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise TC_NR,c = 0dB;

- ΔTIB,c specified in sub-clause 6.2.7 for EN-DC, the individual Power Class defined in table 6.2B.1-3 and any other additional power reductions parameters specified in sub-clauses 6.2.3 and 6.2.4 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX_ NR,c evaluations.

If the transmissions from NR and E-UTRA do not overlap, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

The configured transmission power for EN-DC operation is, for both synchronous and non-synchronous operation,

P_EN-DC_Total = MIN { PEMAX, EN-DC ,PPowerClass, EN-DC}

P_EN-DC_Total is the dB value of  PTotalEN-DC, which is used in [38.213], PEMAX, EN-DC is  p-maxUE-FR1-r15 value signaled by RRC and defined in [36.331] and PPowerClass, EN-DC is defined in sub-clause 6.2B.1.3-1 for inter-band EN-DC.

If the EN-DC UE does not support dynamic power sharing, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications 36.101 and 38.101-1 respectively apply with the modifications specified above and P_EN-DC_Total applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX_ NR,c for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation PTotalEN-DCas specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a slot q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframes and slots durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL slot overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, whee q+n is the last NR UL slot overlapping with LTE subframe p,

If a UE indicates support of dynamicPowerSharing in the UE-MRDC-Capability IE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p)], PTotalEN-DC}

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PTotalEN-DC}

whereas if the a UE is configured with+≤ PTotalEN-DC, for any indication  of dynamicPowerSharing (supported or not supported),

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)]}

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)]}

where

- pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

- pCMAX H _ NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

- pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

- pCMAX L _ NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

- is the linear value of ,  is the linear value of ,

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity LTE-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |

If a UE is configured with <  PTotalEN-DC and the UE supports  dynamic power sharing, the measured maximum output power PUMAX in slot q on CG 2 shall be within

PEN-DC_L,c(2) (p,q) – T(PEN-DC_L,c(2) (p,q)) ≤  10 log10 pUMAX,c(2)  ≤  PEN-DC_H,c(2) (p,q) + T(PEN-DC_H,c(2) (p,q))

when transmission(s) in slot q on CG 2 overlap in time with transmission(s) in slot p on CG 1, where

PEN-DC_L,c(2) (p,q) = MIN{PCMAX_L,f,c,,NR (q), 10 log10 (PTotalEN-DC – pCMAX L _ E-UTRA,c (p))}

PEN-DC_H,c(2) (p,q) = MIN{PCMAX_H,f,c,,NR (q), 10 log10 (PTotalEN-DC – pCMAX L _ E-UTRA,c (p))}

whenever the power reduction {PCMAX,f,c,NR (q) – 10 log10 (PTotalEN-DC – pCMAX L _ E-UTRA,c (p))} rounded up to the next integer value is less than or equal to X_scale; the tolerance T(P) for applicable power levels P is specified in Table 6.2.4-1 of [38.101-1] and

#### X_scale is configured by RRC and can take values [0, 6]

#### PCMAX,f,c,NR (q) is the configured max power for NR as reported in the PHR [38.321]

The evaluation of the measured power pUMAX,c(2)  over slot q is in accordance with subclause 6.2.4 of [38.101-1]; if the slot q on CG 2 overlaps with more than [one symbol] of subframe p – 1 or p + 1 on CG 1, the PCMAX L  E-UTRA,c is the minimum of that evaluated over subframe p and subframe p – 1 or p + 1.

### The measured maximum output power in slot p on CG 1, pUMAX,c(1),  shall meet the requirements in subclause 6.2.5 in [36.101] with the limits PCMAX_L,c and PCMAX_H,c replaced by PCMAX_L_ E-UTRA,c and PCMAX_H_ E- UTRA,c as specified above, respectively.

##### 6.2B.4.1.4	Inter-band EN-DC including FR2

< end of changes >