3GPP TSG-RAN WG4 Meeting #89	R4-1814713

# Spokane, WA, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38-101-3 | CR | <CR#> | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of configured maximum output power for intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add outstanding requirements for dynamic power sharing and correct the A-MPR specification applicable for the configured power levels for each CG (also depends on the support of dynamic power sharing). The NR power reduction in accordance with the Xscale value received from high layers is not specified for UEs supporting dynamic power sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements apply only for “synchronous” EN-DC The specification of the allowed A-MPR for the configured power levels on the MCG and SCG (depends on support of dynamic power sharing) corrected Specification of the total transmitted power PTotalEN-DC(depends on support of dynamic power sharing) added. Requirements on the measured total power Pumax and NR power reduction (in accordance the Xscale indicated) introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification of power control for contiguous intra-band EN-DC is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2B.4	Configured output power for EN-DC

#### 6.2B.4.1	Configured output power level

<Editor’s note: The title of 6.2B.4.1 to be updated by later RAN4 decision>

##### 6.2B.4.1.1	Intra-band contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC.

For intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for EN-DC operation PTotalEN-DC.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in 36.101 sub-clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN {MIN(PEMAX,c , PEMAX, EN-DC , PLTE) – TC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC , PLTE, PPowerClass – ΔPPowerClass}

for a UE indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with the DCI of serving cell c of the CG 1 and the specification in sub-clause 6.2.4 of [36.101];

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with sub-clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

The configured maximum output power PCMAX_ NR,c (q) in slot q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in sub-clause 6.2.4 of 38.101-1 modified by PNR as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC , PNR ) - TC_NR, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC , PNR , PPowerClass – ΔPPowerClass }

- PLTE and PNR are the linear values for the PLTE and PNR respectively signaled by RRC defined in [7]

- ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in 36.101 applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

- TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise TC_NR,c = 0dB;

- ΔTIB,c specified in sub-clause 6.2.7 for EN-DC, the individual Power Class defined in table 6.2B.1-3 and any other additional power reductions parameters specified in sub-clauses 6.2.3 and 6.2.4 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX_ NR,c evaluations.

- for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR with A-MPR determined in accordance with sub-clause 6.2B.3.1 and MPRc = 0 dB;

- for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with sub-clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

If the EN-DC UE is not supporting dynamic power sharing, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications 36.101 and 38.101-1 respectively apply with the modifications specified above; the UE can  configure its configured maximum transmission power for EN-DC operation PTotalEN-DC. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

For UEs not indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE the total configured transmission power is

PEN-DC,tot = 10log10(PTotalEN-DC) = MIN{PPowerClass,EN-DC, PEMAX,EN-DC} + 0.3	[dBm]

where and operation PTotalEN-DC the linear value of a configured maximum transmission power for EN-DC operation as specified in sub-clause 7.6 of [38.213], Ppowerclass,EN-DC the EN-DC power class of the band combination configured and PEMAX,EN-DC the limit of the UE power for EN-DC indicated by the IE p-MaxUE-FR1-r15 [36.331].

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE the UE can configure the total transmission power within the range

PEN-DC,tot_L <  PEN-DC,tot <  PEN-DC,tot_H

where

PEN-DC,tot_L (p,q) = MIN{ PPowerClass,EN-DC – A-MPR, PEMAX,EN-DC}

PEN-DC,tot_H (p,q) = MIN{PPowerClass,EN-DC, PEMAX,EN-DC}

for sub-frame p on CG 1 overlapping with slot q on CG 2 and A-MPR in accordance with sub-clause 6.2B.3.1.

The measured total maximum output power PUMAX over both CGs is

PUMAX = 10 log10 [pUMAX,c(1) + pUMAX,c(2)],

where pUMAX,c(i) denotes the measured output power of serving cell c(j) of CG j expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.1-2.

When an UL subframe transmission p from E-UTRA overlap with a slot q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.1-1 when same or different subframes and slots durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.1-1: PCMAX evaluation window

| Transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL slot overlapping with LTE subframe p.

If a UE indicates support of dynamicPowerSharing

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p)], PEN-DC,tot_H}

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEN-DC,tot_H}

whereas if the a UE is configured with 10 log10 (+) ≤ PEN-DC,tot_L if dynamicPowerSharing is supported or with +≤ PTotalEN-DC if not supported,

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)]}

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)]}

where

- pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

- pCMAX H _ NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

- pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

- pCMAX L _ NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

- is the linear value of ,  is the linear value of ,

Table 6.2B.4.1.1-1: Configured maximum output power tolerance for EN-DC

| P (dBm) | Tolerance Tlow(P) (dB) | Tolerance Thigh(P) (dB) |
| --- | --- | --- |
| 23 < P ≤ 33 | [3.0] | [2.0] |
| 22 ≤ P ≤ 23 | [5.0] | [2.0] |
| 21 ≤ P < 22 | [5.0] | [3.0] |
| 20 ≤ P < 21 | [6.0] | [4.0] |
| 16 ≤ P < 20 | [5.0] | [5.0] |
| 11 ≤ P < 16 | [6.0] | [6.0] |
| -40 ≤ P < 11 | [7.0] | [7.0] |
| NOTE 1: For UEs not indicating support of dynamicPowerSharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

If the UE does not indicate support of dynamicPowerSharing or if transmission(s) in subframe p on CG 1 does not overlap in time with transmission(s) in slot q on CG 2 or if the UE is configured with single UL transmission, then the measured maximum output power pUMAX,c(j)  for cell c(j) of CG j shall be in accordance with [36.101] for CG 1 with limits determined by PCMAX_L_ E-UTRA,c (p) and PCMAX_H_ E-UTRA,c (p) for subframe p and in accordance with [38.101-1] for CG 2 with limits determined by PCMAX_L,f,c,,NR (q) and PCMAX_H,f,c,,NR (q) for slot q.

If a UE is configured with<  PEN-DC,tot_H and the UE supports  dynamic power sharing, the measured maximum output power PUMAX in slot q on CG 2 shall be within

PEN-DC_L,c(2) (p,q) – T(PEN-DC_L,c(2) (p,q)) ≤  10 log10 pUMAX,c(2)  ≤  PEN-DC_H,c(2) (p,q) + T(PEN-DC_H,c(2) (p,q))

when transmission(s) in slot q on CG 2 overlap in time with transmission(s) in slot p on CG 1, where

PEN-DC_L,c(2) (p,q) = MIN{PCMAX_L,f,c,,NR (q), 10 log10 (pEN-DC,tot_H – pCMAX L _ E-UTRA,c (p))}

PEN-DC_H,c(2) (p,q) = MIN{PCMAX_H,f,c,,NR (q), 10 log10 (pEN-DC,tot_H – pCMAX L _ E-UTRA,c (p))}

whenever the power reduction{PCMAX,f,c,NR (q) – 10 log10 (pEN-DC,tot_H – pCMAX L _ E-UTRA,c (p))} rounded up to the next integer value is less than or equal to {X_scale – (PEN-DC,tot_H – PEN-DC,tot_L)}; the tolerance T(P) for applicable power levels P is specified in Table 6.2.4-1 of [38.101-1] and

#### X_scale is configured by RRC and can take values [0, 6]

#### -	   PCMAX,f,c,NR (q) is the configured max power for NR as reported in the PHR [38.321]

The evaluation of the measured power pUMAX,c(2)  over slot q is in accordance with subclause 6.2.4 of [38.101-1].

### The measured maximum output power in slot p on CG 1, pUMAX,c(1),  shall meet the requirements in subclause 6.2.5 in [36.101] with the limits PCMAX_L,c and PCMAX_H,c replaced by PCMAX_L_ E-UTRA,c and PCMAX_H_ E- UTRA,c as specified above, respectively.

##### 6.2B.4.1.2	Intra-band non-contiguous EN-DC

< equations for Pcmax >

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

< end of changes >