3GPP TSG-RAN WG4 Meeting #89	R4-1814729

# Spokane, USA, November 12 – 16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MRTD requirement for intra-band synchronous EN-DC (section 7.6.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MRTD requirement for intra-band synchronous EN-DC does not distingish contiguous and non-contiguous cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification is made in section 7.6.3 to clarify the MRTD requirement for both contiguous and non-contiguous intra-band synchronous EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The statement in TS38.133 section 7.6.3 implies that contiguous and non-contiguous intra-band synchronous EN-DC share the same MRTD requirement, which is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change #1 >>

### Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous EN-DC [16]. The requirements for asynchronous EN-DC are applicable for E-UTRA FDD- NR FDD and E-UTRA TDD- NR TDD intra-band EN-DC.

For intra-band non-contiguous synchronous EN-DC, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.3-1 provided the UE indicates that it is only capable of synchronous EN-DC [16]. The requirements for intra-band non-contiguous synchronous EN-DC are applicable for E-UTRA TDD- NR TDD and E-UTRA FDD- NR FDD intra-band EN-DC.

No requirement on maximum receive timing difference is applicable for intra-band contiguous synchronous EN-DC.

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band non-contiguous synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

Table 7.6.3-2	Void

## << End of Change #1 >>