3GPP TSG RAN WG4 Meeting #89					R4-1814787

Spokane, Washington, U.S., Nov. 12th – 16th, 2018

Title:	Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects

Response to:	R1-1810006

Release:	Release 16

Work Item:	FS_NR_V2X

Source:	3GPP RAN WG4

To:	3GPP RAN WG1

Cc:

Contact Person:

Name:	Suhwan Lim

E-mail Address:	suhwan.lim@lge.com

Attachments:	None.

1. Overall Description:

RAN WG4 discussed the NR V2X RF parameters for NR V2X physical layer design aspect. According to the RAN4 discussion, RAN4 provide following answer for the next questions from RAN1.

The current RAN4 reply only for RAN1 physical design evaluation. This initial RAN4 consensus is not for RAN4 requirements which will be decided in NR V2X WI stage.

- Question 1: RAN1 would like to ask RAN4 to confirm the validity of the evaluation assumptions or provide guidance on appropriate values for:

- AGC settling time.

- TX/RX switching time

- Timing error between a UE and its synchronization reference.

- Frequency error between a UE and its synchronization reference.

- Answer 1: The below RF parameters in Table 1 are acceptable for NR V2X evaluation in RAN1

Table 1: Baseline NR V2X UE RF parameters

| RF parameters | AGC set time | Tx/Rx switching time | Time error | Frequency error |
| --- | --- | --- | --- | --- |
| RAN4 baseline assumption | FR1: 20 us for total power change in a slot for all SCSs. FR2: FFS | FR1: up to 13us, FR2: up to 7us | FR1: Up to ±0.4us all sync source except syncREF NR V2X UE, FFS for syncREF NR V2X UE FR2: FFS | FR1: ±0.1 ppm regardless of sync. Sources. FR2: FFS |

- Question 2: RAN1 would like to know whether IBE model defined in Section A.2.1.5 in TR 36.843 is still applicable to NR sidelink transmission in FR1, including OFDMA and SC-OFDMA. If the model is not applicable, RAN1 would like to request guidance from RAN4 on an appropriate model for system-level simulations.

- Answer 2: In the first stage to design NR V2X physical layer, RAN WG4 recommend to consider the following tighten levels from IBE requirements in Section 6.4.2.3 in TS 38.101-1 at FR1 for both waveforms. Specially, RAN4 recommend the follow tightening level X1, X2,X3, Y1, Y2, Z1, Z2, Z3, and Z4 as below

  - Initial recommend values {X1, X2, X3, Y1, Y2, Z1, Z2, Z3 and Z4} = {6, 3, 6, 3, 3, 3, 3, 3, 3}

- Question 3: RAN1 would like to request guidance from RAN4 on an IBE model for sidelink communication in FR2

- Answer 3: In the first stage to design NR V2X physical layer, RAN WG4 recommend to consider the IBE requirements in Section 6.4.2.3.4 for power class 3 UE in TS 38.101-2 at FR2 for both waveforms.

2. Actions:

To RAN WG1:

RAN WG4 respectfully asks RAN WG1 to evaluate the NR V2X sidelink operation taking into account the above recommendation.

3. Date of Next TSG-RAN Meetings:

3GPP RAN WG4 #90	   25th Feb. – 1st March 2019				Athens, Greece

3GPP RAN WG4 #90BIS    8th – 12th April  2019				China