3GPP TSG-RAN WG4 Meeting #89	R4-1814818

Spokane, USA, 12 - 16 November, 2018

Agenda Item:	7.13.2.3.2

Source:	ZTE Corporation

Title:	TP for TS38.141-2: PUCCH format 1 OTA conformance test

Document for:	Approval

# Introduction

This contribution provides a text proposal on PUCCH format 1 for Chapter 8 for TS38.141-2 [1] according to the approved specification drafting plan [2].

# Reference

[1] 3GPP TS38.141-2, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing, v1.0.0, September 2018.

[2] R4-1811729, Specification drafting plan for NR BS demodulation performance requirements, China Telecom, August 2018.

# Text Proposal

## ----- Start of TP -----

#### 8.3.1.2	Performance requirements for PUCCH format 1

Apply the requirements defined in TS38.141-2 [TBD] subclause 8.3.2 for 2Rx.

********************Start of next changes*****************

#### 8.3.2.2	Performance requirements for PUCCH format 1

##### 8.3.2.2.1	NACK to ACK detection

###### 8.3.2.2.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is [TBD].

###### 8.3.2.2.1.2	Minimum Requirement

The minimum requirement is in TS 38.104 [TBD] subclause 11.3.2.2.

###### 8.3.2.2.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.2.2.1.4	Method of test

8.3.2.2.1.4.1	Initial Conditions

Test environment:	Normal; see annex [B.2].

RF channels to be tested:	for single carrier (SC): M; see sub-clause [4.9.1].

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2].

8.3.2.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the combinations of SCS and channel bandwidth defined in Table 8.3.2.2.1.4.2-1.

Table 8.3.2.2.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 60 kHz | 100 |  |
| 120 kHz | 100 |  |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [TBD], and the specific test parameters are configured as below:

Table 8.3.2.2.1.4.2-2: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in [TBD].

4)	Adjusting the equipment so that the SNR specified in Table 8.3.2.2.1.5-1 and Table 8.3.2.2.1.5-2 is achieved at the BS input during the transmissions.

5)	The signal generator sends test signals, in regular time periods. The following statistics are kept: the number of NACK bits detected as ACK.

###### 8.3.2.2.1.5	Test Requirement

The fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Tables 8.3.2.2.1.5-1 and Table 8.3.2.2.1.5-2.

Table 8.3.2.2.1.5-1 Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth / SNR [dB] |
| --- | --- | --- | --- | --- |
|  |  |  |  | 100 MHz |
| 1 | 2 | Normal | TDLA-30-300 Low |  |

Table 8.3.2.2.1.5-2 Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth / SNR [dB] |
| --- | --- | --- | --- | --- |
|  |  |  |  | 100 MHz |
| 1 | 2 | Normal | TDLA-30-300 Low |  |

##### 8.3.2.2.2	ACK missed detection

###### 8.3.2.2.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

For a BS supporting multiple combinations of SCS and channel bandwidth, the applicable rule is defined in [TBD].

###### 8.3.2.2.2.2	Minimum Requirement

The minimum requirement is in TS 38.104 [TBD] subclause 11.3.2.2.

###### 8.3.2.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

###### 8.3.2.2.2.4	Method of test

8.3.2.2.2.4.1	Initial Conditions

Test environment:	Normal; see annex [B.2].

RF channels to be tested:	for single carrier (SC): M; see sub-clause [4.9.1].

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex [TBD].

8.3.2.2.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the combinations of SCS and channel bandwidth defined in Table 8.3.2.2.2.4.2-1.

Table 8.3.2.2.2.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 60 kHz | 100 |  |
| 120 kHz | 100 |  |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [TBD], and the specific test parameters are configured as below:

Table 8.3.2.2.2.4.2-2: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in [TBD].

4)	Adjusting the equipment so that the SNR specified in Table 8.3.2.2.2.5-1 and Table 8.3.2.2.2.5-2 is achieved at the BS input during the transmissions.

5) The signal generator sends a test pattern with the pattern outlined in figure 8.3.2.2.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.2.2.2.4.2-1: Test signal pattern for PUCCH format 1 demodulation tests

###### 8.3.2.2.2.5	Test Requirement

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in Tables 8.3.2.2.2.5-1 and Table 8.3.2.2.2.5-2.

Table 8.3.2.2.2.5-1 Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth / SNR [dB] |
| --- | --- | --- | --- | --- |
|  |  |  |  | 100 MHz |
| 1 | 2 | Normal | TDLA-30-300 Low |  |

Table 8.3.2.2.2.5-2 Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth / SNR [dB] |
| --- | --- | --- | --- | --- |
|  |  |  |  | 100 MHz |
| 1 | 2 | Normal | TDLA-30-300 Low |  |

----- End of TP -----