3GPP TSG-RAN WG4 meeting #89	R4-1814821
Spokane, USA, 12 Nov – 16 Nov, 2018

Source:	ZTE Corporation

Title:	Simulation results on NR PRACH

Agenda item:	7.13.2.4.1

Document for:	Approval

## 1. Background

In RAN4#88bis, the simulation setup was agreed for NR PRACH demodulation [1] [2] for both FR1 and FR2. In this contribution we provide our corresponding simulation results.

## 2. Discussion

Table – 1 PRACH simulation results for FR1

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | ZTE |
| --- | --- | --- | --- | --- |
| Format 0 | 1.25kHz | AWGN | 0 | -16.66 |
|  |  | TDLC300-100 | 400 | -14.8 |
| Format A1 | 15kHz | AWGN | 0 | -11.65 |
|  |  | TDLC300-100 | 400 | -10.98 |
|  | 30kHz | AWGN | 0 | -11.66 |
|  |  | TDLC300-100 | 400 | -10.92 |
| Format A2 | 15kHz | AWGN | 0 | -14.6 |
|  |  | TDLC300-100 | 400 | -13.9 |
|  | 30kHz | AWGN | 0 | -14.72 |
|  |  | TDLC300-100 | 400 | -14.03 |
| Format A3 | 15kHz | AWGN | 0 | -16.39 |
|  |  | TDLC300-100 | 400 | -15.54 |
|  | 30kHz | AWGN | 0 | -16.51 |
|  |  | TDLC300-100 | 400 | -15.75 |
| Format B4 | 15kHz | AWGN | 0 | -19.46 |
|  |  | TDLC300-100 | 400 | -17.46 |
|  | 30kHz | AWGN | 0 | -19.46 |
|  |  | TDLC300-100 | 400 | -18.49 |
| Format C0 | 15kHz | AWGN | 0 | -8.64 |
|  |  | TDLC300-100 | 400 | -8.03 |
|  | 30kHz | AWGN | 0 | -8.59 |
|  |  | TDLC300-100 | 400 | -8.04 |
| Format C2 | 15kHz | AWGN | 0 | -14.69 |
|  |  | TDLC300-100 | 400 | -13.91 |
|  | 30kHz | AWGN | 0 | -14.74 |
|  |  | TDLC300-100 | 400 | -13.95 |

Table – 2 PRACH simulation results for FR2

| PRACH FR2 1T2R |  | Propagation condition | Frequency offset (Hz) | ZTE |
| --- | --- | --- | --- | --- |
| Format A1 | 60kHz | AWGN | 0 | -12.3 |
|  |  | TDLA30-300 | 4000 | -5.5 |
|  | 120kHz | AWGN | 0 | -11.2 |
|  |  | TDLA30-300 | 4000 | -5 |
| Format A2 | 60kHz | AWGN | 0 | -15.3 |
|  |  | TDLA30-300 | 4000 | -8.4 |
|  | 120kHz | AWGN | 0 | -14.2 |
|  |  | TDLA30-300 | 4000 | -7.9 |
| Format A3 | 60kHz | AWGN | 0 | -17.1 |
|  |  | TDLA30-300 | 4000 | -10.2 |
|  | 120kHz | AWGN | 0 | -16.1 |
|  |  | TDLA30-300 | 4000 | -9.8 |
| Format B4 | 60kHz | AWGN | 0 | -20.1 |
|  |  | TDLA30-300 | 4000 | -12.9 |
|  | 120kHz | AWGN | 0 | -19.1 |
|  |  | TDLA30-300 | 4000 | -12.6 |
| Format C0 | 60kHz | AWGN | 0 | -9.2 |
|  |  | TDLA30-300 | 4000 | -2.4 |
|  | 120kHz | AWGN | 0 | -8.2 |
|  |  | TDLA30-300 | 4000 | -2 |
| Format C2 | 60kHz | AWGN | 0 | -15.3 |
|  |  | TDLA30-300 | 4000 | -8.5 |
|  | 120kHz | AWGN | 0 | -3 |
|  |  | TDLA30-300 | 4000 | -8 |

## 3. Conclusions

In this contribution, simulation results under the agreed setup are provided.

# References

[1] R4-1813755	Way forward on NR BS demodulation general part, Huawei

[2] R4-1813757	Way forward on NR PRACH demodulation requirements, Ericsson