3GPP TSG-RAN WG4 Meeting #89															R4-1814851

Spokane, USA, 12-16 Nov, 2018

Source: 	Qualcomm Incorporated

Title: 	n41 – B40 Coexistence

Agenda item:	7.6.9.5

Document for:	Approval

# Introduction

In this document, we discuss n41 – B40 Coexistence.

# Discussion

## B41 ISM and Band 40 Coexistence issue

NR band n41 (2496MHz to 2690MHz) lies near ISM band (2402-2478MHz) as well as E-UTRA band 40 (2300-2400MHz). Various filters exist to attenuate emissions in these bands. The proximity of all these bands introduces various filter design challenges. Some vendors have been successful in attenuating the WIFI band except for the region > 2473MHz, which essentially is the skirt of the n41 passband. However, improving the WIFI rejection comes at a cost in terms of insertion loss and a well-known “fly-back” effect that compromises rejection of emissions in Band 40 as shown in Figure 2.1-1. Some vendors do not even specify the B40 emission region, because it is assumed that the coexistence requirement of -50dBm/MHz can be met with the allowed MPR. Table 2.1-1 shows the vendor filter data.

Table 2.1-1: Comparison of available vendor filter data

| Vendor | Insertion Loss, dB | WIFI rejection, dB | Band 40 rejection, dB |
| --- | --- | --- | --- |
| A | 3.9 | 26.1 | 11.1 |
| B | 3.5 | 31.5 | 9.5 |
| C | 3.7 | 34.3 | 1.3 |
| Note: Rejection is attenuation relative to passband. |  |  |  |

Figure 2.1-1: Filter difficulty for both B41 with WIFI and B40 coexistence

## Emission due to large TX BW

The “fly-back” effect is not so much of a factor with narrow band signals such as mobile TX with LTE BW <=40MHz such as BW class A, B and C. But, as TXBW grows, as in n41, where TX BW is, 60MHz, 80MHz, and 100MHz, 5th order emissions become prominent, and the allowed MPR is simply not sufficient for the B40 filter rejection even with recent filter technologies designed for mobile to operate with WIFI coexistence in mind.

Such emissions do not exist in B40 region because the TX channel BW’s considered in LTE BW class C is not large enough to generate IM5 distortion in this region. Only 7th order emissions exist for this BW class, and filtering is not an issue because IM7 products are lower in level. As we increase the BW, then under certain RB configurations, the lack of rejection due to multiple coexistence requirement becomes an issue, where 5th order emissions come into play and the filter rejection in Band 40 is not enough. Table 2.2-1 shows a typical 5th order distortion budget for contiguous and non-contiguous RB allocations assuming 5th order products reach Band 40. As shown in the table, over 13dB of rejection is required to meet 1RB emissions. The culprit is essentially the composite distortion of CIM3 intermodulation with the TX as well as the direct CIM5.

Table 2.2-1: TX Budget for 5th order distortion

| Parameter | RB allocations |  |  |
| --- | --- | --- | --- |
|  | 1RB Contiguous (NR BW > 50MHz) | 1RB+1RB+1RB Non-Contiguous (LTE BW Class D) | 1RB+1RB Non-Contiguous (EN-DC) |
| TX Power, dBm | 24 | 24 | 24 |
| DFT-s-OFDM QPSK MPR, dB | 0.5 | 8.5 | TBD |
| Composite CIM5, dBc | -60.5 | -60.5 | -60.5 |
| IM5, dBc | N/A | -45 | -45 |
| Total IM5D, dBc | -60.5 | -44.4 | -44.4 |
| Filter Rejection, dB | 13 | 21.1 | TBD |

## Proposed Solution using RB restriction

So, either we must introduce A-MPR or use RB restrictions. We choose to use RB restrictions because of the uncertainty of how much A-MPR to use since some variability exists among vendor data to the amount of rejection to apply for B40.

The simplest method is to restrict the same amount of RB’s until the carrier frequency threshold is reached, as shown in Figure 2.3-1. Such a method unnecessarily restricts RB usage as the carrier frequency approaches the frequency threshold where no IM5 will occur. A compromise, would be to lower the RB restriction in a linearly fashion as the B41 carrier frequency increases further away from B40 edge, also shown in Figure 2.3-1.

Figure 2.3-1: Reducing RB start as a function of Offset frequency to maximize RB usage for suspect carrier frequencies causing 5th order distortion. (BW1 and BW2 are example TX BW’s where BW2 > BW1).

For NR bandwidths > 50MHz (60M, 80M, 90M, 100M), it can be shown that for no IM5 distortion landing in E-UTRA band 40, we need the following relation to hold:

RBstart>1.1*BW-Offset2-54*30KHzSCS,

where the Offset is defined from the lower band edge of NR band 41 to the lower channel edge as shown in Figure 2.3-1. Just to note, the above relation yields an integer and is a curve fitted approximation derived from the following theoretical formula.

RBstart>FB40_high-FC+5*BW2-GB5*12*SCS+12

So, from the top figure, any RB start below the green line has IM5 distortion in Band 40, and any RB start above this line does not have any IM5 distortion in Band 40.

A table of RB start Vs bandwidth and SCS for Offset = 0MHz is shown in table 2.3-1. How RB start reduces as a function of frequency offset V’s BW and SCS is shown in Figure 2.3-2. Note that as SCS increases, the slope of the boundary line reduces.

Proposal: Applicable for NR bandwidths > 50MHz, this requirement applies for RB start > [1.1 * (BW - Offset/2) - 54] * 30KHz/SCS, where the Offset is defined from 2496MHz to the lower edge of the assigned UL channel bandwidth frequency.

Table 2.3-1: RB start Vs BW and SCS for Offset Frequency = 0MHz

| RB start as a function of BW and SCS |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS, KHz |  |  |
|  |  | 15 | 30 | 60 |
| BW, MHz | 100 | N/A | 56 | 28 |
|  | 90 | N/A | 46 | 22 |
|  | 80 | N/A | 34 | 17 |
|  | 60 | 22 | 13 | 6 |
|  | 55 | 10 | N/A | N/A |
|  | 50 | 1 | 0 | 0 |

Figure 2.3-2: Restricted RB start boundary Vs Bandwidth, SCS, and Offset Frequency

# Conclusion

Proposal: Applicable for NR bandwidths > 50MHz, this requirement applies for RB start > [1.1 * (BW - Offset/2) - 54] * 30KHz/SCS, where the Offset is defined from 2496MHz to the lower edge of the assigned UL channel bandwidth frequency.

# Reference:

- 3GPP TS 38.101-1 TS Ver. 15.3 (2018-09)

# Appendix: