# 3GPP TSG-RAN4 Meeting #89	                                                                         R4-1814865

# Spokane, US, 12th Nov – 16th Nov, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RMC for RLM (section A.3.8) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To add RMC configurations for RLM test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update PRACH configuration table; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test will not be correctly applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.x.2.3 	FR1 PRACH configuration 3

FR1 PRACH configuration 3 in this section provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR1.

Table A.3.x.2.3-1 Parameters for FR1 PRACH configuration 3

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Reference Channel |  | [PRACH 2.3-1] | [PRACH 2.3-2] |  | [PRACH 2.3-3] |  |  |
| prach-ConfigurationIndex |  | 87 | [0] |  | [0] |  | PRACH 2.3-2 is for paired spectrum/supplementary uplink. PRACH 2.3-3 is for unpaired spectrum. |
| msg1-SubcarrierSpacing |  | Same as UL carrier SCS | Same as UL carrier SCS |  | Same as UL carrier SCS |  |  |
| totalNumberOfRA-Preambles |  | 48 | 48 |  | 48 |  | Total number of preambles used for contention based and contention free random acces |
| numberOfRA-PreamblesGroupA |  | 48 | 48 |  | 48 |  | No group B. |
| prach-RootSequenceIndex |  | 0 | 0 |  | 0 |  | Logic equence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion |  | oneFourth | oneFourth |  | oneFourth |  | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM |  | One | One |  | One |  | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep |  | dB2 | dB2 |  | dB2 |  |  |
| preambleReceivedTargetPower |  | dBm-120 | dBm-120 |  | dBm-120 |  |  |
| preambleTransMax |  | n6 | n200 |  | n200 |  | Max number of RA preamble transmission perfomed before declaring |
| ra-ResponseWindow |  | sl10 | sl10 |  | sl10 |  | 10 slots |
| zeroCorrelationZoneConfig |  | 11 | 11 |  | 11 |  | N-CS configuration, NCS = 23 |
| Backoff Parameter Index |  | 2 | 2 |  | 2 |  | 20ms, as defined in table 7.2-1 in TS 38.321. |
| csirs-ResourceList |  | ra-PreambleIndex = 50 | ra-PreambleIndex = 50 |  | ra-PreambleIndex = 50 |  | Assocated with CSI-RS configured |
| ra-OccasionList |  | 1 | 1 |  | 1 |  | RA occasions allowed corresponding to CSI-RS |
| rsrp-ThresholdCSI-RS |  | RSRP_51 | RSRP_51 |  | RSRP_51 |  | -106dBm≤ rsrp-ThresholdSSB <-105dBm |
| Note: For further information see Clause 6.3.2 in TS 38.331. |  |  |  |  |

<End of Change 1>

<Start of Change 2>

A.3.x.3.3 	FR2 PRACH configuration 3

FR2 PRACH configuration 3 in this section provides the typical PRACH configuration for CSI-RS based non-contention based random access in FR2.

Table A.3.x.3.3-1 Parameters for FR2 PRACH configuration 3

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| Reference Channel |  | [PRACH 3.3-1] | [PRACH 3.3-2] |  |  |  |
| prach-ConfigurationIndex |  | 236 | [144] |  |  |  |
| msg1-SubcarrierSpacing |  | Same as UL carrier SCS | Same as UL carrier SCS |  |  |  |
| totalNumberOfRA-Preambles |  | 48 | 48 |  | Total number of preambles used for contention based and contention free random acces |  |
| numberOfRA-PreamblesGroupA |  | 48 | 48 |  | No group B. |  |
| prach-RootSequenceIndex |  | 0 | 0 |  | Logic equence index = 0, resulting in root sequence = 1. |  |
| ssb-perRACH-Occasion |  | oneFourth | oneFourth |  | OneFourth: 1 SSB associated with 4 RACH occasions |  |
| msg1-FDM |  | One | One |  | One PRACH transmission occasions FDMed in one time instance. |  |
| powerRampingStep |  | dB2 | dB2 |  |  |  |
| preambleReceivedTargetPower |  | dBm-120 | dBm-120 |  |  |  |
| preambleTransMax |  | n6 | n200 |  | Max number of RA preamble transmission perfomed before declaring a failure |  |
| ra-ResponseWindow |  | sl10 | Sl40 |  |  |  |
| zeroCorrelationZoneConfig |  | 11 | 11 |  | N-CS configuration, NCS = 23 |  |
| Backoff Parameter Index |  | 2 | 2 |  | 20ms, as defined in table 7.2-1 in TS 38.321. |  |
| csirs-ResourceList |  | ra-PreambleIndex = 50 | ra-PreambleIndex = 50 |  | Assocated with CSI-RS configured |  |
| ra-OccasionList |  | 1 | 1 |  | RA occasions allowed corresponding to CSI-RS |  |
| rsrp-ThresholdCSI-RS |  | RSRP_51 | RSRP_51 |  | -106dBm≤ rsrp-ThresholdSSB <-105dBm |  |
| Note: For further information see Clause 6.3.2 in TS 38.331. |  |  |  |

<End of Change 2>