# 3GPP TSG-RAN4 Meeting #89	                                                                         R4-1814869

# Spokane, US, 12th Nov – 16th Nov, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for EN-DC FR2 SSB based RLM (section A.5.5.1.1, A.5.5.1.2, A.5.5.1.3, A.5.5.1.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some test cases which are not agreed |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove some test cases based on RAN4-88b agreement 2. Editorial change 3. Updated based on the new RMC sections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for EN-DC FR2 SSB based RLM are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.1.1, A.5.5.1.2, A.5.5.1.3, A.5.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.5.5.1.1			Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode

A.5.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.5.5.1.1.1-1. The test parameters are given in Tables A.5.5.1.1.1-2, A.5.5.1.1.1-3, and A. 5.5.1.1.1-4 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The E-UTRAN PCell setting refers to Table A.3.7.2.1-2. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.5.5.1.1.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.1.1-2: General test parameters for FR2 out-of-sync testing in non-DRX mode

| Parameter |  |  |  |  | Unit |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Test 1 |  | Test 2 |  |
| Active E-UTRA PCell |  |  |  |  |  |  | Cell 1 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  |  |  |  | 1 |  | 1 |  |
| Active PSCell |  |  |  |  |  |  | Cell 2 |  | Cell 2 |  |
| RF Channel Number |  |  |  |  |  |  | 2 |  | 2 |  |
| Duplex mode |  |  |  |  |  |  | TDD |  | TDD |  |
| TDD Configuration |  | Config 1 |  | [ TDDConf.3.1] | [ TDDConf.3.1] |
|  |  | Config 2 |  | [ TDDConf.3.1] | [ TDDConf.3.1] |
| CORESET Reference Channel |  | Config 1 |  | [CR. 3.1 TDD] | [CR. 3.1 TDD] |
|  |  | Config 2 |  | [CR. 3.1 TDD] | [CR. 3.1 TDD] |
| SSB Configuration |  | Config 1 |  | [Table A.3.2.2.2.1-1] | [Table A.3.2.2.2.1-1] |
|  |  | Config 2 |  | [Table A.3.2.2.2.1-1] | [Table A.3.2.2.2.1-1] |
| SMTC Configuration |  | Config 1 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
|  |  | Config 2 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz | 120 KHz |
|  |  | Config 2 |  | 120 KHz | 120 KHz |
| SSB index assigned as RLM RS |  | Config 1 |  | TBD | TBD |
|  |  | Config 2 |  | TBD | TBD |
| PRACH Configuration |  |  | Config 1 |  |  |  | TBD |  | TBD |  |
|  |  | Config 2 |  | TBD | TBD |
| OCNG parameters |  |  |  |  |  |  | Table A.3.2.1.1-1 |  | Table A.3.2.1.1-1 |  |
| CP length |  |  |  |  |  |  | Normal |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  | [2x2 Low] |  | [2x2 Low] |  |
| Out of sync transmission parameters |  | DCI format |  |  |  |  | 1-0 |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  |  | 2 |  | 2 |  |
|  |  | Aggregation level |  |  | CCE |  | 8 |  | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB |  | 4 |  | 4 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB |  | 4 |  | 4 |  |
|  |  | DMRS precoder granularity |  |  |  |  | REG bundle size |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  |  | 6 |  | 6 |  |
| DRX |  |  |  |  |  |  | OFF |  | OFF |  |
| Gap pattern ID |  |  |  |  |  |  | [N.A.] |  | *[gp0] |  |
| Layer 3 filtering |  |  |  |  |  |  | Enabled |  | Enabled |  |
| T310 timer |  |  |  |  | ms |  | 0 |  | 0 |  |
| T311 timer |  |  |  |  | ms |  | 1000 |  | 1000 |  |
| N310 |  |  |  |  |  |  | 1 |  | 1 |  |
| N311 |  |  |  |  |  |  | 1 |  | 1 |  |
| NZP CSI-RS configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| ZP CSI-RS configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| CSI-IM configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| Periodic CSI reporting |  |  |  |  |  |  | PUCCH |  | PUCCH |  |
| CSI reporting periodicity |  |  | Config 1 |  | slot |  | [40] |  | [40] |  |
|  |  |  | Config 2 |  |  |  | [40] |  | [40] |  |
| NZP CSI-RS configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| ZP CSI-RS configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| CSI-IM configuration |  |  |  |  |  |  | TBD |  | TBD |  |
| T1 |  |  |  |  | s |  | TBD |  | TBD |
| T2 |  |  |  |  | s |  | TBD |  | TBD |
| T3 |  |  |  |  | s |  | TBD |  | TBD |
| D1 |  |  |  |  | s |  | TBD |  | TBD |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. ote 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.1.1.1-3: OTA related cell specific test parameters for FR2 (Cell 2) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| UE orientation around TBD axis and TBD axis |  | degrees | TBD |  |  |  | TBD |  |  |
| PDCCH_beta |  | dB | 4 |  |  |  | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  |  | 0 |  |  |
| PSS_beta |  | dB |  |  |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |  |  |
| SNR | Config 1 | dB | TBD | TBD | TBD |  | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD |  | TBD | TBD | TBD |
|  |  | dBm/15 KHz | TBD |  |  |  | TBD |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.5.5.1.1.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Figure A.5.5.1.1.1-1: SNR variation for out-of-sync testing

A.5.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.2			Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode

A.5.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.5.5.1.2.1-1. The test parameters are given in Tables A.5.5.1.2.1-2, A.5.5.1.2.1-3, and A.5.5.1.2.1-4 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The E-UTRAN PCell setting refers to Table A.3.7.2.1-2. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.5.5.1.2.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.2.1-2: General test parameters for FR2 in-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 |
| Active E-UTRA PCell |  |  |  | Ce1l 1 | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 | 1 |
| Active PSCell |  |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  |  | 2 | 2 |
| Duplex mode |  |  |  | TDD | TDD |
| TDD Configuration |  | Config 1 |  | [ TDDConf.3.1] | [ TDDConf.3.1] |
|  |  | Config 2 |  | [ TDDConf.3.1] | [ TDDConf.3.1] |
| CORESET Reference Channel |  | Config 1 |  | [CR. 3.1 TDD] | [CR. 3.1 TDD] |
|  |  | Config 2 |  | [CR. 3.1 TDD] | [CR. 3.1 TDD] |
| SSB Configuration |  | Config 1 |  | [Table A.3.2.2.2.1-1] | [Table A.3.2.2.2.1-1] |
|  |  | Config 2 |  | [Table A.3.2.2.2.1-1] | [Table A.3.2.2.2.1-1] |
| SMTC Configuration |  | Config 1 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
|  |  | Config 2 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
| SSB index assigned as RLM RS |  | Config 1 |  | TBD | TBD |
|  |  | Config 2 |  | TBD | TBD |
| PRACH Configuration |  | Config 1 |  | TBD | TBD |
|  |  | Config 2 |  | TBD | TBD |
| OCNG parameters |  |  |  | Table A.3.2.1.1-1 | Table A.3.2.1.1-1 |
| CP length |  |  |  | Normal | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters |  | DCI format |  | 1-0 | 1-0 |
|  |  | Number of Control OFDM symbols |  | 2 | 2 |
|  |  | Aggregation level | CCE | 4 | 4 |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  |  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  |  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters |  | DCI format |  | 1-0 | 1-0 |
|  |  | Number of Control OFDM symbols |  | 2 | 2 |
|  |  | Aggregation level | CCE | 8 | 8 |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  |  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  |  | REG bundle size |  | 6 | 6 |
| DRX |  |  |  | OFF | OFF |
| Gap pattern ID |  |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  |  | Enabled | Enabled |
| T310 timer |  |  | ms | 2000 | 2000 |
| T311 timer |  |  | ms | 1000 | 1000 |
| N310 |  |  |  | 1 | 1 |
| N311 |  |  |  | 1 | 1 |
| NZP CSI-RS configuration |  |  |  | TBD | TBD |
| ZP CSI-RS configuration |  |  |  | TBD | TBD |
| CSI-IM configuration |  |  |  | TBD | TBD |
| Periodic CSI reporting |  |  |  | PUCCH | PUCCH |
| CSI reporting periodicity | Config 1 |  | slot | [40] | [40] |
|  | Config 2 |  |  | [40] | [40] |
| NZP CSI-RS configuration |  |  |  | TBD | TBD |
| ZP CSI-RS configuration |  |  |  | TBD | TBD |
| CSI-IM configuration |  |  |  | TBD | TBD |
| T1 |  |  | s | TBD | TBD |
| T2 |  |  | s | TBD | TBD |
| T3 |  |  | s | TBD | TBD |
| T4 |  |  | s | TBD | TBD |
| T5 |  |  | s | TBD | TBD |
| D1 |  |  | s | TBD | TBD |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.5.5.1.2.1-3: OTA related cell specific test parameters for FR2 (Cell 2) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| UE orientation around TBD axis and TBD axis |  | degrees | TBD |  |  |  |  | TBD |  |  |  |  |
| PDCCH_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | Config 1 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  |  | dBm/15 KHz | TBD |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively inFigure A.5.5.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section [A.3.6]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.1.2.1-4: Measurement gap configuration for in-sync tests in non-DRX mode.

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [TBD] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |

Figure A.5.5.1.2.1-1: SNR variation for in-sync testing

A.5.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.3			Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode

A.5.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.5.5.1.3.1-1. The test parameters are given in Tables A.5.5.1.3.1-2, A.5.5.1.3.1-3, A.5.5.1.3.1-4, and A.5.5.1.3.1-5. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The E-UTRAN PCell setting refers to Table A.3.7.2.1-2. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.5.5.1.3.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.3.1-2: General test parameters for FR2 out-of-sync testing in DRX mode

| Parameter |  |  |  | Unit |  | Value |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  |  |  | Ce1l 1 |
| E-UTRA RF Channel Number |  |  |  |  |  | 1 |
| Active PSCell |  |  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  |  |  | 2 |
| Duplex mode |  |  |  |  |  | TDD |
| TDD Configuration |  |  | Config 1 |  |  | [ TDDConf.3.1] |
|  |  |  | Config 2 |  |  | [ TDDConf.3.1] |
| CORESET Reference Channel |  |  | Config 1 |  |  | [CR. 3.1 TDD] |
|  |  |  | Config 2 |  |  | [CR. 3.1 TDD] |
| SSB Configuration |  |  | Config 1 |  |  | [Table A.3.2.2.2.1-1] |
|  |  |  | Config 2 |  |  | [Table A.3.2.2.2.1-1] |
| SMTC Configuration |  |  | Config 1 |  |  | Table A.3.2.3.1-1 |
|  |  |  | Config 2 |  |  | Table A.3.2.3.1-1 |
| SSB index assigned as RLM RS |  |  | Config 1 |  |  | TBD |
|  |  |  | Config 2 |  |  | TBD |
| PRACH Configuration |  |  | Config 1 |  |  | TBD |
|  |  |  | Config 2 |  |  | TBD |
| OCNG parameters |  |  |  |  |  | Table A.3.2.1.1-1 |
| CP length |  |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | [2x2 Low] |
| Out of sync transmission parameters | DCI format |  |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |
|  | Aggregation level |  |  | CCE |  | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB |  | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB |  | 4 |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  |  | 6 |
| DRX cycle |  |  |  |  |  | 640 |
| Gap pattern ID |  |  |  |  |  | [N.A.] |
| Layer 3 filtering |  |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms |  | 0 |
| T311 timer |  |  |  | ms |  | 1000 |
| N310 |  |  |  |  |  | 1 |
| N311 |  |  |  |  |  | 1 |
| NZP CSI-RS configuration |  |  |  |  |  | TBD |
| ZP CSI-RS configuration |  |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  |  | TBD |
| Periodic CSI reporting |  |  |  |  |  | PUCCH |
| CSI reporting periodicity |  | Config 1 |  | slot |  | [40] |
|  |  | Config 2 |  |  |  | [40] |
| NZP CSI-RS configuration |  |  |  |  |  | TBD |
| ZP CSI-RS configuration |  |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  |  | TBD |
| T1 |  |  |  | s |  | TBD |
| T2 |  |  |  | s |  | TBD |
| T3 |  |  |  | s |  | TBD |
| D1 |  |  |  | s |  | TBD |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.5.5.1.3.1-3: OTA related cell specific test parameters for FR2 (Cell 2) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| UE orientation around TBD axis and TBD axis |  | degrees | TBD |  |  |
| PDCCH_beta |  | dB | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  |
| PSS_beta |  | dB |  |  |  |
| SSS_beta |  | dB |  |  |  |
| PDSCH_beta |  | dB |  |  |  |
| OCNG_beta |  | dB |  |  |  |
| SNR | Config 1 | dB | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD |
|  |  | dBm/15 KHz | TBD |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.5.5.1.3.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.5.5.1.3.1-4: DRX-Configuration for out-of-sync tests

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms640] |
| shortDRX | disable |

Table A.5.5.1.3.1-5: TimeAlignmentTimer -Configuration for out-of-sync testing

| Field | Test1 |
| --- | --- |
|  | Value |
| TimeAlignmentTimer | [Infinity] |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] |

Figure A.5.5.1.3.1-1: SNR variation for out-of-sync testing

A.5.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.4			Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode

A.5.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.5.5.1.4.1-1. The test parameters are given in Tables A.5.5.1.4.1-2, A.5.5.1.4.1-3, A.5.5.1.4.1-4 and Table A.5.5.1.4.1-5. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The E-UTRAN PCell setting refers to Table A.3.7.2.1-2. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.5.5.1.4.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, NR 120 KHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.4.1-2: General test parameters for FR2 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  |  | Ce1l 1 |
| E-UTRA RF Channel Number |  |  |  |  | 1 |
| Active PSCell |  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  |  | 2 |
| Duplex mode |  |  |  |  | TDD |
| TDD Configuration |  |  | Config 1 |  | [ TDDConf.3.1] |
|  |  |  | Config 2 |  | [ TDDConf.3.1] |
| CORESET Reference Channel |  |  | Config 1 |  | [CR. 3.1 TDD] |
|  |  |  | Config 2 |  | [CR. 3.1 TDD] |
| SSB Configuration |  |  | Config 1 |  | [Table A.3.2.2.2.1-1] |
|  |  |  | Config 2 |  | [Table A.3.2.2.2.1-1] |
| SMTC Configuration |  |  | Config 1 |  | Table A.3.2.3.1-1 |
|  |  |  | Config 2 |  | Table A.3.2.3.1-1 |
| SSB index assigned as RLM RS |  |  | Config 1 |  | TBD |
|  |  |  | Config 2 |  | TBD |
| PRACH Configuration |  |  | Config 1 |  | TBD |
|  |  |  | Config 2 |  | TBD |
| OCNG parameters |  |  |  |  | Table A.3.2.1.1-1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | [2x2 Low] |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX cycle |  |  |  | ms | 40 |
| Gap pattern ID |  |  |  |  | [N.A.] |
| Layer 3 filtering |  |  |  |  | Enabled |
| NZP CSI-RS configuration |  |  |  |  | TBD |
| ZP CSI-RS configuration |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  | TBD |
| Periodic CSI reporting |  |  |  |  | PUCCH |
| CSI reporting periodicity |  | Config 1 |  | slot | [40] |
|  |  | Config 2 |  |  | [40] |
| NZP CSI-RS configuration |  |  |  |  |  |  | TBD |
| ZP CSI-RS configuration |  |  |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  |  |  | TBD |
| T1 |  |  |  | s | TBD |
| T2 |  |  |  | s | TBD |
| T3 |  |  |  | s | TBD |
| T4 |  |  |  | s | TBD |
| T5 |  |  |  | s | TBD |
| D1 |  |  |  | s | TBD |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.5.5.1.4.1-3: OTA related cell specific test parameters for FR2 (Cell 2) for in-sync radio link monitoring test in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| UE orientation around TBD axis and TBD axis |  | degrees | TBD |  |  |  |  |
| PDCCH_beta |  | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |
| SNR | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
|  |  | dBm/15 KHz | TBD |  |  |  |  |
| Propagation condition |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.5.5.1.4.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section [A.3.6]. |  |  |  |  |  |  |  |

Table A.5.5.1.4.1-4: DRX-Configuration for in-sync tests

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms40] |
| shortDRX | disable |

Table A.5.5.1.4.1-5: TimeAlignmentTimer -Configuration for in-sync testing

| Field | Test 1 |
| --- | --- |
|  | Value |
| TimeAlignmentTimer | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] |

Figure A.5.5.1.4.1-1: SNR variation for in-sync testing.

A.5.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #2>>