3GPP TSG-RAN WG4 Meeting #89	R4-1814876

# Spokane, US, 12th – 16th Nov, 2018

Agenda item:		7.11.8.2

Source:		MediaTek Inc.

Title:		Discussion on requirements for L1-RSRP measurement for reporting in FR2

Document for:	Discussion

- Introduction

In last meeting, some open issues related to beam management are addressed in [1]. In this the paper, our views on requirement for L1-RSRP measurement for reporting in FR2 are provided.

- N=1 conditions and scheduling restrictions for L1-RSRP measurement for reporting in FR2

As discussed in N=1 conditions for CBD, UE would apply ‘fine’ RX beam for CBD and ‘rough’ RX beam for SMTC [2], so N=1 based on SMTC is not feasible.

Proposal 1: In FR2, for SSB based L1-RSRP measurement for reporting, N=8.

Similar to CBD, the scheduling restriction due to L1-RSRP reporting will be impacted by the assumptions on RX beam sweeping:

- Option 1: Scheduling restriction on CSI-RS resource configured for L1-RSRP reporting in FR2.

- Option 2: Scheduling restriction on CSI-RS resource configured with ‘repetition’ set as ‘ON’ for L1-RSRP reporting in FR2.

Option 2 can further to reduce the impact on scheduling restriction due to CBD, by assuming RX beam sweeping performed when ‘repetition’ is configured as ‘ON’, which means gNB will fix the direction of TX beam for CSI-RS resources to allow UE to sweep its RX beam among the resource set. If ‘repetition’ is configured as ‘OFF’ for the CSI-RS resource set, the TCI state and QCL-D assumption should be given, otherwise UE is not able to receive the data on the same OFDM symbol.

Proposal 2: In FR2, for CSI-RS based L1-RSRP for reporting, N=8 with scheduling restriction if the CSI-RS resource configured with ‘repetition’ set as ‘ON’; otherwise, N=1 and without scheduling restriction.

- Measurement period (P scaling factor) of L1-RSRP measurement for reporting in FR2

As discussed in CBD, similar to RLM/BFD, UE would apply deferent RX beam for SMTC and for serving cell, so the collision between L1-RSRP measurement for reporting and SMTC can be handled in the same ways as RLM/BFD.

Proposal 3: In FR2, regarding the scaling factor P of measurement period of L1-RSRP measurement due to colliding with MG and SMTC, P factor specified for RLM/BFD evaluation period is reused.

- Conclusion

In the contribution, we discuss the requirement for L1-RSRP measurement for reporting. We have the following observations and proposals:

Proposal 1: In FR2, for SSB based L1-RSRP measurement for reporting, N=8.

Proposal 2: In FR2, for CSI-RS based L1-RSRP for reporting, N=8 with scheduling restriction if the CSI-RS resource configured with ‘repetition’ set as ‘ON’; otherwise, N=1 and without scheduling restriction.

Proposal 3: In FR2, regarding the scaling factor P of measurement period of L1-RSRP measurement due to colliding with MG and SMTC, P factor specified for RLM/BFD evaluation period is reused.

- Reference

[1] R4-1813736, “Way forward on NR beam management”, Huawei, Nokia.

[2] R4-1814313, “Way forward on the remaining open issues for RRM test methods,” Intel Corporation, Anritsu.