3GPP TSG-RAN WG4 RAN4#89 Meeting	R4-1814930

# Spokane, USA, 12 November - 16 November, 2018

Source:	Verizon, LG Electronics Finland

Title:	TP for TR 36.716-03-02: LTE inter-band CA 3 bands DL with 2 bands UL

Agenda item:	8.6.2

Document for:	Approval

Introduction

The purpose of this document is to introduce co-existence studies and MSD analysis results [1] of LTE inter-band carrier aggregation 3 bands DL with 2 bands UL combinations for TR 36.716-03-0.

Text Proposal

<Start of TP>

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 4 and Band 13 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] |  |  | Bandwidth combination set |  |
| CA_2A-4A-13A |  | CA_2A-13A |  | 2 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 50 |  | 0 |  |
|  |  |  |  | 4 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  | 13 |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | CA_4A-13A |  | 2 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 50 |  | 0 |  |
|  |  |  |  | 4 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  | 13 |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-4A-13A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1 and Table 6.x.1.2-2.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-4A-13A with UL CA_2A-13A

| UE UL carriers |  | Fx_low |  | Fx_high |  | Fy_low |  | Fy_high |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  | 3700 |  | 3820 |  | 1554 |  | 1574 |  |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 5550 |  | 5730 |  | 2331 |  | 5550 |  |
| 2nd order IMD products |  | ¦fy_low – fx_high¦ |  | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ |  | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1133 |  | 1063 |  | 2627 |  | 2697 |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low – fy_high¦ |  | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) |  | 2913 |  | 3043 |  | 356 |  | 276 |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 4477 |  | 4607 |  | 3404 |  | 3484 |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ |  | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 4763 |  | 4953 |  | 421 |  | 511 |  |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 1298 |  | 1198 |  | 6863 |  | 6613 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ |  | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 4958 |  | 5058 |  | 8177 |  | 8427 |  |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1339 |  | 1489 |  | 4176 |  | 3976 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 6031 |  | 6181 |  | 7104 |  | 7304 |  |

Table 6.x.1.2-2: Co-existence study for DL CA_2A-4A-13A with UL CA_4A-13A

| UE UL carriers |  | Fx_low |  | Fx_high |  | Fy_low |  | Fy_high |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 777 | 787 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  | 3420 |  | 3510 |  | 1554 |  | 1574 |  |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 5130 |  | 5265 |  | 2331 |  | 2361 |  |
| 2nd order IMD products |  | ¦fy_low – fx_high¦ |  | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ |  | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 978 |  | 923 |  | 2487 |  | 2542 |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low – fy_high¦ |  | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) |  | 2633 |  | 2733 |  | 201 |  | 136 |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 4197 |  | 4297 |  | 3264 |  | 3329 |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ |  | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 4343 |  | 4488 |  | 576 |  | 651 |  |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |
| IMD frequency limits (MHz) | 5907 | 6052 | 4041 | 4116 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1846 |  | 1956 |  | 4974 |  | 5084 |  |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 1438 |  | 1353 |  | 6243 |  | 6053 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ |  | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 4818 |  | 4903 |  | 7617 |  | 7807 |  |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1059 |  | 1179 |  | 3711 |  | 3556 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 5751 |  | 5871 |  | 6684 |  | 6839 |  |

- 6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-4A-13A, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 4.

When uplink CA_4A_13A is paired with downlink CA_2A-4A-13A, the 4th order IMD product by Band 4 and band 13 falls into its own Rx frequency band 2.

In Table 6.X.1.3-1, evaluated MSD values of 3 bands DL_CA_2A-4A-13A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-4A-13A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | TBD |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | TBD |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 4 and Band 5 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  |  | Uplink CA configurations |  |  | E-UTRA Bands |  |  | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  |  |  | Maximum aggregated bandwidth [MHz] |  |  | Bandwidth combination set |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  |  |  | 4 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-4A-5A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-4A-5A with UL CA_2A-5A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

- 6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-4A-5A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 4.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-4A-5A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-4A-5A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration |  | EUTRA band |  |  | UL Fc (MHz) |  | UL BW (MHz) |  | UL CLRB |  | DL Fc (MHz) |  | DL BW (MHz) |  | MSD (dB) |  | Duplex mode |  | Source of IMD |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | TBD |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-5A-66A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-5A-66A with UL CA_2A-5A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

- 6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-4A-66A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5A-66A with 2 bands UL is shown.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | TBD |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

- The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | Uplink CA configurations |  |  | E-UTRA Bands |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] |  |  | Bandwidth combination set |  |  |
| CA_2A-5B-66A-66A |  |  | CA_2A-5A |  |  | 2 |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 5 |  |  | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 |  | 0 |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-5B-66A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-5B-66A with UL CA_2A-5A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 2002 |  | 2172 |  | 5348 |  | 5518 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

- 6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-5B-66A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5B-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-5B-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | TBD |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 46 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-5A-46D | CA_2A-5A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 46 |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-5A-46D and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-5A-46D with UL CA_2A-5A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 5550 |  | 5730 |  | 2472 |  | 2547 |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products |  | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ |  | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 5146 |  | 5306 |  | 8224 |  | 8489 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

In Table 6.x.1.2-1, 3rd harmonic from the band 2 can impact the band 46 and the band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. Therefore, there is no need to study for this harmonic problem.

- 6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-5A-46D, the 4th and 5th  order IMD products by band 2 and band 5 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5A-46D with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-5A-46D

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-5A-46D | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5488 | 20 | 100 | 5488 | 20 | TBD | TDD |  |
|  | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD5 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5276 | 20 | 100 | 5276 | 20 | TBD | TDD |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 5 and Band 46 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | Uplink CA configurations |  |  | E-UTRA Bands |  |  | 1.4 MHz |  | 3 MHz |  |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] |  |  |  | Bandwidth combination set |
| CA_5A-46D-66A |  | CA_5A_46A |  |  | 5 |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  | 90 |  |  |  | 0 |
|  |  |  |  |  |  | 46 |  |  |  |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | CA_5A_66A | 5 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  |  |  |  |  | 46 |  |  |  |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_5A-46D-66A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1 and Table 6.x.1.2-2.

Table 6.x.1.2-1: Co-existence study for DL CA_5A-46D-66A with UL CA_5A-46A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 10300 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 15450 | 17775 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4301 | 5101 | 5974 | 6774 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4277 | 3452 | 9451 | 11026 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6798 | 7623 | 11124 | 12699 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3453 | 2603 | 14601 | 16951 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7622 | 8472 | 16274 | 18624 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10202 | 8602 | 11948 | 13548 |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 22876 |  | 19751 |  | 1754 |  | 2629 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 21424 | 24549 | 8446 | 9321 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16127 | 13752 | 7753 | 9378 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17098 | 19473 | 12772 | 14397 |

Table 6.x.1.2-2: Co-existence study for DL CA_5A-46D-66A with UL CA_5A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 2472 |  | 2547 |  | 5130 |  | 5340 |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 132 | 12 | 2571 | 2736 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4182 | 4327 | 5954 | 6189 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1912 |  | 1722 |  | 5068 |  | 5258 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products |  | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ |  | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 7664 |  | 7969 |  | 5006 |  | 5176 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 6778 |  | 7038 |  | 5892 |  | 6107 |  |

In Table 6.x.1.2-1, 3rd harmonic from the band 66 can impact the band 46. Since the band 46 is already specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101, there is no need to study for this harmonic problem.

- 6.x.1.3	MSD

When uplink CA_5A_46A is paired with downlink CA_5A-46D-66A, the 5th order IMD product by band 5 and band 46 falls into the own Rx frequency band 66.

When uplink CA_5A_66A is paired with downlink CA_5A-46D-66A, the 4th and 5th order IMD products by band 5 and band 66 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD values of 3 bands DL_CA_5A-46D-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_5A-46D-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  | TDD |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | TBD | FDD |  |
|  | CA_5A_66A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 |  |  |  |
|  |  | 46 | 5208 | 20 | 100 | 5208 | 20 | TBD | TDD |  |
|  | CA_5A_66A | 5 | 846.5 | 5 | 25 | 891.5 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1774 | 5 | 25 | 2194 | 5 |  |  |  |
|  |  | 46 | 5160 | 20 | 100 | 5160 | 20 | TBD | TDD |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 MHz |  | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-13A-66A-66B and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-13A-66A-66B with UL CA_2A-13A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 2126 |  | 2266 |  | 5254 |  | 5394 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

- 6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-66A-66B, the 4th order IMD product by band 2 and band  13 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-13A-66A-66B with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-66A-66B

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | TBD |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 13 and Band 46 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  |  | Uplink CA configurations |  |  | E-UTRA Bands |  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  |  | 20 MHz | Maximum aggregated bandwidth [MHz] |  |  | Bandwidth combination set |  |
| CA_13A-46D-66A | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  |  |  | 46 |  |  | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_13A-46D-66A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_13A-46D-66A with UL CA_13A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 2331 |  | 2361 |  | 5130 |  | 5340 |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |
| IMD frequency limits (MHz) |  | 4041 |  | 4141 |  | 5907 |  | 6127 |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 6684 |  | 6914 |  | 5751 |  | 5921 |  |

In Table 6.x.1.2-1, 3rd harmonic from the band 66 can impact the band 46 and this band is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. Therefore, there is no need to study for this harmonic problem.

- 6.x.1.3	MSD

When uplink CA_13A_66A is paired with downlink CA_13A-46D-66A, the 4th and 5th order IMD products by band 13 and band 66 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_13A-46D-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_13A-46D-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_13A-46D-66A | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |  |
|  |  | 46 | 5922.5 | 20 | 100 | 5922.5 | 20 | TBD | TDD |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1727 | 5 | 25 | 2127 | 5 |  |  |  |
|  |  | 46 | 5800 | 20 | 100 | 5800 | 20 | TBD | TDD |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 46 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-46D | CA_2A-13A | 2 |  |  | Yes | Yes | Yes |  | Yes | 90 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 46 |  | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-13A-46D and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_2A-13A-46D with UL CA_2A-13A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 5550 |  | 5730 |  | 2331 |  | 2361 |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 2126 |  | 2266 |  | 5254 |  | 5394 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

In Table 6.x.1.2-1, 3rd harmonic from the band 2 can impact the band 46. Since the band 46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101, there is no need to study for this harmonic problem.

- 6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-46D, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-13A-46D with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-46D

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-46D | CA_2A-13A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 46 | 5374 | 20 | 100 | 5374 | 20 | TBD | TDD |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

----- Unchanged sections omitted -----

<End of TP>

- References

[1] R4-1812214, “ TP on summary of self-interference analysis for new x bands DL (x=3,4,5) with 2 bands UL CA in rel-16”, LG Electronics Finland

[2] TS 36.101 v15.4.0