3GPP TSG-RAN WG4 RAN4#89 Meeting	R4-1814932

# Spokane, USA, 12 November - 16 November, 2018

Source:	Verizon, LG Electronics Finland

Title:	TP for TR 36.716-03-02: LTE inter-band CA 4 bands DL with 2 bands UL

Agenda item:	8.6.2

Document for:	Approval

Introduction

The purpose of this document is to introduce co-existence studies and MSD analysis results [1] of LTE inter-band carrier aggregation 4 bands DL with 2 bands UL combinations for TR 36.716-03-0.

Text Proposal

<Start of TP>

----- Unchanged sections omitted -----

7.x LTE-A inter-band CA: Band 2 and Band 13 and Band 48  and Band 66 DL with 
2 bands UL

7.x.1	List of specific combination issues

- 7.x.1.1	Channel bandwidth per operating band for CA

Table 7.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-48C-66A |  | CA_2A-13A |  |  | 2 |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 90 |  |  | 0 |  |
|  |  |  |  |  | 13 |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 48 |  |  |  | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 66 |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | CA_13A-66A |  |  | 2 |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 90 |  |  | 0 |  |
|  |  |  |  |  | 13 |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 48 |  |  |  | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 66 |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |

- 7.x.1.2	Co-existence studies for LTE-A inter-band CA 4 bands DL CA_2A-13A-48C-66A and 
2 bands UL

For 4 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1 and Table 6.x.1.2-2.

Table 7.x.1.2-1: Co-existence study for DL CA_2A-13A-48C-66A with UL CA_2A-13A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  | 3700 |  | 3820 |  | 1554 |  | 1574 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

In Table 6.x.1.2-1, 2nd harmonic from the band 2 can possibly impact high frequency edge of the band 48 and there is 4th IMD product by the band 2 and band 13 falls into the band 66.

Table 7.x.1.2-2: Co-existence study for DL CA_2A-13A-48C-66A with UL CA_13A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 3786 |  | 3556 |  | 1059 |  | 1229 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

In Table 6.x.1.2-2, 2nd harmonic from the band 66 can impact the band 48 and there is 4th and 5th IMD products by the band 13 and band 66 falls into the band 2 and the band 48, respectively.

- 7.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-48C-66A, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 66. For this case, the MSD value from 3 DL 2A-13A-66A / 2 UL 2A-13A can be applied.

When uplink CA_13A_66A is paired with downlink CA_2A-13A-48C-66A, the 4th and 5th order IMD products by Band 13 and band 44 falls into its own Rx frequency band 2 and band 48, respectively. The MSD value from 3 DL 2A-13A-66A / 2 UL 13A-66A can be applied to 4th IMD product and the same MSD value also can be applied to 5th IMD product.

- 7.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

7.x LTE-A inter-band CA: Band 2 and Band 13 and Band 48 and Band 66 DL with 
2 bands UL

7.x.1	List of specific combination issues

- 7.x.1.1	Channel bandwidth per operating band for CA

Table 7.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 MHz |  | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] |  | Bandwidth combination set |  |
| CA_2A-13A-48A-48A-66A | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | CA_13A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

- 7.x.1.2	Co-existence studies for LTE-A inter-band CA 4 bands DL CA_2A-13A-48A-48A-66A and 2 bands UL

For 4 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1 and Table 6.x.1.2-2.

Table 7.x.1.2-1: Co-existence study for DL CA_2A-13A-48A-48A-66A with UL CA_2A-13A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6031 | 6181 | 7104 | 7304 |

In Table 6.x.1.2-1, 2nd harmonic from the band 2 can possibly impact high frequency edge of the band 48 and there is 4th IMD product by the band 2 and band 13 falls into the band 66.

Table 7.x.1.2-2: Co-existence study for DL CA_2A-13A-48A-48A-66A with UL CA_13A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  | 1554 |  | 1574 |  | 3420 |  | 3560 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 3786 |  | 3556 |  | 1059 |  | 1229 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

In Table 6.x.1.2-2, 2nd harmonic from the band 66 can impact the band 48 and there is 4th and 5th IMD products by the band 13 and band 66 falls into the band 2 and the band 48, respectively.

- 7.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-48A-48A-66A, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 66. The MSD value from 3 DL 2A-13A-66A / 2 UL 13A-66A can applied to 4th IMD product.

When uplink CA_13A_66A is paired with downlink CA_2A-13A-48A-48A-66A, the 4th and 5th order IMD products by Band 13 and band 44 falls into its own Rx frequency band 2 and band 48, respectively. The MSD value from 3 DL 2A-13A-66A / 2 UL 13A-66A can be re-used for 4th IMD product and 5th IMD product also can take the same MSD value.

- 7.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

----- Unchanged sections omitted -----

<End of TP>

- References

[1] R4-1812214, “ TP on summary of self-interference analysis for new x bands DL (x=3,4,5) with 2 bands UL CA in rel-16”, LG Electronics Finland

[2] TS 36.101 v15.4.0