3GPP TSG-RAN WG4 RAN4#89	R4-1814950

Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 RRM Side Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Side conditions for FR2 requirements are not yet defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR2 Side conditions are updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 side conditions will not be defined and test cases cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.1, 10.1.5.1, 10.1.5.2, 10.1.8.1, 10.1.13.1, 10.1.15.1, B.2.2, B.2.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS .133 V15.3.0 (2018-09)

.

## << Start of changes >>

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

- Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [8] | [11] | NR_TDD_FR2_A, NR_TDD_FR2_B, NR_TDD_FR2_F, NR_TDD_FR2_G, NR_TDD_FR2_T, NR_TDD_FR2_Y | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3 NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 4: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency in FR2.

The accuracy requirements in Table 10.1.3.1.2-1 are valid under the following conditions:

- Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 4: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 10.1.5	Inter-frequency RSRP accuracy requirementsfor FR2

#### 10.1.5.1	Inter-frequency SS-RSRP accuracy requirements

#### 10.1.5.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5.1.1-1 are valid under the following conditions:

- Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [8] | [11] | NR_TDD_FR2_A, NR_TDD_FR2_B, NR_TDD_FR2_F, NR_TDD_FR2_G, NR_TDD_FR2_T, NR_TDD_FR2_Y | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 2: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |  |  |  |

#### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.5.1.2-1 are valid under the following conditions:

- Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.5.1.2-1: SS-RSRP Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 4: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 10.1.8	Intra-frequency RSRQ accuracy requirements for FR2

#### 10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.8.1.1-1 are valid under the following conditions:

- Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.8.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [2.5] | [4] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [3.5] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 10.1.10	Inter-frequency RSRQ accuracy requirements for FR2

#### 10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.10.1.1	Aboslute Accuracy of SS-RSRQ in FR2

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.10.1.1-1 are valid under the following conditions:

- Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.10.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [2.5] | [4] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [3.5] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 4: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

##### 10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR2 compared to the RSRP measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.10.1.2-1 are valid under the following conditions:

- Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB,

- Other conditions are TBD.

Table 10.1.10.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [3] | [4] | NR_TDD_FR2_A | -1054 | -1024 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1024 | -994 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [4] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are defined in Section 3.5.3. NOTE 3: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 10.1.13	Intra-frequency SINR accuracy requirements for FR2

#### 10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2

##### 10.1.13.1.1	Absolute SS-SINR Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.13.1.1-1 are valid under the following conditions:

Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

Other conditions are TBD.

Table 10.1.13.1.1-1: SS-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [3.0] | [4] | NR_TDD_FR2_A | -1053 | -1023 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1023 | -993 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [3.5] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 10.1.15	Inter-frequency SINR accuracy requirements for FR2

#### 10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2

##### 10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.15.1.1-1 are valid under the following conditions:

Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

Other conditions are TBD.

Table 10.1.15.1.1-1: SS-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [3.0] | [4] | NR_TDD_FR2_A | -1053 | -1023 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1023 | -993 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [3.5] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

##### 10.1.15.1.2	Relative Accuracy of SS-SINR in FR2

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR2 compared to the SS-SINR measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.15.1.2-1 are valid under the following conditions:

Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

Other conditions are TBD.

Table 10.1.15.1.2-1: SS-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [3.5] | [4] | NR_TDD_FR2_A | -1053 | -1023 | N/A | -70 |
|  |  | NR_TDD_FR2_B | -1023 | -993 | N/A | -70 |
|  |  | NR_TDD_FR2_F | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_G | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_T | TBD | TBD | N/A | -70 |
|  |  | NR_TDD_FR2_Y | TBD | TBD | N/A | -70 |
| [4] | [4] | Note 4 | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

## << Unchanged sections omitted >>

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.1	Introduction

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in Section 9,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Section 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Section 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Section 10.

## B.2.2	Conditions for NR intra-frequency measurements

This section defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |
| Conditions | NR_TDD_FR2_A (n257, n258, n261) | -1053 | -1023 |
|  | NR_TDD_FR2_B (n260) | -1023 | -993 |
|  | NR_TDD_FR2_F | TBD | TBD |
|  | NR_TDD_FR2_G | TBD | TBD |
|  | NR_TDD_FR2_T | TBD | TBD |
|  | NR_TDD_FR2_Y | TBD | TBD |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. NOTE 2: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |

## B.2.3	Conditions for NR inter-frequency measurements

This section defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |
| Conditions | NR_TDD_FR2_A (n257, n258, n261) | -1053 | -1023 |
|  | NR_TDD_FR2_B (n260) | -1023 | -993 |
|  | NR_TDD_FR2_F | TBD | TBD |
|  | NR_TDD_FR2_G | TBD | TBD |
|  | NR_TDD_FR2_T | TBD | TBD |
|  | NR_TDD_FR2_Y | TBD | TBD |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. NOTE 2: Signal levels are defined at the center of the quiet zone. NOTE 3: Value corresponding to EIS spherical coverage as defined in 38.101-2, side condition applies in the directions in which EIS spherical coverage requirement is met |  |  |  |

## << End of changes >>