3GPP TSG-RAN WG4 Meeting #89	　　　　　　　　　　　　　　　　　　　　　　　　　　R4-1815055

Spokane, USA, 12th – 16th, November, 2018

Source: 	NTT DOCOMO, INC., KDDI Corporation

Title:	FR2 ETC requirements

Agenda item:	7.6.12

Document for:   Approval

- Introduction

In RAN4#88bis, there was a discussion about handling of ETC requirements in FR2, and the related WF was approved [1]. This paper discusses separately this issue from core requirement perspective and from actual test perspective. The contribution is to propose to apply the current requirement values to both NC and ETC for specific RF requirement based on the regulation perspective.

- Core requirement perspective

This section focuses core requirement perspective separately from actual test perspective: how to test and the difficulty of implementation of testing equipment are discussed in the next section. In our view, the purpose of core requirement is to show the technical requirement of UE to realize 5G communications, and thus core requirements itself should be discussed apart from the testing.

In addition, making Japanese regulation for NR mobile communications has been proceeded. We would like to note that some RF requirements have been already captured according to the current 3GPP specifications. For the regulation, it is significantly important to align with 3GPP spec because the regulation might be a barrier to entry in Japan for 3GPP-aligned NR devices. Therefore, introducing some relaxation values on ETC has serious impact on not only Japanese companies but also related all companies from the regulation perspective.

Observation 1: Introducing relaxation values on ETC causes the situation that the Japanese regulation is a barrier for 3GPP-aligned NR devices to entry in Japan since the regulation has already captured some RF requirements according to the current 3GPP specifications.

Based on the above reasons, our proposal is that the current values should be applied for both NC and ETC for at least RF requirement specified in Japanese regulation summarized in Table 2-1.

Proposal 1: RAN4 core requirement should apply the current specification values to both NC and ETC for the requirement summarized in Table 2-1.

Table 2-1: RF requirement in FR2 specified in Japanese regulation

|  | RF requirement in FR2 specified in Japanese regulation |
| --- | --- |
| Tx | - 6.2.1 UE maximum output power - Max TRP - Max EIRP - 6.3.2 Transmit OFF power - 6.4.1 Frequency Error - 6.5.1 Occupied bandwidth - 6.5.2.3 Adjacent channel leakage ratio - 6.5.2.1 Spectrum emission mask - 6.5.3 Spurious emissions |
| Rx | - 7.3.2 Reference sensitivity power level - Peak EIS - 7.5 Adjacent channel selectivity - 7.6.2 In-band blocking - 7.9 Spurious emissions |

- Test perspective

For test perspective, ideally, the all requirement should be tested on ETC. However, there are already real problems raised from the feasibility of testing equipment perspective [2]. In addition, it might be difficult for UE to meet some requirements due to expected different performance in extreme temperature conditions. To deal with this issue, we need to discuss how to guarantee the RF characteristics. A compromised solution is to allow UE to be tested on only NC instead of being tested on ETC, which is the same approach used for LTE.

Observation 2: To deal with the difficulty of testing on ETC, one possible solution is to allow UE to be tested on only NC instead of being tested on ETC, which is the same approach used for LTE.

We need to discuss carefully which requirements should be tested on ETC. We propose at least the below requirements should be tested on ETC, which are also tested in LTE.

- 6.2	Transmitter power

  - 6.2.1	UE maximum output power

  - 6.2.2	UE maximum output power reduction

  - 6.2.4	Configured transmitted power

- 6.4.1	Frequency Error

- 6.5.2.3	Adjacent channel leakage ratio

This is because that Transmitter power and ACLR is important requirements to guarantee the interference level for other UE and other system. Furthermore, Frequency error is more important requirements since it guarantees the basis of wireless communication operation. If the frequency used by UE is different from the configured frequency, the all frequency related characteristics are changed and the system might not work well.

Proposal 2: For testing perspective, at least “Transmitter power”, “Frequency error”, and “Adjacent channel leakage ratio” should be tested on ETC.

- Conclusion

This paper discusses separately this issue from core requirement perspective and from actual test perspective. The contributions of this paper are summarized as below:

Observation 1: Introducing relaxation values on ETC causes the situation that the Japanese regulation is a barrier for 3GPP-aligned NR devices to entry in Japan since the regulation has already captured some RF requirements according to the current 3GPP specifications.

Proposal 1: RAN4 core requirement should apply the current specification values to both NC and ETC for the requirement summarized in Table 2-1.

Table 2-1: RF requirement in FR2 specified in Japanese regulation

|  | RF requirement in FR2 specified in Japanese regulation |
| --- | --- |
| Tx | - 6.2.1 UE maximum output power - Max TRP - Max EIRP - 6.3.2 Transmit OFF power - 6.4.1 Frequency Error - 6.5.1 Occupied bandwidth - 6.5.2.3 Adjacent channel leakage ratio - 6.5.2.1 Spectrum emission mask - 6.5.3 Spurious emissions |
| Rx | - 7.3.2 Reference sensitivity power level - Peak EIS - 7.5 Adjacent channel selectivity - 7.6.2 In-band blocking - 7.9 Spurious emissions |

Observation 2: To deal with the difficulty of testing on ETC, one possible solution is to allow UE to be tested on only NC instead of being tested on ETC, which is the same approach used for LTE.

Proposal 2: For testing perspective, at least “Transmitter power”, “Frequency error”, and “Adjacent channel leakage ratio” should be tested on ETC.

5	References

- R4-1814173, “WF on FR2 temperature conditions for UE RF requirement verifications”, MediaTek Inc, 3GPP TSG RAN4 meeting #88bis, Chengdu, China, October 8-12, 2018

- R4-1812323, “	FR2 environmental conditions”, MediaTek Inc, 3GPP TSG RAN4 meeting #88bis, Chengdu, China, October 8-12, 2018