3GPP TSG-RAN WG4 Meeting #89	                                                      R4-1815056

Spokane, USA, 12th – 16th, November, 2018

Source: 	NTT DOCOMO, INC., Toyota InfoTechnology Center, Sharp

Title:	Handling UE types in FR2

Agenda item:	7.6.13.1

Document for:   Approval

- Introduction

In RAN4#87, additional power class 1, 2, 4 for FR2 were introduced to TS 38.101-2. Some UE types such as “Handheld UE” and “FWA” is specified in the current spec. In the previous meeting, there was a discussion whether or not UE types should be introduced to spec [2, 3], but there is no conclusion. This paper shares pros and cons of introducing UE types and proposes to remove the UE designator from TS 38.101-2.

- Background

In RAN4#87, additional power class 1, 2, 4 for FR2 were introduced to TS 38.101-2. To proceed with the discussion, some UE types were assumed such that PC3 was assumed to be used for Handheld UE and PC1 was assumed to be used for FWA UE. In the ad-hoc session, there was a discussion about the handing the UE types, that is, whether such UE types should be described in TS 38.101-2 or not. After the discussion, the below agreement were reached[3]:

Agreement: Don’t introduce type designator as a field in the specification. But if needed, RAN4 can consider to add notes (or other methods) for some power class.

Observation 1: There was the previous agreement not to introduce the UE type designator in TS 38.101-2.

However, against the above agreements, there are some UE type designator in a field of the specification in the current spec.

- Discussion

The below table summarize Pros and Cons:

|  | Specification with UE types | Specification without UE types |
| --- | --- | --- |
| Pros | - It can be helpful for spec readers to understand the background of discussion introducing each PC. | - Clear definition based on power requirement. - No restriction for any kind of actual UE product |
| Cons | - Definition of each UE type is ambiguous and might confusing the spec readers. - Other types of mmWave devices in future might be precluded. | - Spec readers cannot know the background of discussion. |

Our preference is not introducing any UE types designator in TS 38.101-2. The main concern is that UE types would restrict flexibility of implementation of mmWave devices and hinder the development of mmWave industry. Assuming that we put the UE types in spec based on the current assumption, e.g., “Handheld UE”, however we might come up with new ideas using PC3 in the future, and then what if the new devices seems not be “Handheld UE”?

Observation 2: If UE types are introduced in the spec, there were concerns that UE types would restrict flexibility of implementation of mmWave devices and hinder the development of mmWave industry.

If we manage to solve this issue in such case, there is another issue. If we might try to put new definition of UE types or introduce new power class one by one, however, it has serious impact on the RAN4 workload. Furthermore, since there are power class-basis specifications in spec, and thus introducing many power class makes spec complicated.

Observation 3: If UE types are introduced in the spec, we need to put new definition of UE types or introduce new power class one by one, which impact on the RAN4 workload and complexity of spec

Proposal 1: RAN4 should make specification not to restrict flexibility of implementation of mmWave devices and hinder the development of mmWave industry.

To avoid these issue, we should define each power class based on only power requirement. It is more clear for people to decide which power class each UE is categorized. All companies can make any types of mmWave devices without taking care of UE types.

Observation 4: It can provide a clear understanding if just specifying the power class requirement without introducing UE types designator.

Proposal 2: Exact UE types such as FWA and Handheld UE for each power class should be removed from TS 38.101-2.

- Conclusion

The contributions of this paper are summarized as below:

Observation 1: There was the previous agreement not to introduce the UE type designator in TS 38.101-2.

Observation 2: If UE types are introduced in the spec, there were concerns that UE types would restrict flexibility of implementation of mmWave devices and hinder the development of mmWave industry.

Observation 3: If UE types are introduced in the spec, we need to put new definition of UE types or introduce new power class one by one, which impact on the RAN4 workload and complexity of spec

Observation 4: It can provide a clear understanding if just specifying the power class requirement without introducing UE types designator.

Proposal 1: RAN4 should make specification not to restrict flexibility of implementation of mmWave devices and hinder the development of mmWave industry.

Proposal 2: Exact UE types such as FWA and Handheld UE for each power class should be removed from TS 38.101-2.

-----------------------------------TS 38.101-2-----------------------------------

### 6.2.1	UE maximum output power

#### 6.2.1.1	UE maximum output power for power class 1

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.1-1: UE minimum peak EIRP for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1-2: UE maximum output power limits for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.1-3: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| NOTE 1: Minimum EIRP at 85%-tile CDF is defined as the lower limit without tolerance |  |

#### 6.2.1.2	UE maximum output power for power class 2

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.2-1: UE minimum peak EIRP for power class 2

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 29 |
| n258 | 29 |
| n261 | 29 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.2-2: UE maximum output power limits for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 60th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.2-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.2-3: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0 |
| n258 | 18.0 |
| n261 | 18.0 |
| NOTE 1: Minimum EIRP at 60%-tile CDF is defined as the lower limit without tolerance |  |

#### 6.2.1.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found on the Table6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

#### 6.2.1.4	UE maximum output power for power class 4

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.4-1: UE minimum peak EIRP for power class 4

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 34 |
| n258 | 34 |
| n260 | 31 |
| n261 | 34 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.4-2: UE maximum output power limits for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 20th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.4-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.4-3: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |
| NOTE 1: Minimum EIRP at 20%-tile CDF is defined as the lower limit without tolerance |  |

-------------------------------------------------------------------------------------

5	References

- RP-181473, “Ad-Hoc Minutes for FWA and New UE Type Related Requirements”, Samsung, 3GPP TSG RAN4 meeting #87, Busan, Korea, May 21-25, 2018

- R4-1812421, “UE type for FR2 Power Class in Rel-15”, LG Electronics, 3GPP TSG RAN4 meeting #88bis, Chengdu, China, October 8-12, 2018

- R4-1813071, “Handling UE types in FR2”, NTT DOCOMO, INC, 3GPP TSG RAN4 meeting #88bis, Chengdu, China, October 8-12, 2018