3GPP TSG-RAN WG4 Meeting #89	R4-1815078

Spokane, USA, 12th – 16th, November, 2018

Agenda item:	9.6.2

Source:	NTT DOCOMO, INC.

Title:	TP for TR 37.716-41-11 EN-DC 3-19-21-42_n78

Document for:	Approval

1	Introduction

In RAN4#88bis meeting, the revised WID EN-DC of 4 LTE band and 1 NR band was approved [1]. In this contribution, a text proposal is provided for EN-DC 3-19-21-42_n78 for TR 38.716-41-11.

2	Text proposal

----- < Start of Text Proposal for TR 38.716-41-11>-----

## 5.1.x	DC_3-19-21-42_n78

### 5.1.x.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-19-21-42_n781 | CA_3-19-21-42 | n78 | DC_3_n78 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

### 5.1.x.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-21A-42A_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42A | n78 |
| DC_3A-19A-21A-42A_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42A | CA_n78C |
| DC_3A-19A-21A-42C_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42C | n78 |
| DC_3A-19A-21A-42C_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42C | CA_n78C |

### 5.1.x.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

----- < End of Text Proposal >-----

3 Reference

- R4-1812045, “Revised WID of EN-DC of 4 LTE band and 1 NR band”, Nokia, Nokia Shanghai Bell