3GPP TSG-RAN WG4 Meeting #89                	R4-1815086

Spokane, US, 12 – 16 November 2018

Title:	Draft LS on UE measurement capability

Response to:

Release:	Release 15

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Xuesong Wang

E-mail Address:	cedar.wang@hisilicon.com

Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 the following progress RAN4 has made on UE measurement capability:

- For each intra-frequency layer the UE shall be capable of monitoring at least 6 cells on a single serving carrier (PCC or PSCC or the SCC with MO associated to report configuration with rsType set to ssb if PCC/PSCC is in a band different from SCC) out of all the serving carriers configured in the same band.

- For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 24 SSB with different SSB index and/or PCI on a single serving carrier (PCC or PSCC or the SCC with MO associated to report configuration with rsType set to ssb if PCC/PSCC is in a band different from SCC) out of all the serving carriers configured in the same band. UE shall be capable of monitoring 2 SSB(s) on serving cell for each of the other serving carrier(s) in the same band. UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR on all above-mentioned SSBs

2. Actions:

To TSG RAN WG2

RAN4 kindly asks RAN2 to take above information into account in their work.

3. Date of Next TSG-RAN WG4 Meeting:

3GPPRAN4#89		     12 November – 16 November 2018     Spokane, USA

3GPPRAN4#90		     25 February – 1 March 2019                Athens, Greece

References

[1] R4-1815085 Discussion on the remaining issues of UE measurement capability