3GPP TSG-RAN WG4 Meeting #89   							   R4-1815091

Spokane, US, 12 - 16 November, 2018

Title: 	Discussion on the impact of dual SMTC periodicities on intra-frequency carrier

Source: 		Huawei, Hisilicon

Agenda item:		7.11.3.2.2

Document for:			Discussion

Introduction

Impact of dual SMTC periodicities is under discussion in RAN4. Following agreements are reached in RAN4 #88

RAN4 #88 Agreements: - RAN4 to decide in the next meeting on which SMTC periodicity is assumed in a intra-frequency layer in below requirements - Category A: Intra-frequency measurement requirements on the frequency layer with dual SMTC periodicities - Category B: Other RRM requirements confirmed that is impacted - Possible list of category B requirements - In TS 38.133 - 8.2.1.2.5.1 Interruptions during measurements on deactivated NR SCC - 8.5 Link Recovery Procedures - 9.2 NR intra-frequency measurements - 9.2.5.3 Scheduling availability of UE during intra-frequency - 9.3 NR inter-frequency measurements - 9.4 Inter-RAT measurement measurements - 8.1 Radio link monitoring - In TS 36.133 - 8.17.3 E-UTRA Inter-frequency Measurements when configured with EN-DC Operation - 8.17.4 E-UTRA Inter-RAT NR Measurements when Configured with EN-DC Operation - Companies are encouraged to confirm the list of requirements are impacted by dual SMTC periodicities - RAN4 to discuss which options will be used to define the requirements: - Option 1: - Both Category A and Category B assume smtc1 - Option 2: - Both Category A and Category B assume smtc2 - Option 3: - Category A always assumes smtc1 - Category B always assumes smtc2 - Option 4: - Scenario-based rule - Category A always assumes smtc1 - Category A always assumes smtc1 Smtc2. Fully overlapped; Partially overlapped; Fully non-overlapped. Smtc1; Fully overlapped; Meas. within MG smtc2; Meas. within MG smtc1; N/A. Meas. outside MG N/A; Meas. outside MG smtc2. Partially overlapped; N/A; Meas. within MG N/A; N/A. Meas. outside MG smtc2. Fully non-overlapped; N/A; Meas. within MG N/A; Meas. within MG N/A. Meas. outside MG smtc2; Meas. outside MG smtc2. - Option 5: - Category A assumes the configured smtc1 or smtc2 depending on PCI - Category B: FFS

In this contribution, we continue to discuss on this issue.

Discussion on impact of dual SMTC periodicities

Connected state is our only concern since dual SMTC periodicities for intra-frequency carrier are only available in connected state. In this section, we list all requirements related to SMTC periodicities in connected state and analyze the possible impacts.

- Handover, RRC re-establishment, RRC release with redirection and SCell activation.

Generally speaking, dual SMTCs have nothing to do with delay requirements for handover, re-establishment, release with redirection and SCell activation since only single SMTC periodicity will be configured in RRC command according to 38.331 (texts labeled yellow in the following table).

However, as the texts labeled blue in the following tables, there exists a possibility that smtc IE is absent. Then according to the description in 38.331 UE should use the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If there are dual SMTC periodicities configured in that MO it is unclear which SMTC periodicity should be used. Since smtc2 in measObjectNR contains a PCI list for all cells which use smtc2, we propose to use smtc2 for those cells in PCI list of smtc2 and use smtc1 for other cells.

ReconfigurationWithSync ::= SEQUENCE { spCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Need M newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, rach-ConfigDedicated CHOICE { uplink RACH-ConfigDedicated, supplementaryUplink RACH-ConfigDedicated } OPTIONAL, -- Need N ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]] }

ReconfigurationWithSync field descriptions. rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).. smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and intra-NR handover. For case of intra-NR handover, it is based on the timing reference of PCell. For case of NR PSCell change, it is based on the timing reference of PSCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing..

Table 1: SMTC configuration for handover

SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd sCellConfigDedicated ServingCellConfig OPTIONAL, -- Cond SCellAddMod ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]] } SCellConfig field descriptions. smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. It is based on the timing reference of SpCell of associated cell group. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Table 2: SMTC configuration for SCell activation

RedirectedCarrierInfo ::= CHOICE { nr CarrierInfoNR, eutra RedirectedCarrierInfo-EUTRA, ... } RedirectedCarrierInfo-EUTRA ::= SEQUENCE { eutraFrequency ARFCN-ValueEUTRA, cnType-r15 ENUMERATED {epc,fiveGC} OPTIONAL } CarrierInfoNR ::= SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, smtc SSB-MTC OPTIONAL, -- Need S ... } CarrierInfoNR field descriptions. carrierFreq Indicates the redirected NR frequency.. ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.. smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing..

Table 3: SMTC configuration for release with redirection

Proposal 1: If smtc IE is absent in RRC command for handover/RRC re-establishment/RRC release with redirection/SCell activation, smtc2 in MO shall be used for cells in pci-list of smtc2 and smtc1 in MO shall be used for other cells.

- RLM, BFD and L1-RSRP.

According to the conclusion of RAN4, RLM/BFD/L1-RSRP are only performed on SSB/CSI-RS outside the MG in FR1, and only perform RLM/BFD/L1-RSRP on the SSBs/CSI-RSs outside the MG and SMTC in FR2 if such SSBs/CSI-RSs exists. And RLM/BFD/L1-RSRP and RRM measurement should share the SSB/CSI-RS occasion if all SSB/CSI-RS for RLM/BFD/L1-RSRP outside MG are fully overlapped with SMTC. In this two case, dual SMTC periodicities should be considered in calculating the scaling factor for RLM/BFD/L1-RSRP evaluation period and so on.

In RAN4 AH1807 we have discussed the impact of dual SMTC periodicities on RLM evaluation period. Contradiction occurs in certain scenarios when second SMTC is configured for intra-frequency carrier. Take the scenario depicted in fig. 1 as an example. Assume RLM-RS and smtc2 have the same periodicity which is twice the periodicity of smtc1. RLM-RS and smtc2 are fully overlapped. RLM-RS and smtc1 are partially overlapped. If we treat two SMTCs separately, then UE should perform intra-frequency measurements on all SMTC occasions of smtc1 and do RLM on RLM-RSs which are non-overlapped with smtc1. On the other hand, RLM and intra-frequency should share all the SMTC occasions of smtc2 since RLM-RS and smtc2 are fully overlapped. Then comes the contradiction: shall we only perform RLM measurement on SMTC window A in fig.1 or can do both RLM and intra-frequency measurement? Shall we only perform intra-frequency measurement on SMTC window B in fig.1 or can do both RLM and intra-frequency measurement?

Figure.1 Demonstration of contradiction occurred when treat smtc1 and smtc2 separately

In order to avoid confusion, RAN4 has agreed that periodicity of SMTC2 should be used when calculating the scaling factor for RLM. In our opinion this principle can also applies to BFD and L1-RSRP. So we propose to use periodicity of SMTC2 in RLM/BFD/L1-RSRP requirements.

Proposal 2: If configured, smtc2 in MO is used in RLM/BFD/L1-RSRP requirements, otherwise smtc1 is used.

- Scheduling availability for SSB based intra-frequency measurements.

It is already agreed that smtc2 will be used.

Observation 1: It is agreed that smtc2 should be used in scheduling availability for SSB based intra-frequency measurements if configured, Otherwise SMTC1 should be used.

- Impacts on measuring multiple intra-frequency carriers.

In RAN4 #88 some company mentions that collision of intra-frequency SMTCs will makes requirements for intra-frequency measurements more complicated [1]. However, this concern is unnecessary. Although discussion on the scaling factor KCA isn’t finished yet, RAN4 has already reached conclusion on KCA for FR1. For FR1 two searcher are used as baseline. One searcher is used for PCell/PSCell and another one is shared among SCells.

RAN4 #87 Agreements： FR1 only operation - 1 additional searcher equivalent (2 searchers) RAN4 #88 Agreements： Kca for FR1 PCell/PSCell; Kca for FR1 SCells; FR1 overlap definition; Kca for FR2 PCell/PScell; Kca for FR2 SCC where UE measures up to 6 cells; Kca for other FR2 SCells; FR2 overlap definition. FR1 CA without NR-NR DC; 1; Number of configured SCells; N/A; N/A; N/A. FR2 CA without NR-NR DC; N/A; N/A; N/A; 1; N/A. Note : Only intraband CA is considered in FR2, all NR serving cells are within the same band in FR2.

So two intra-frequency carriers can be measured in parallel if they are one PCC/PSCC and one SCC. And if they are two SCCs, they can be measured in TDM manner depicted in fig.2 since requirement has already been scaled by KCA which equals to number of configured SCells. So at least for FR1, collision of intra-frequency SMTC won’t affect requirements for intra-frequency measurements.

Fig.2 impact of dual SMTC periodicities don’t exist for measuring multiple intra-frequency carriers

Observation 2: collision of intra-frequency SMTCs won’t affect requirements for intra-frequency measurement for FR1 so there is no need to consider dual SMTCs in the discussion of measurement on multiple serving CCs.

Proposal 3: Impact of dual SMTCs on measurement on multiple serving CCs is not considered.

- Intra-frequency measurement requirements

In last meetings RAN4 has discussed the collision of SMTC for intra-frequency measurement without gap (type A measurement) or intra-frequency measurement with interruption (type B measurement) and MG. The principle to solve the collision is:

Principle 1: If SMTC is fully overlapped with MG, Type A/B measurement should share the measurement occasion with measurements which need gaps such as intra-frequency measurements with gap (Type C measurements) and inter-frequency measurements/inter-RAT measurements (Type D measurements).

Principle 2: If SMTC is partially overlapped with MG, Type A/B measurement are only performed on SMTC occasions outside gap.

However, dual SMTCs are seldom considered in previous discussion. Due to the different periodicities, smtc1 and smtc2 may have different overlapping relationship with MG. Here we list all possible scenarios.

- Case 1: smtc1 and smtc2 are both fully overlapped with MG. In this case (fig.3), principle 1 should apply. Both SMTC1 and SMTC2 should share with MG.

Fig.3 demonstration of case 1.

- Case 2: smtc1 is fully overlapped with MG, smtc2 is partially overlapped with MG. If we use principle 2 to only perform type A/B measurement outside MG, then the cells which are not in PCI-list of smtc2 will never be measured.

Fig.4 demonstration of case 2

To solve this problem, we propose that UE performs measurements on cells which are not listed in pci-list of smtc2 in SMTC occasions only in MG hence CSSFwithin_gap shall apply, and cells listed in pci-list of smtc2 shall be only measured outside MG hence CSSFwithout_gap shall apply..

- Case 3: smtc1 and smtc2 are both partially overlapped with MG. In this case (fig.5), principle 2 should apply. Both smtc1 and smtc2 should be measured outside MG.

Fig.5 demonstration of case 3

- Case 4: smtc1 is non-overlapped, smtc2 is partially overlapped with MG. In this case (fig. 6a) principle 2 should apply. Both smtc1 and smtc2 should be measured outside MG. There is a possibility that all SMTC occasions only belongs to smtc2 are overlapped with MG. As a result cells in PCI-list will be measured as frequently as cells not in PCI-list (fig. 6b). However, UE can work anyway.

Fig.6a demonstration of case 4

Fig.6b another demonstration of case 4

- Case 4: smtc1 and smtc2 are both non-overlapped with MG. In this case (fig.6), both smtc1 and smtc2 are measured normally.

Fig.6 Another case of case 4

Our opinions on impact of dual SMTC periodicities on intra-frequency measurements are summarized in the following table:

Proposal 4: When dual SMTC periodicities are configured for intra-frequency carrier, UE behavior given in the following table should be adopted.

|  |  | smtc2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Fully overlapped | Partially overlapped | Fully non-overlapped |
| smtc1 | Fully overlapped | Measure within MG smtc1 | Measure within the MG smtc1 | N/A |
|  |  | Measure within MG smtc2 | Measure outside MG smtc2 |  |
|  | Partially overlapped | N/A | Measure outside the MG smtc1 | N/A |
|  |  |  | Measure outside the MG smtc2 |  |
|  | Fully non-overlapped | N/A | Measure outside MG smtc1 | Measure outside MG smtc1 |
|  |  |  | Measure outside MG smtc2 | Measure outside MG smtc2 |

Conclusion

In this contribution remaining issues on impact of dual SMTCs are discussed and the following observations and proposals are given.

Observation 1: It is agreed that SMTC2 should be used in scheduling availability for SSB based intra-frequency measurements if configured, Otherwise SMTC1 should be used.

Observation 2: collision of intra-frequency SMTCs won’t affect requirements for intra-frequency measurement for FR1 so there is no need to consider dual SMTCs in the discussion of measurement on multiple serving CCs.

Proposal 1: SMTC1 in MO should be used for handover, RRC re-establishment and RRC release with redirection if SMTC is absent in RRC command.

Proposal 2: If configured, SMTC2 in MO should be used in RLM/BFD/L1-RSRP requirements. Otherwise SMTC1 should be used.

Proposal 3: Impact of dual SMTCs on measurement on multiple serving CCs is not considered.

Proposal 4: When dual SMTCs are configured for intra-frequency carrier, UE behavior given in the following table should be adopted.

|  |  | Smtc2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Fully overlapped | Partially overlapped | Fully non-overlapped |
| Smtc1 | Fully overlapped | Measure within MG smtc1 | Measure within the MG smtc1 | N/A |
|  |  | Measure within MG smtc2 | Measure outside MG smtc2 |  |
|  | Partially overlapped | N/A | Measure outside the MG smtc1 | N/A |
|  |  |  | Measure outside the MG smtc2 |  |
|  | Fully non-overlapped | N/A | Measure outside MG smtc1 | Measure outside MG smtc1 |
|  |  |  | Measure outside MG smtc2 | Measure outside MG smtc2 |

Reference

[1] R4-1810948

[2] R4-1811688