3GPP TSG-RAN WG4 Meeting #89	R4-1815202

Spokane, Washington, USA 12 – 16 November 2018

Source:	Ericsson

Title:	TP to TS 38.141-2: Section 4.9.2 2 NR FR2 Test Models

Agenda item:	7.9.4.2

Document for:	Approval

# 1	Introduction

In this contribution some additional updates were made for clarification and missed implementation based upon previous agreements.

# 2	References

- TS 38.141-1 v1.1.0

[Start of Text Proposal]

#### 4.9.2.2	NR FR2 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR- FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier space

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR2 TDD, test models are derived based on the uplink/downlink configuration as showing in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon.

[If the number of downlink symbols in a special slot is less than 7 symbols, no measurements will be performed on that special slot.]

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | 60 kHz SCS | 120 kHz SCS |
| --- | --- | --- |
| referenceSubcarrierSpacing |  |  |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 ms | 1.25 ms |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all NR FR2 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH. Specific physical channel parameters for NR FR2 test models are described in sections 4.9.2.2.1 to 4.9.2.2.3.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH

| Parameter |  |
| --- | --- |
| # of symbols used for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| Note: Remaining empty REs in PDCCH not containing PDCCH will be filled with PDSCH |  |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the each symbol | NRB – 6 |
| DM-RS configuration and density |  | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |  |
| ratio of PDCCH EPRE to DM-RS EPRS | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH

| Parameter |  |
| --- | --- |
| DM-RS configuration and density | Type 1, comb 2 I0 = 2 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

[End of Text Proposal]