3GPP TSG-RAN WG4 Meeting #89	R4-1815221

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5214 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Slot/Subslot-PDSCH demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are no Slot/Subslot-PDSCH demodulation requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the UE demodulation requirements for slot-PDSCH and subslot-PDSCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot verify the performance of slot-PDSCH and subslot-PDSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14 (new), A.3.1.1, A.3.16 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## 8.14	Demodulation (UE supporting Short TTI)

The performance requirements specified in this clause are valid for UEs capable of short TTI.

### 8.14.1	Slot-PDSCH and Subslot-PDSCH

#### 8.14.1.1	FDD (Fixed Reference Channel)

The parameters specified in Table 8.14.1.1-1 are valid for FDD unless otherwise stated.

Table 8.14.1.1-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Number of HARQ processes |  |  | Processes | 8 for Slot-PDSCH [8] for subslot-PDSCH with proc-Timeline=set1 (Note 1) [12] for subslot-PDSCH with proc-Timeline=set2 (Note 1) |  |
| Minimum Processing Time (Note 2) |  |  |  | Xp=[4] (proc-Timeline=set1) Xp=[6] (proc-Timeline=set2) |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Note 1: As specified in TS36.211 Table 8.1-2. Note 2: Subslot-PDSCH only. As specified in TS36.213 7.3. |  |  |  |  |  |

##### 8.14.1.1.1	Open-loop spatial multiplexing performance

The requirements are specified in Table 8.14.1.1.1-3, with the addition of the parameters in Table 8.14.1.1.1-1 and Table 8.14.1.1.1-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.1.1-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Cell specific reference signal |  |  | 0,1,2,3 | 0,1,2,3 |
| DL TTI length |  |  | Slot | Subslot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| Used/Unused SPDCCH resource indication |  |  | N/A | ‘10’ |
| PDSCH transmission mode |  |  | TM3 | TM3 |
| DCI format |  |  | 7-1B | 7-1B |

Table 8.14.1.1.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 1 | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Rate Matching mode |  | Mode 1 | Not configured |
| SPDCCH L1 Reuse Indication |  | Not configured | {2, 0} |
| Aggregation level | SCCE | 2 | 2 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1, and PRB = {35, 36, …, 49} for Set 2 |  |  |  |

Table 8.14.1.1.1-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 0.5 | R.sTTI.1 FDD | OP.1 FDD | EVA30 | 4x2 Low | 70 | TBD | ≥2 |
| 2 | 10 MHz 16QAM 0.45 | R.sTTI.2 FDD | OP.1 FDD | EVA30 | 4x2 Low | 70 | TBD | ≥2 |

##### 8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)

The requirements are specified in Table 8.14.1.1.2-3, with the addition of the parameters in Table 8.14.1.1.2-1 and Table 8.14.1.1.2-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.1.2-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| CSI-RS signals |  |  | 15, 16 | 15, 16 |
| CSI-RS periodicity and subframe offset |  |  | 5 / 2 | 5 / 2 |
| CSI-RS configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration |  |  | Not configured | Not configured |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | 2 | 2 |
|  |  | ms | 1 | 1 |
| Beamforming precoder |  |  | Annex B.4.2 | Annex B.4.2 |
| Cell specific reference signal |  |  | 0, 1 | 0, 1 |
| DL TTI length |  |  | Slot | Subslot |
| DMRS position indicator |  |  | N/A | 0 for subslots 1,3, and 5 1 for subslots 2 and 4 |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| Used/Unused SPDCCH resource indication |  |  | ’10’ | N/A |
| PDSCH transmission mode |  |  | TM9 | TM9 |
| DCI format |  |  | 7-1F | 7-1F |

Table 8.14.1.1.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence signal |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 2 | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 8 | 8 |
| Rate Matching mode |  | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication |  | {1, 1} | Not configured |
| Aggregation level | SCCE | 4 | 4 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 7} for Set 1, and PRB = {8, 9, …, 15} for Set 2 |  |  |  |

Table 8.14.1.1.2-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.sTTI.3 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | TBD | ≥2 |
| 2 | 10 MHz QPSK 1/3 | R.sTTI.4 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | TBD | ≥2 |

#### 8.14.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.14.1.2-1 are valid for TDD unless otherwise stated.

Table 8.14.1.2-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Number of HARQ processes |  |  | Processes | 8 (Slot-PDSCH) |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4] Note 2: as specified in Table 4.2-1 in TS 36.211 [4] |  |  |  |  |  |

##### 8.14.1.2.1	Open-loop spatial multiplexing performance

The requirements are specified in Table 8.14.1.2.1-3, with the addition of the parameters in Table 8.14.1.2.1-1 and Table 8.14.1.2.1-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.2.1-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Subframe configuration |  |  | Non-MBSFN |
| Cell specific reference signal |  |  | 0,1,2,3 |
| DL TTI length |  |  | Slot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |
| Used/Unused SPDCCH resource indication |  |  | N/A |
| PDSCH transmission mode |  |  | TM3 |
| DCI format |  |  | 7-1B |

Table 8.14.1.2.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence signal |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 1 | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Rate Matching mode |  | Mode 1 | Not configured |
| SPDCCH L1 Reuse Indication |  | Not configured | {2, 0} |
| Aggregation level | SCCE | 2 | 2 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1, and PRB = {35, 36, …, 49} for Set 2 |  |  |  |

Table 8.14.1.2.1-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 0.5 | R.sTTI.1 FDD | OP.1 TDD | EVA30 | 4x2 Low | 70 | TBD | ≥2 |

##### 8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)

The requirements are specified in Table 8.14.1.2.2-3, with the addition of the parameters in Table 8.14.1.2.2-1 and Table 8.14.1.2.2-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.2.2-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| CSI-RS signals |  |  | 15, 16 |
| CRS-RS periodicity and subframe offset |  |  | 5 / 4 |
| CSI-RS configuration |  |  | 0 |
| Zero-power CRI-RS configuration |  |  | Not configured |
| Subframe configuration |  |  | Non-MBSFN |
| Precoder update granularity |  | SREG | 2 |
|  |  | ms | 1 |
| Beamforming precoder |  |  | Annex B.4.2 |
| Cell specific reference signal |  |  | 0,1 |
| DL TTI length |  |  | Slot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |
| Used/Unused SPDCCH resource indication |  |  | ’10’ |
| PDSCH transmission mode |  |  | TM9 |
| DCI format |  |  | 7-1F |

Table 8.14.1.2.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 2 | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 8 | 8 |
| Rate Matching mode |  | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication |  | {1, 1} | Not configured |
| Aggregation level | SCCE | 4 | 4 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 7} for Set 1, and PRB = {8, 9, …, 15} for Set 2 |  |  |  |

Table 8.14.1.2.2-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.sTTI.2 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | TBD | ≥2 |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### A.3.1.1	Overview of DL reference measurement channels

Table A.3.1.1-1W: Overview of DL reference measurement channels (Slot-PDSCH/Subslot-PDSCH)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.16.1-1 | R.sTTI.1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥2 | Slot-PDSCH |
| FDD | Table A.3.16.1-2 | R.sTTI.2 FDD | 10 | 16QAM | 0.45 | 50 |  | ≥2 | Subslot-PDSCH |
| FDD | Table A.3.16.1-3 | R.sTTI.3 FDD | 10 | QPSK | 1/3 | 50 |  | ≥2 | Slot-PDSCH |
| FDD | Table A.3.16.1-4 | R.sTTI.4 FDD | 10 | QPSK | 1/3 | 50 |  | ≥2 | Subslot-PDSCH |
| TDD |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.16.2-1 | R.sTTI.1 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥2 | Slot-PDSCH |
| TDD | Table A.3.16.2-1 | R.sTTI.2 TDD | 10 | QPSK | 1/3 | 50 |  | ≥2 | Slot-PDSCH |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

## A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements

### A.3.16.1	FDD

Table A.3.16.1-1: Fixed Reference Channel Slot-PDSCH (Cell-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.1 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frames 0,5 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Slot index 0 |  | 2 |
| Slot index 1 |  | 4 |
| For Sub-Frame 0,5 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Slot index 0 | Bits | 22400 |
| Slot index 1 | Bits | 28288 |
| For Sub-Frame 0,5 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 20.448 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission happens only on slot 0. For an information bit payload first transmitted at slot 1, any retransmission happens on only slot 1. |  |  |

Table A.3.16.1-2: Fixed Reference Channel Subslot-PDSCH (Cell-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.2 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.45 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 5160 |
| Subslot index 2 | Bits | 3880 |
| Subslot index 3 | Bits | 2664 |
| Subslot index 4 | Bits | 3880 |
| Subslot index 5 | Bits | 5160 |
| For Sub-Frames 0,5 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frame 0,5 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 12032 |
| Subslot index 2 | Bits | 8832 |
| Subslot index 3 | Bits | 5888 |
| Subslot index 4 | Bits | 8832 |
| Subslot index 5 | Bits | 12288 |
| For Sub-Frame 0,5 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 16.5952 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at subslot 1, any retransmission happens only on subslots 1 and 5. For an information bit payload first transmitted at subslot 2, any retransmission happens only on subslots 2 and 4. For an information bit payload first transmitted at subslot 3, any retransmission happens only on subslot 3. For an information bit payload first transmitted at subslot 4, any retransmission happens only on subslots 2 and 4. For an information bit payload first transmitted at subslot 5, any retransmission happens only on subslots 1 and 5. |  |  |

Table A.3.16.1-3: Fixed Reference Channel Slot-PDSCH (User-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.3 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 3624 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 2,7 |  |  |
| Slot index 0 | Bits | 2856 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 0,5 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frames 2,7 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frame 0,5 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 10000 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frames 2,7 |  |  |
| Slot index 0 | Bits | 9600 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frame 0,5 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 6.2592 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission happens only on slot 0. For an information bit payload first transmitted at slot 1, any retransmission happens only on slot 1. |  |  |

Table A.3.16.1-4: Fixed Reference Channel Subslot-PDSCH (User-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.4 FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Allocated subframes per Radio Frame |  | 8 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | Bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frames 2,7 |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | Bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frames 0,5 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frames 2,7 |  |  |
| Subslot index 1 |  | 1 |
| Subslot index 2 |  | 1 |
| Subslot index 3 |  | 1 |
| Subslot index 4 |  | 1 |
| Subslot index 5 |  | 1 |
| For Sub-Frame 0,5 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | Bits | 4032 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frames 2,7 |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | Bits | 3696 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frame 0,5 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 5.3056 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at subslot 1, any retransmission happens only on subslots 1 and 5. For an information bit payload first transmitted at subslot 2, any retransmission happens only on subslots 2 and 4. For an information bit payload first transmitted at subslot 3, any retransmission happens only on subslot 3. For an information bit payload first transmitted at subslot 4, any retransmission happens only on subslots 2 and 4. For an information bit payload first transmitted at subslot 5, any retransmission happens only on subslots 1 and 5. Note 2: In any retransmission, no information bit payloads are scheduled at subslot #n when information bit payloads are not scheduled at subslot #(n-1). |  |  |

### A.3.16.2	TDD

Table A.3.16.2-1: Fixed Reference Channel Slot-PDSCH (Cell-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.1 TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Uplink-Downlink Configurtion |  | 1 |
| Allocated subframes per Radio Frame (D) |  | 2 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 1/2 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 |  | 2 |
| Slot index 1 |  | 4 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 22400 |
| Slot index 1 | Bits | 28288 |
| For Sub-Frame 0,1,5,6 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 5.112 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission happens only on slot 0. For an information bit payload first transmitted at slot 1, any retransmission happens only on slot 1. |  |  |

Table A.3.16.2-2: Fixed Reference Channel Slot-PDSCH (User-Specific Reference Signals)

| Parameter | Unit |  |
| --- | --- | --- |
| Reference channel |  | R.sTTI.2 TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Uplink-Downlink Configurtion |  | 1 |
| Allocated subframes per Radio Frame (D) |  | 2 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 2856 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 0,1,5,6 |  | N/A |
| Number of Code Blocks |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 |  | 1 |
| Slot index 1 |  | 1 |
| For Sub-Frames 0,1,5,9 |  | N/A |
| Binary Channel Bits |  |  |
| For Sub-Frames 4,9 |  |  |
| Slot index 0 | Bits | 9600 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frame 0,1,5,6 |  | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 1.4496 |
| UE Category |  | ≥2 |
| Note 1: For an information bit payload first transmitted at slot 0, any retransmission happens only on slot 0. For an information bit payload first transmitted at slot 1, any retransmission happens only on slot 1. |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------