3GPP TSG-RAN WG4 Meeting #89	R4-1815224

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	6.7.6.2

Source:	Ericsson

Title:	sTTI CQI reporting tests

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 agreed with the way forward on sTTI demodulation and CQI reporting test [1], and we provided the simulation results in RAN4#88bis for CRS-based PDSCH [2] and DMRS-based PDSCH [3]. This contribution discusses the SNR test points and requirements of sTTI CQI reporting test for both CRS-based and DMRS-based PDSCH.

# 2	CRS-based PDSCH

## 2.1	CQI to MCS table

Table 1 and Table 2 show the CQI to MCS table and effective coding rate, and the difference from the target coding rate for slot-PDSCH and subslot-PDSCH, respectively. Available resource elements are calculated based on the assumption:

- SF#0 and SF#5 are DTX as same as the existing wideband CQI reporting test. In this case, we only need to consider the subframes without PSS/SSS/PBCH and SI transmission.

- Reduce the REs used for SPDCCH transmission (AL4 = 16 RBs with one OFDM symbol)

- CFI=2

- 2 CRS ports

It is observed from Table 1 and Table 2 that there are several CQI indexes mapping to MCS whose coding rate is deviated from the target coding rate, as highlighted in yellow (threshold = 0.05). From the CQI reporting test point of view, it is not preferable to set SNR test points corresponding to these CQI indexes.

Table 1	CQI-to-MCS table and difference to the target coding rate for CRS-based Slot-PDSCH.

|  |  |  | Slot 0 |  |  | Slot 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  |
|  |  | Available REs | 2800 |  |  | 3672 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.13 | 0.05 | 0 | 0.10 | 0.02 |
| 2 | 0.12 | 2 | 0 | 0.13 | 0.01 | 1 | 0.13 | 0.01 |
| 3 | 0.19 | 2 | 2 | 0.2 | 0.01 | 3 | 0.20 | 0.01 |
| 4 | 0.3 | 2 | 4 | 0.32 | 0.02 | 5 | 0.31 | 0.00 |
| 5 | 0.44 | 2 | 6 | 0.47 | 0.03 | 7 | 0.43 | 0.01 |
| 6 | 0.59 | 2 | 7 | 0.56 | 0.03 | 9 | 0.55 | 0.04 |
| 7 | 0.37 | 4 | 10 | 0.36 | 0.01 | 13 | 0.39 | 0.02 |
| 8 | 0.48 | 4 | 12 | 0.45 | 0.03 | 15 | 0.48 | 0.00 |
| 9 | 0.6 | 4 | 14 | 0.51 | 0.02 | 16 | 0.53 | 0.07 |
| 10 | 0.46 | 6 | 17 | 0.46 | 0.01 | 20 | 0.45 | 0.00 |
| 11 | 0.55 | 6 | 19 | 0.55 | 0.01 | 23 | 0.58 | 0.02 |
| 12 | 0.65 | 6 | 21 | 0.64 | 0.01 | 25 | 0.64 | 0.01 |
| 13 | 0.75 | 6 | 23 | 0.75 | 0.00 | 27 | 0.72 | 0.03 |
| 14 | 0.85 | 6 | 25 | 0.85 | 0.01 | 28 | 0.84 | 0.02 |
| 15 | 0.93 | 6 | 26 | 0.91 | 0.01 | 28 | 0.84 | 0.09 |

Table 2	CQI-to-MCS table and difference to the target coding rate for CRS-based Subslot-PDSCH.

|  |  |  | Subslot 1 |  |  | Subslot 2 |  |  | Subslot 3 |  |  | Subslot 4 | Subslot 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 | 50 |  |  |
|  |  | Available REs | 1408 |  |  | 1008 |  |  | 872 |  |  | 1008 | 1472 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.09 | 0.01 | 0 | 0.12 | 0.05 | 0 | 0.14 | 0.07 | 0 | 0 | 0.08 | 0.01 |
| 2 | 0.12 | 2 | 1 | 0.11 | 0.00 | 0 | 0.12 | 0.01 | 0 | 0.14 | 0.03 | 0 | 1 | 0.11 | 0.01 |
| 3 | 0.19 | 2 | 3 | 0.18 | 0.01 | 2 | 0.20 | 0.01 | 1 | 0.18 | 0.01 | 2 | 3 | 0.17 | 0.02 |
| 4 | 0.3 | 2 | 6 | 0.32 | 0.02 | 4 | 0.31 | 0.01 | 3 | 0.28 | 0.02 | 4 | 6 | 0.30 | 0.00 |
| 5 | 0.44 | 2 | 8 | 0.42 | 0.02 | 6 | 0.44 | 0.01 | 5 | 0.44 | 0.00 | 6 | 9 | 0.46 | 0.02 |
| 6 | 0.59 | 2 | 9 | 0.48 | 0.11 | 8 | 0.59 | 0.00 | 7 | 0.61 | 0.02 | 8 | 9 | 0.46 | 0.13 |
| 7 | 0.37 | 4 | 14 | 0.39 | 0.02 | 11 | 0.37 | 0.00 | 10 | 0.39 | 0.02 | 11 | 14 | 0.37 | 0.00 |
| 8 | 0.48 | 4 | 16 | 0.45 | 0.02 | 13 | 0.48 | 0.01 | 12 | 0.49 | 0.01 | 13 | 16 | 0.43 | 0.04 |
| 9 | 0.6 | 4 | 16 | 0.45 | 0.15 | 15 | 0.59 | 0.01 | 14 | 0.62 | 0.02 | 15 | 16 | 0.43 | 0.17 |
| 10 | 0.46 | 6 | 22 | 0.46 | 0.01 | 18 | 0.46 | 0.00 | 17 | 0.49 | 0.03 | 18 | 22 | 0.44 | 0.01 |
| 11 | 0.55 | 6 | 24 | 0.55 | 0.01 | 20 | 0.56 | 0.01 | 18 | 0.53 | 0.03 | 20 | 25 | 0.54 | 0.01 |
| 12 | 0.65 | 6 | 27 | 0.64 | 0.01 | 22 | 0.65 | 0.00 | 20 | 0.65 | 0.00 | 22 | 27 | 0.61 | 0.04 |
| 13 | 0.75 | 6 | 28 | 0.74 | 0.01 | 24 | 0.76 | 0.01 | 22 | 0.75 | 0.01 | 24 | 28 | 0.71 | 0.04 |
| 14 | 0.85 | 6 | 28 | 0.74 | 0.11 | 26 | 0.86 | 0.00 | 24 | 0.88 | 0.03 | 26 | 28 | 0.71 | 0.14 |
| 15 | 0.93 | 6 | 28 | 0.74 | 0.18 | 27 | 0.89 | 0.04 | 25 | 0.92 | 0.01 | 27 | 28 | 0.71 | 0.22 |

## 2.2	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

To decide the test points, we choose CQI indexes 7 and 11 for both slot-PDSCH and subslot-PDSCH because there CQI indexes map to MCS whose coding rate is close to the target. We can also choose lower CQI indexes such as 3 or 4, however our simulation result shows the corresponding SNR is -6dB or lower. We also want to avoid setting low SNR region considering the less reliability of SNR estimation.

Table 3 and Table 4 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. Figure 1 and Figure 2 show the median CQI and throughput ratio values for reference. For the test metrics, our simulation results show we can reuse the existing metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: For CRS-based PDSCH, reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table 3	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 1 | 7 | 41.4 | 0.09 | 1.40 |
| 2 | 7 | 42.8 | 0.08 | 1.35 |
| 9 | 11 | 37.6 | 0.09 | 1.50 |
| 10 | 11 | 39.6 | 0.10 | 1.35 |

Table 4	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 1 | 7 | 41.1 | 0.18 | 1.32 |
| 2 | 7 | 42.3 | 0.19 | 1.26 |
| 9 | 11 | 35.7 | 0.14 | 1.44 |
| 10 | 11 | 37.0 | 0.13 | 1.30 |

Figure 1	Median CQI.

Figure 2	Throughput ratio between the follow CQI and Fixed median CQI.

# 3	DMRS-based PDSCH

## 2.1	CQI to MCS table

Table 5, Table 6, and Table 7 show the CQI to MCS table and effective coding rate, and the difference from the target coding rate for slot-PDSCH and subslot-PDSCH, respectively. Available resource elements are calculated based on the assumption:

- SF#0 and SF#5 are DTX as same as the existing wideband CQI reporting test. In this case, we only need to consider the subframes without PSS/SSS/PBCH and SI transmission.

- Reduce the REs used for SPDCCH transmission (AL8 = 16 RBs with two OFDM symbols)

- CFI=2

- 1 CRS port

- 2 DMRS ports (CSI-RS configuration 0).

- 2 NZP-CSI-RS ports (SF#1 and #6)

As same as CRS-based transmission, it is observed from Table 5, Table 6, and Table 7 that there are several CQI indexes mapping to MCS whose coding rate is deviated from the target coding rate, as highlighted in yellow (threshold = 0.05).

Table 5	CQI-to-MCS table and difference to the target coding rate for DMRS-based Slot-PDSCH.

|  |  |  | Non CSI-RS |  |  |  |  |  | 2 CSI-RS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 |  |  | Slot 1 |  |  | Slot 0 |  |  | Slot 1 |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 |
|  |  | Available REs | 2600 |  |  | 3348 |  |  | 2500 (CSI-RS) |  |  | 3348 |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |
| 1 | 0.08 | 2 | 0 | 0.14 | 0.06 | 0 | 0.11 | 0.03 | 0 | 0.14 | 0.07 | 0 |
| 2 | 0.12 | 2 | 0 | 0.14 | 0.02 | 0 | 0.11 | 0.01 | 0 | 0.14 | 0.03 | 0 |
| 3 | 0.19 | 2 | 1 | 0.18 | 0.01 | 2 | 0.17 | 0.02 | 1 | 0.19 | 0.00 | 2 |
| 4 | 0.3 | 2 | 3 | 0.28 | 0.02 | 4 | 0.27 | 0.03 | 3 | 0.29 | 0.01 | 4 |
| 5 | 0.44 | 2 | 5 | 0.43 | 0.01 | 7 | 0.47 | 0.01 | 5 | 0.45 | 0.01 | 7 |
| 6 | 0.59 | 2 | 7 | 0.50 | 0.02 | 9 | 0.60 | 0.00 | 7 | 0.63 | 0.04 | 9 |
| 7 | 0.37 | 4 | 10 | 0.39 | 0.02 | 12 | 0.37 | 0.01 | 10 | 0.40 | 0.03 | 12 |
| 8 | 0.48 | 4 | 12 | 0.48 | 0.00 | 14 | 0.49 | 0.02 | 12 | 0.50 | 0.02 | 14 |
| 9 | 0.6 | 4 | 14 | 0.63 | 0.03 | 16 | 0.58 | 0.02 | 13 | 0.58 | 0.03 | 16 |
| 10 | 0.46 | 6 | 17 | 0.50 | 0.05 | 19 | 0.46 | 0.00 | 17 | 0.52 | 0.07 | 19 |
| 11 | 0.55 | 6 | 18 | 0.53 | 0.02 | 21 | 0.54 | 0.02 | 18 | 0.55 | 0.00 | 21 |
| 12 | 0.65 | 6 | 20 | 0.64 | 0.01 | 23 | 0.63 | 0.02 | 20 | 0.67 | 0.02 | 23 |
| 13 | 0.75 | 6 | 22 | 0.74 | 0.02 | 26 | 0.76 | 0.01 | 22 | 0.77 | 0.01 | 26 |
| 14 | 0.85 | 6 | 24 | 0.87 | 0.02 | 27 | 0.79 | 0.06 | 23 | 0.84 | 0.01 | 27 |
| 15 | 0.93 | 6 | 25 | 0.91 | 0.02 | 28 | 0.92 | 0.01 | 24 | 0.91 | 0.02 | 28 |

Table 6	CQI-to-MCS table and difference to the target coding rate for DMRS-based Subslot-PDSCH (non-CSI-RS subframe).

|  |  |  | Subslot 1 |  |  | Subslot 2 |  |  | Subslot 3 |  |  | Subslot 4 | Subslot 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 | 50 |  |  |
|  |  | Available REs | 1180 |  |  | 680 |  |  | 612 |  |  | 680 | 1212 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.11 | 0.03 | 0 | 0.18 | 0.11 | 0 | 0.20 | 0.13 | 0 | 0 | 0.10 | 0.02 |
| 2 | 0.12 | 2 | 0 | 0.11 | 0.01 | 0 | 0.18 | 0.07 | 0 | 0.20 | 0.09 | 0 | 1 | 0.13 | 0.01 |
| 3 | 0.19 | 2 | 2 | 0.17 | 0.02 | 0 | 0.18 | 0.01 | 0 | 0.02 | 0.01 | 0 | 3 | 0.20 | 0.02 |
| 4 | 0.3 | 2 | 5 | 0.33 | 0.02 | 2 | 0.29 | 0.01 | 2 | 0.32 | 0.02 | 2 | 5 | 0.32 | 0.02 |
| 5 | 0.44 | 2 | 7 | 0.45 | 0.01 | 4 | 0.46 | 0.02 | 3 | 0.41 | 0.03 | 4 | 7 | 0.44 | 0.00 |
| 6 | 0.59 | 2 | 9 | 0.57 | 0.02 | 5 | 0.56 | 0.02 | 5 | 0.63 | 0.04 | 5 | 9 | 0.55 | 0.03 |
| 7 | 0.37 | 4 | 12 | 0.36 | 0.01 | 10 | 0.49 | 0.12 | 10 | 0.55 | 0.18 | 10 | 12 | 0.35 | 0.02 |
| 8 | 0.48 | 4 | 14 | 0.46 | 0.02 | 10 | 0.49 | 0.02 | 10 | 0.55 | 0.07 | 10 | 15 | 0.49 | 0.01 |
| 9 | 0.6 | 4 | 16 | 0.54 | 0.06 | 12 | 0.62 | 0.02 | 11 | 0.61 | 0.01 | 12 | 16 | 0.53 | 0.07 |
| 10 | 0.46 | 6 | 19 | 0.44 | 0.01 | 17 | 0.63 | 0.17 | 17 | 0.70 | 0.24 | 17 | 20 | 0.47 | 0.01 |
| 11 | 0.55 | 6 | 22 | 0.55 | 0.00 | 17 | 0.63 | 0.07 | 17 | 0.70 | 0.14 | 17 | 22 | 0.54 | 0.02 |
| 12 | 0.65 | 6 | 24 | 0.65 | 0.00 | 17 | 0.63 | 0.02 | 17 | 0.70 | 0.05 | 17 | 25 | 0.66 | 0.01 |
| 13 | 0.75 | 6 | 27 | 0.76 | 0.00 | 19 | 0.77 | 0.01 | 18 | 0.75 | 0.00 | 19 | 27 | 0.74 | 0.01 |
| 14 | 0.85 | 6 | 28 | 0.89 | 0.03 | 20 | 0.83 | 0.02 | 19 | 0.85 | 0.00 | 20 | 28 | 0.86 | 0.01 |
| 15 | 0.93 | 6 | 28 | 0.89 | 0.04 | 22 | 0.89 | 0.03 | 20 | 0.92 | 0.00 | 22 | 28 | 0.86 | 0.06 |

Table 7	CQI-to-MCS table and difference to the target coding rate for DMRS-based Subslot-PDSCH (CSI-RS subframe).

|  |  |  | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 1180 | 612 (CSI-RS) | 612 | 680 | 1212 |
| CQI index | Code rate | Modulation |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 2 | 0 | 0 | 0 | 3 |
| 4 | 0.3 | 2 | 5 | 2 | 2 | 2 | 5 |
| 5 | 0.44 | 2 | 7 | 3 | 3 | 4 | 7 |
| 6 | 0.59 | 2 | 9 | 5 | 5 | 5 | 9 |
| 7 | 0.37 | 4 | 12 | 10 | 10 | 10 | 12 |
| 8 | 0.48 | 4 | 14 | 10 | 10 | 10 | 15 |
| 9 | 0.6 | 4 | 16 | 11 | 11 | 12 | 16 |
| 10 | 0.46 | 6 | 19 | 17 | 17 | 17 | 20 |
| 11 | 0.55 | 6 | 22 | 22 | 17 | 17 | 22 |
| 12 | 0.65 | 6 | 24 | 24 | 17 | 17 | 25 |
| 13 | 0.75 | 6 | 27 | 27 | 18 | 19 | 27 |
| 14 | 0.85 | 6 | 28 | 28 | 19 | 20 | 28 |
| 15 | 0.93 | 6 | 28 | 28 | 20 | 22 | 28 |

## 2.2	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

As we discussed in CRS-based CQI test, we should avoid setting SNR test points whose CQI indexes result in large coding difference between target and actual one. For slot-PDSCH, we choose CQI indexes 8 and 12 because there CQI indexes map to MCS whose coding rate is close to the target. Like CRS-based transmission, we don’t choose lower CQI index. For subslot-PDSCH, it is difficult to find CQI index for 16QAM mapping to MCS close to target coding rate. Therefore we assume 64QAM only, and which corresponds to CQI index 13.

Table 7 and Table 8 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. Figure 3 and Figure 4 show the median CQI and throughput ratio values for reference. For the test metrics, our simulation results show we can reuse the existing metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 3: For DMRS-based PDSCH, set two sets of test points {3, 4} and {12, 13} for both slot-based wideband CQI reporting test.

Proposal 4: For DMRS-based PDSCH, set one set of test points {15, 16} for subslot-based wideband CQI reporting test.

Proposal 5: For DMRS-based PDSCH, reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table 8	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 3 | 8 | 40.6 | 0.09 | 1.40 |
| 4 | 8 | 40.9 | 0.09 | 1.31 |
| 12 | 12 | 36.2 | 0.09 | 1.40 |
| 13 | 13 | 36.3 | 0.08 | 1.56 |

Table 9	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 15 | 13 | 32.8 | 0.17 | 1.23 |
| 16 | 13 | 34.2 | 0.19 | 1.13 |

Figure 3	Median CQI.

Figure 4	Throughput ratio between the follow CQI and Fixed median CQI.

# 4	Conclusion

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: For CRS-based PDSCH, reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Proposal 3: For DMRS-based PDSCH, set two sets of test points {3, 4} and {12, 13} for both slot-based wideband CQI reporting test.

Proposal 4: For DMRS-based PDSCH, set one set of test points {15, 16} for subslot-based wideband CQI reporting test.

Proposal 5: For DMRS-based PDSCH, reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table below summarizes our proposal.

|  |  | SNR test points (dB) | Alpha | Gamma | Delta |
| --- | --- | --- | --- | --- | --- |
| CRS-based | Slot-PDSCH | {1, 2}, {9,10} | 20% | 1.05 | 0.02 |
|  | Subslot-PDSCH | {1, 2}, {9,10} | 20% | 1.05 | 0.02 |
| DMRS-based | Slot-PDSCH | {3, 4}, {12, 13} | 20% | 1.05 | 0.02 |
|  | Subslot-PDSCH | {15, 16} | 20% | 1.05 | 0.02 |

# 5	References

- R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

- R4-1812823, “CQI reporting tests for CRS-based PDSCH”, Ericsson.

- R4-1812824, “CQI reporting tests for DMRS-based PDSCH”, Ericsson.

# Appendix

Table 10	TBS table for slot/subslot-PDSCH of 50PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |