3GPP TSG-RAN WG4 Meeting #89	R4-1815234

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4805 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of BS demodulation requirements for eFeMTC (TS36.104) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Perf |  |  |  |  |  | Date: |  |  | 2018-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of new PUSCH demodulation requirement for eFeMTC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of PUSCH demodulation requirements for CEModeA with EPA200 Introduction of PUSCH demodulation requirements with subPRB transmission |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 eFeMTC features are not verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8,2,10 (new), A.25 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.141 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 8.2.7	Requirements for PUSCH supporting coverage enhancement

For the parameters specified in Table 8.2.7-1 the throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables8.2.7-2 to 8.2.7-11 at the given SNR.

Table 8.2.7-1 Test Parameters for PUSCH

| Parameter | unit | Mode A | Mode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 TDD: 0, 0, 0, 0, 0, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 1, 1, 1, 1,1 |
| Number of PUSCH repetitions |  | 8 | 256 |
| Frequency hopping |  | ON | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD | 4: FDD 5: TDD |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] |  |  |  |

Table 8.2.7-2 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A3-2 | 70% | -6.2 |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% | [-7.1] |

Table 8.2.7-3 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A3-2 | 70% | -6.6 |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% | [-7.1] |

Table 8.2.7-4 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A3-2 | 70% | -6.9 |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% | [-7.1] |

Table 8.2.7-5 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A3-2 | 70% | -6.9 |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% | [-7.1] |

Table 8.2.7-6 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A3-2 | 70% | -7.0 |
| 1 | 2 | Mode A | ETU 200Hz Low | A3-2 | 70% | [-7.1] |

Table 8.2.7-7 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | ETU 1Hz Low | A3-1 | 70% | -15.0 |

Table 8.2.7-8 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | ETU 1Hz Low | A3-1 | 70% | -15.2 |

Table 8.2.7-9 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | ETU 1Hz Low | A3-1 | 70% | -15.3 |

Table 8.2.7-10 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | ETU 1Hz Low | A3-1 | 70% | -15.1 |

Table 8.2.7-11 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | ETU 1Hz Low | A3-1 | 70% | -15.2 |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### 8.2.10	Requirements for PUSCH supporting subPRB transmission

For the parameters specified in Table 8.2.10-1 the throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables8.2.10-2 to 8.2.10-11 at the given SNR.

The requirements apply to the BS supporting the PUSCH with subPRB transmission.

Table 8.2.10-1 Test Parameters for PUSCH

| Parameter | unit | Mode A | Mode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences (Note 2) |  | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions |  | 1 | 16 |
| Frequency hopping |  | ON | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD | 4: FDD 5: TDD |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] Note 2: RV changes per repetition every (RU size x number of RUs) transmission subframes. |  |  |  |

Table 8.2.10-2 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A25-1 | 70% | [-0.3] |

Table 8.2.10-3 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A25-1 | 70% | [-0.5] |

Table 8.2.10-4 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A25-1 | 70% | [-0.5] |

Table 8.2.10-5 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A25-1 | 70% | [-0.7] |

Table 8.2.10-6 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low | A25-1 | 70% | [-0.7] |

Table 8.2.10-7 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | EPA 5Hz Low | A25-2 | 70% | [-9.9] |

Table 8.2.10-8 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | EPA 5Hz Low | A25-2 | 70% | [-10.2] |

Table 8.2.10-9 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | EPA 5Hz Low | A25-2 | 70% | [-10.3] |

Table 8.2.10-10 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | EPA 5Hz Low | A25-2 | 70% | [-10.2] |

Table 8.2.10-11 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode B | EPA 5Hz Low | A25-2 | 70% | [-10.3] |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

# A.25	Fixed Reference Channels for PUSCH with SubPRB transmission

Table A.25-1 FRC parameters for performance requirements (subPRB transmission)

| Reference channel | A25-1 | A25-2 |
| --- | --- | --- |
| Diversity | No | No |
| Allocated resource blocks | 1 | 1 |
| Number of subcarriers | 2 out of 3 | 2 out of 3 |
| DFT-OFDM Symbols per subframe | 12 | 12 |
| Modulation | π/2 BPSK | π/2 BPSK |
| Code rate | 1/3 | 1/3 |
| MCS index | 0 | 0 |
| Payload size (bits) | 32 | 88 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96*2 | 96*2 |
| Total symbols per resource unit | 96*2 | 96*2 |
| Frequency offset | 0 | 0 |
| Number of RUs | 1 | 2 |
| Tx time (ms) | 8 | 16 |

------------------------------------------------------------- End of change ------------------------------------------------------------