3GPP TSG-RAN WG4 Meeting #89	R4-1815236

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	5.3.2

Source:	Ericsson

Title:	Test case for MSG3-based channel quality report

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed with the finalization of core requirements of the MSG3-based channel quality reporting [1][2]. RAN4 also discussed the test case for the MSG3-based channel quality reporting and agreed with the way forward [3].

This contribution discusses the test cases based on the way forward.

# 2	Discussion

## 2.1	Test case scenario

RAN4 agreed with the test procedure below for the scenarios listed in Table 1 [3].

Procedure

- TE sets Es/Iot.

- TE receives CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB depending on the RRC message

- TE transmits NPDCCH with repetition number R and verifies NPDCCH BLER does not exceed 1%.

- TE transmits NPDCCH with repetition number based on TS36.133 Table 9.1.22.16-1 and verifies NPDCCH BLER is larger than 1%.

Table 1	Test parameters for MSG3-based channel quality report.

| Test number | Test 1 | Test 2 |
| --- | --- | --- |
| Ês/Iot | More than -6dB, e.g. -4dB | Less than -6dB, e.g., -12dB |
| DCI format | N1 | N1 |
| Antenna configuration | 2x1 | 2x1 |
| Channel model | AWGN | AWGN |
| Aggregation level | 2 | 2 |
| DRX | OFF | OFF |
| Channel quality IE | CQI-NPDCCH-NB | CQI-NPDCCH-NB |

## 2.2	NPDCCH simulation results

Figure 1 shows NPDCCH simulation results for in-band and shows the possible test points. For normal coverage, if we set Es/Noc = -6dB, we expect UE reports “candidateRep-C”, which corresponds to the repetition level 4. From our simulation results, with Es/Noc = -6dB, Pm-dsg < 1% with repetition level 4 and Pm-dsg > 1% with repetition level 1 (=4/4).

On the other hand, for enhanced coverage, if we set Es/Noc = -12dB, we expect UE reports “candidateRep=E”, which corresponds to the repetition level 16. From our simulation results, with Es/Noc = -12dB, Pm-dsg < 1% with repetition level 16 and Pm-dsg > 1% with repetition level 2 (=16/8).

Since we ran the simulation under ideal condition, considering the impairment margin, we propose to set Es/Iot = -4dB for test 1 (normal coverage) and Es/Iot = -10dB for test 2 (enhanced coverage).

Figure 1	NPDCCH simulation result for in-band operation.

Proposal: RAN4 sets the Es/Noc as follows:

|  | Test 1 | Test 2 |
| --- | --- | --- |
| Coverage mode | Normal coverage | Enhanced coverage |
| Es/IoT | -4 dB | -10 dB |

# 3	Conclusion

Proposal: RAN4 sets the Es/Noc as follows:

|  | Test 1 | Test 2 |
| --- | --- | --- |
| Coverage mode | Normal coverage | Enhanced coverage |
| Es/IoT | -4 dB | -10 dB |

# References

- R4-1813669, “Finalize MSG3-based channel quality report for NB-IoT”, Ericsson.

- R4-1813504, “Correction to downlink channel quality reporting”, Qualcomm.

- R4-1813670, “Way forward on Test case list for MSG3 based channel quality report”, Ericsson.

# Appendix

Table 9.1.22.15-1: Downlink channel quality measurement report mapping of CQI-NPDCCH-NB when the DL channel quality reporting is supported [7]

| Reported value | NPDCCH repetition level |
| --- | --- |
| noMeasurement | No measurement reporting |
| candidateRep-A | 1 |
| candidateRep-B | 2 |
| candidateRep-C | 4 |
| candidateRep-D | 8 |
| candidateRep-E | 16 |
| candidateRep-F | 32 |
| candidateRep-G | 64 |
| candidateRep-H | 128 |
| candidateRep-I | 256 |
| candidateRep-J | 512 |
| candidateRep-K | 1024 |
| candidateRep-L | 2048 |

Table 9.1.22.16-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |