3GPP TSG-RAN WG4 Meeting #89	R4-1815243

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	7.6.2

Source:	Ericsson

Title:	TDD configuration for UE RF Tx requirements RMC

Document for:	Discussion

# 1	Introduction

RAN4 received the LS from RAN5 on the issue for UE Tx test for simultaneous LTE and NR transmissions in EN-DC [1].

Issue 1 As a fundamental issue regardless of test cases and EN-DC types, it is not clear from TS 38.101-3 v15.3.0 whether the each TRx requirment should be tested with synchrous EN-DC or asynchrnous EN-DC. In case simultansous EN-DC shall apply, UL RMC of LTE and NR, measurement period and time offset between LTE and NR should be harmonizedly specified. Issue 2 Even if the TRx requirements are tested with synchronous EN-DC, it is observed simultaneous transmission of LTE and NR is not ensured for SCS 15kHz and SCS 30kHz as analyzed above. Issue 3 For TDD-TDD intra-band LTE-NR EN-DC case, current NR UL RMC and LTE UL RMC do not avoid DL/UL interference.

To response to RAN5, we discuss the TDD configuration used for UE Tx test, and propose a new TDD configuration to solve the issues.

# 2	Discussion

## 2.1	LTE TDD configuration for LTE-NR EN-DC

According to the LTE RAN4 UE RF requirements [2] or RAN5 conformance test specification [3], it is assumed to set TDD UL-DL configuration to 1 (a.k.a. DUSSD DSUUD). For special subframe configuration, although RAN4 requirements do not specify any configuration, RAN5 specification sets to 5.

Table 6.3.4.1.4.3-4: TDD-Config-DEFAULT: On/OFF time mask measurement (TS36.521-1)

| Derivation Path: 36.508 clause 5.3.1 Table 5.3.1-1 (SystemInformationBlockType1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp5 | To enable two symbol UpPTS, and to have 9 symbols GP. |  |
| } |  |  |  |

According to LS from RAN5 [1], they are also considering TE configures the same TDD UL/DL configuration and special subframe configuration as LTE UE Tx test (TS36.521-1) for EN-DC test. Even for RAN4, we think it is straightforward to reuse the same TDD UL-DL configuration and special subframe configuration as LTE UE test because the Tx requirements for LTE part is same as TS36.101 according to TS38.101-3 4.2 as follows:

d) Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

## 2.2	NR TDD configuration for UE Tx test

As pointed out by RAN5 LS, TDD configurations of both LTE and NR should be aligned for TDD-TDD intra-band LTE-NR EN-DC scenario to avoid the cross link interference. If RAN4 agree to configure the same LTE TDD UL/DL configuration as LTE UE test for LTE-NR EN-DC, NR TDD configuration should also align with LTE configuration.

Another issue is the contiguous UL transmission period. RAN4 received another LS from RAN5 on remaining pending issues [5], and it points out the measurement period UE maximum output power, Transmit OFF power, and frequency error, shall be at least one sub frame (1ms) according to TS38.101-1 and TS38.101-2.

| TC | Title | Items pending on RAN4 | R4 part | R5 target |
| --- | --- | --- | --- | --- |
| 6.2.1 6.3.2 6.4.1 | UE maximum output power Transmit OFF power Frequency error | “Confirm measurement period for MOP, transmit off power and frequency error: - On one hand minimum time unit that can be scheduled in NR is one slot - If 1ms is the measurement period for these requirements, there will be discontinuous measurements in time to collect 1ms according to current UL RMC definition for certain SCS.” | RF, Demod | RAN5#81 (Nov-18) |

On the other hand, the current TDD RMC assumes the same TDD configuration used for REFSENS test. This means the UL transmission time for SCS=60kHz is 0.25ms and for SCS=120kHz is 0.125ms per one TDD DL-UL pattern. As RAN5 commented in LS,

If 1ms is the measurement period for these requirements, there will be discontinuous measurements in time to collect 1ms according to current UL RMC definition for certain SCS.

If RAN5 specified the test procedure to collect 4 or more UL slots and verify UE Tx performance for SCS=60kHz, it makes RAN5 more complex. We therefore propose to introduce a new TDD configuration for UL test which ensures at least 1ms contiguous transmission period for FR2.

Figure 1	Proposed TDD configuration for NR.

Figure 2	Proposed special slot configuration for NR.

Figure 1 shows the proposed TDD UL-DL configuration (slot-based) for NR and Figure 2 shows the TDD configuration (symbol-based) corresponding to LTE special subframe. As it is shown our proposal is simply to follow LTE TDD configuration. With this configuration, FR2 UE test secures the UL contiguous measurement period of 2ms.

TDD configurations in Figure 1 and Figure 2 can be configured by RRC signaling as follows:

| Parameters | SCS=15kHz | SCS=30kHz | SCS=60kHz | SCS=120kHz |
| --- | --- | --- | --- | --- |
| Pattern 1 (4ms periodicity) |  |  |  |  |
| Number of downlink slots | 1 | 2 | 4 | 9 |
| Number of downlink symbols | 3 | 6 | 12 | 10 |
| Number of symbols for gap | (9) | (18) | (36) | (72) |
| Number of uplink symbols | 2 | 4 | 8 | 2 |
| Number of uplink slots | 2 | 4 | 8 | 17 |
| Pattern 2 (1ms periodicity) |  |  |  |  |
| Number of downlink slots | 1 | 2 | 4 | 8 |
| Number of downlink symbols | 0 | 0 | 0 | 0 |
| Number of symbols for gap | (0) | (0) | (0) | (0) |
| Number of uplink symbols | 0 | 0 | 0 | 0 |
| Number of uplink slots | 0 | 0 | 0 | 0 |

# 3	Conclusion

Proposal 1: RAN4 TS38.101-3 should specify the LTE UL-DL configuration 1 and special subframe configuration 5 for TDD-TDD LTE-NR EN-DC Tx UE test.

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink-Downlink Configuration Note1 |  | 1 |
| Special subframe configuration Note 2 |  | 5 |
| Note 1: As per Table 4.2-2 in TS 36.211. Note 2: As per Table 4.2-1 in TS 36.211. |  |  |

Proposal 2: TS38.101-1 should specify the following NR TDD configuration for UL RMC to synchronize with the LTE UL-DL configuration 1 and special subframe configuration 5, and to ensure the contiguous UL measurement period of 1ms.

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| UL-DL configuration | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60 kHz |
|  | Pattern 1 |  |  |  |
|  | dl-UL-TransmissionPeriodicity | 4 ms | 4 ms | 4 ms |
|  | nrofDownlinkSlots | 1 | 2 | 4 |
|  | nrofDownlinkSymbols | 3 | 6 | 12 |
|  | nrofUplinkSlot | 2 | 4 | 8 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
|  | Pattern 2 |  |  |  |
|  | dl-UL-TransmissionPeriodicity | 1 ms | 1 ms | 1 ms |
|  | nrofDownlinkSlots | 1 | 2 | 4 |
|  | nrofDownlinkSymbols | 0 | 0 | 0 |
|  | nrofUplinkSlot | 0 | 0 | 0 |
|  | nrofUplinkSymbols | 0 | 0 | 0 |

Proposal 3: TS38.101-2 should specify the following NR TDD configuration for UL RMC to ensure the contiguous UL measurement period of 1ms.

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) |
| UL-DL configuration | referenceSubcarrierSpacing | 60 kHz | 120 kHz |
|  | Pattern 1 |  |  |
|  | dl-UL-TransmissionPeriodicity | 4 ms | 4 ms |
|  | nrofDownlinkSlots | 4 | 9 |
|  | nrofDownlinkSymbols | 12 | 10 |
|  | nrofUplinkSlot | 8 | 17 |
|  | nrofUplinkSymbols | 8 | 2 |
|  | Pattern 2 |  |  |
|  | dl-UL-TransmissionPeriodicity | 1 ms | 1 ms |
|  | nrofDownlinkSlots | 4 | 8 |
|  | nrofDownlinkSymbols | 0 | 0 |
|  | nrofUplinkSlot | 0 | 0 |
|  | nrofUplinkSymbols | 0 | 0 |

Proposal 4: Send LS response to RAN5 that RAN4 specifies the TDD configuration above in TS38.101-1, 101-2, and 101-3.

# References

- R4-1814414, “LS to RAN4 on simultaneous LTE and NR transmissions in EN-DC”, RAN5

- TS36.101

- TS36.521

- TS38.331 V15.3.0.

- R4-1814415, “LS on RAN4-RAN5 5G-NR RF pending issues during RAN5#3-5G-NR Adhoc”, RAN5.

# Appendix

## Example of UL RMC (TS36.101)

Table A.2.3.1.1-1: Reference Channels for QPSK with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/5 | 1/6 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 600 | 1544 | 2216 | 5160 | 4392 | 4584 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 1728 | 4320 | 7200 | 14400 | 21600 | 28800 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: As per Table 4.2-2 in TS 36.211 [4] |  |  |  |  |  |  |  |